- Bridge CLI commands

This chapter describes the Bridge CLI commands.

Summary

    - Bridge CLI commands

The table below lists the Bridge commands provided by the CLI:

| Command | Reference |
| --- | --- |
| bridge add interface | bridge add interface |
| bridge add sourcemac | bridge add sourcemac |
| bridge attach | bridge attach |
| bridge clear interfaces | bridge clear interfaces |
| bridge clear sourcemacs | bridge clear sourcemacs |
| bridge delete interface | bridge delete interface |
| bridge delete sourcemac | bridge delete sourcemac |
| bridge detach interface | bridge detach interface |
| bridge list interfaces | bridge list interfaces |
| bridge list sourcemacs | bridge list sourcemacs |
| bridge set filterage | bridge set filterage |
| bridge set interface filtertype | bridge set interface filtertype |
| bridge set interface portfilter | bridge set interface portfilter |
| bridge set spanning enabled/disabled | bridge set spanning |
| bridge set spanning forwarddelay | bridge set spanning forwarddelay |
| bridge set spanning hellotime | bridge set spanning hellotime |
| bridge set spanning maxage | bridge set spanning maxage |
| bridge set spanning priority | bridge set spanning priority |
| bridge show | bridge show |
| bridge show interface | bridge show interface |

    - Bridge Console commands

The table below lists the bridge console commands and, if available, their CLI equivalent command:

| Command | CLI Equivalent |
| --- | --- |
| bridge device add | Replaced by CLI command bridge add interface |
| bridge device delete | Replaced by CLI command bridge delete interface |
| bridge device list | Replaced by CLI command bridge list interfaces |
| bridge ethertype | Replaced by CLI command bridge set interface filtertype |
| bridge filter | Blacklisted command, see filter |
| bridge filterage | Replaced by CLI command bridge set filterage |
| bridge flush | Usable command, see flush |
| bridge help | Usable command |
| bridge info | Replaced by CLI command bridge show |
| bridge interface | Blacklisted command, see interface |
| bridge portfilter | Blacklisted command, see portfilter |
| bridge spanning enable/disable | Replaced by CLI command bridge set spanning |
| bridge spanning forwarddelay | Replaced by CLI command bridge set spanning forwarddelay |
| bridge spanning hellotime | Replaced by CLI command bridge set spanning hellotime |
| bridge spanning info | Replaced by CLI command bridge show |
| bridge spanning event | Usable command, see spanning |
| bridge spanning maxage | Replaced by CLI command bridge set spanning maxage |
| bridge spanning port | Blacklisted command, see spanning |
| bridge spanning priority | Replaced by CLI command bridge set spanning priority |
| bridge spanning status | Replaced by CLI command bridge show |
| bridge spanning version | Replaced by CLI command bridge show |
| bridge status | Replaced by CLI command bridge show and bridge list interfaces |
| bridge version | Replaced by CLI command bridge show |

bridge add interface

    - Syntax

bridge add interface <name>

    - Description

This command adds a named interface to the bridge.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the interface. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |

    - Example

prompt> bridge add interface bridge1

    - See also

bridge attach

bridge list interfaces

Take an example to create and attach interfaces and transports of wireless as below:

prompt> ethernet add transport wlan wireless

prompt> bridge add interface brwlan

prompt> bridge attach brwlan wlan

  - bridge add sourcemac

Note - This CLI command is only available if the bridge module is built with the feature option of :

Set bridge_sourcemac_forwarding

    - Syntax

bridge add sourcemac <name> <macAddress> <sourceinterface> <destinationinterface>

    - Description

This command creates a source MAC forwarding rule.

When your system receives an Ethernet frame, the system examines the source MAC address of the frame. If both the source MAC address and source interface match the address and interface specified in this command, the system forwards the frame to the specified destination interface. This feature could be useful when the system is configured as multiple PVC connection in bridged mode.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the bridge MAC forwarding rule. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| macaddress | A valid unicast Ethernet MAC address displayed in the following format: ##:##:##:##:##:## | N/A |
| source interface | The name of the existing bridge interface that Ethernet frames are received on. The interface must not be the same as the destination interface and must be attached to a valid transport. To display interface names and their transport attachment details, use the bridge list interfaces command. | N/A |
| destination interface | The name of the existing bridge interface that Ethernet frames are forwarded to. The interface must not be the same as the source interface and must be attached to a valid transport. To display interface names and their transport attachment details, use the bridge list interfaces command. | N/A |

    - Example

prompt> bridge add sourcemac rule1 00:20:2B:07:17:06 eth1 eth2

    - See also

bridge clear sourcemacs

bridge delete sourcemac

bridge list sourcemacs

bridge attach

    - Syntax

bridge attach {<name>|<number>} <transport>

    - Description

This command attaches an existing transport to an existing bridge interface to allow data to be bridged via the transport.

Only one transport can be attached to an interface. If you use this command when there is already a transport attached to the interface, the previous transport is replaced by the new one.

This command implicitly enables the transport being attached.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing bridge interface. To display interface names, use the bridge list interfaces command. | N/A |
| number | A number that identifies an existing bridge interface. To display interface numbers, use the bridge list interfaces command. The number appears in the first column under the heading ID. | N/A |
| transport | A name that identifies an existing transport. To display transport names, use the <transport type> list transports command. | N/A |

    - Example

prompt> bridge attach bridge1 my1483

    - See also

bridge add interface

bridge list interfaces

bridge clear interfaces

    - Syntax

bridge clear interfaces

    - Description

This command deletes all bridge interfaces that were created using the bridge add interface command. Any source MAC forwarding rules associated with the interfaces are also deleted by this command.

    - Example

prompt> bridge clear interfaces

    - See also

bridge delete interface

bridge add sourcemac

bridge clear sourcemacs

Note - This CLI command is only available if the system is built with the following compiler option:

Set bridge_sourcemac_forwarding

    - Syntax

bridge clear sourcemacs

    - Description

This command deletes all source MAC forwarding rules that were created using the bridge add sourcemac command.

    - Example

prompt> bridge clear sourcemacs

    - See also

bridge add sourcemac

bridge delete sourcemac

bridge list sourcemacs

bridge delete interface

    - Syntax

bridge delete interface {<name>|<number>}

    - Description

This command deletes a single interface from the bridge configuration. All source MAC forwarding rules associated with the interface that you want to delete are also deleted by this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing bridge interface. To display interface names, use the bridge list interfaces command. | N/A |
| number | A number that identifies an existing bridge interface. To display interface numbers, use the bridge list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> bridge delete interface 1

    - See also

bridge list interfaces

bridge add sourcemac

bridge delete sourcemac

Note - This CLI command is only available if the system is built with the following compiler option:

Set bridge_sourcemac_forwarding

    - Syntax

bridge delete sourcemac {<name>|<number>}

    - Description

This command deletes a single source MAC forwarding rule that was created using the bridge add sourcemac command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Name | A name that identifies an existing source MAC forwarding rule. To display source MAC forwarding rules, use the bridge list sourcemacs command. | N/A |
| Number | A number that identifies an existing source MAC forwarding rule. To display source MAC forwarding rule numbers, use the bridge list sourcemacs command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> bridge delete sourcemac rule1

    - See also

bridge add sourcemac

bridge clear sourcemacs

bridge list sourcemacs

bridge detach interface

    - Syntax

bridge detach interface {<name>|<number>}

    - Description

This command detaches the transport that was attached to the bridge interface using the bridge attach interface command. All source MAC forwarding rules associated with the interface that you want to detach are deleted by this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing bridge interface. To display interface names, use the bridge list interfaces command. | N/A |
| number | A number that identifies an existing bridge interface. To display interface numbers, use the bridge list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> bridge detach interface 2

    - See also

bridge list interfaces

bridge add sourcemac

bridge list interfaces

    - Syntax

bridge list interfaces

    - Description

This command lists all bridge interfaces that have been created using the bridge add interface command. It displays the following information about bridge interfaces:

- interface ID number

- interface name

- filter type

- name of attached transport (if applicable)

    - Example

prompt> bridge list interfaces

Bridge Interfaces:

ID |   Name   | Filter Type |  Transport

----|----------|-------------|------------

1  | bridge1  | All         | eth1

2  | bridge2  | All         | eth2

3  | bridge3  | All         | eth3

------------------------------------------

    - See also

bridge set filterage

bridge set interface filtertype

bridge list sourcemacs

Note - This CLI command is only available if the system is built with the following compiler option:

Set bridge_sourcemac_forwarding

    - Syntax

bridge list sourcemacs

    - Description

This command lists all source MAC forwarding rules that have been created using the bridge add sourcemac command. It displays the following information about MAC forwarding rules:

- MAC forwarding rule ID number

- MAC forwarding rule name

- source MAC address

- source interface

- destination interface

    - Example

prompt> bridge list sourcemacs

Bridge Source Mac Entries:

ID  |  Name  |    Mac Address    |Source Interface| Dest Interface

-----|--------|-------------------|---------------------------------

1 | rule1  | 00:20:2B:07:17:06 |      e1        |      a1

2 | rule2  | 00:20:2B:07:17:07 |      e1        |      a2

3 | rule3  | 00:20:2B:07:17:08 |      a1        |      e1

--------------------------------------------------------------------

    - See also

bridge add sourcemac

bridge clear sourcemacs

bridge delete sourcemac

bridge set filterage

    - Syntax

bridge set filterage <filter age>

    - Description

This command specifies the maximum age of filter table entries for the bridge. The filter age for the bridge is displayed by the bridge show command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| filter age | The filter age is the time (in seconds) after which MAC addresses are removed from the filter table when there has been no activity. The time may be an integer value between 10 and 100,000 seconds. | 300 seconds |

    - Example

prompt> bridge set filterage 2000

    - See also

bridge show

bridge set interface filtertype

    - Syntax

bridge set interface {<name>|<number>} filtertype {all|ip|pppoe}

    - Description

This command specifies the type of ethernet frame filtering performed by the named bridge interface.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing bridge interface. To display interface names, use the bridge list interfaces command. | N/A |
| number | A number that identifies an existing bridge interface. To display interface numbers, use the bridge list interfaces command. The number appears in the first column under the heading ID. | N/A |
| all | Allows all types of ethernet packets through the bridge interface | All |
| ip | Allows only IP/ARP types of ethernet packets through the bridge interface. |  |
| pppoe | Allows only PPPoE types of ethernet packets through the bridge interface |  |

    - Example

prompt> bridge set interface bridge2 filtertype ip

    - See also

bridge list interfaces

bridge set interface portfilter

    - Syntax

bridge set interface {<name>|<number>} portfilter {all|<port>}

    - Description

This command controls the bridge’s forwarding and broadcasting behavior. It allows you to set a portfilter on a bridge interface to determine which port or ports multicast and unknown packets should be forwarded to.

This command sets one destination port at a time. If you want to forward packets to several ports, enter a bridge set interface portfilter <port> command for each port. If you want to forward packets to all ports, enter the command and specify the all value.

Note - If a unicast packet is received by an interface with a portfilter set to all, the portfilter rule is ignored. The unicast packet is still only sent to one port.

Note - If the bridge itself is attached to the router, the bridge itself will always forward to all ports and will always be forwarded to by all ports.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing bridge interface. To display interface names, use the bridge list interfaces command. | N/A |
| number | A number that identifies an existing bridge interface. To display interface numbers, use the bridge list interfaces command. The number appears in the first column under the heading ID. | N/A |
| port | The name of the existing port that you want packets, received on a specified bridge interface, to be forwarded to. To display port names, use the port ? command. | all |
| all | Forwards packets, received on a specified bridge interface, to all existing bridge ports. |  |

    - Example

prompt> bridge set interface bridge1 portfilter ethernet

    - See also

bridge list interfaces

bridge show interface

bridge set spanning

    - Syntax

bridge set spanning {enabled|disabled}

    - Description

This command specifies whether or not the bridge is to implement the spanning tree protocol (STP). The current spanning tree setting is displayed by the bridge show command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | Allows the bridge to use the spanning tree protocol. | disabled |
| disabled | Ensures that the bridge acts as a transparent bridge. |  |

    - Example

prompt> bridge set spanning enabled

    - See also

bridge set spanning forwarddelay

bridge set spanning hellotime

bridge set spanning maxage

bridge set spanning priority

bridge show

bridge set spanning forwarddelay

    - Syntax

bridge set spanning forwarddelay <delay>

    - Description

This command sets the time that the bridge spends in listening or learning states when the bridge is or is attempting to become the root bridge. The current forwarddelay setting is displayed by the bridge show command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| delay | This can be any value (in seconds) between 4 and 30 BUT it is constrained by the maxage and hellotimes. The maxage, hellotime and forwarddelay times are constrained as follows: 2 x (forwarddelay - 1)>maxage maxage>2 x (hellotime + 1) For example, the default settings are: 2 x (15 - 1)>20 20>2 x (2 + 1) | 15 |

    - Example

prompt> bridge set spanning forwarddelay 20

    - See also

bridge set spanning

bridge show

bridge set spanning hellotime

    - Syntax

bridge set spanning hellotime <hellotime>

    - Description

This command sets the time after which the spanning tree process sends notification of topology changes to the root bridge. This is used when the bridge is or is attempting to become the root bridge. The hellotime setting is displayed by the bridge show command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| hellotime | This can be any value (in seconds) between 1 and 10 BUT it is constrained by the maximum age and forwarddelay times. The maxage, hellotime and forwarddelay times are constrained. | 2 |

    - Example

prompt> bridge set spanning hellotime 10

    - See also

bridge set spanning

bridge set spanning forwarddelay

bridge set spanning maxage

bridge set spanning priority

bridge show

bridge set spanning maxage

    - Syntax

bridge set spanning maxage <maxage>

    - Description

This command sets the maximum age of received spanning tree protocol information before it is discarded. This is used when the bridge is or is attempting to become the root bridge. The maxage setting is displayed by the bridge show command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| maxage | This can be any value (in seconds) between 6 and 40 BUT it is constrained by the hellotime and forwarddelay times. The maxage, hellotime and forwarddelay times are constrained. | 20 |

    - Example

prompt> bridge set spanning maxage 30

    - See also

bridge set spanning

bridge set spanning forwarddelay

bridge set spanning hellotime

bridge set spanning priority

bridge show

bridge set spanning priority

    - Syntax

bridge set spanning priority <priority>

    - Description

This command sets the spanning tree protocol priority. Where two bridges have the same priority, their MAC address is compared and the smaller MAC address is treated as the most significant.

Spanning tree must be enabled before you can use this command. The priority setting is displayed by the bridge show command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| priority | A value that assigns priority to the bridge: the lower the priority number, the more significant the bridge becomes in protocol terms. The priority can be any value (in seconds) between 0 and 65535. | 32768 |

    - Example

prompt> bridge set spanning priority 1000

    - See also

bridge set spanning

bridge set spanning forwarddelay

bridge set spanning hellotime

bridge set spanning maxage

bridge show

bridge show

    - Syntax

bridge show

    - Description

This command shows the global configuration settings for the bridge. The following bridge information is displayed:

- filter age

- spanning tree setting (true or false)

- spanning tree priority value

- spanning tree forward delay time (seconds)

- spanning tree hello time (seconds)

- spanning tree maximum age (seconds)

    - Example

prompt> bridge show

Global bridge configuration:

Filter age: 2000

Spanning bridge configuration:

Spanning: true

Priority: 1000

Forward delay: 20

Hello time: 10

Max. age: 30

    - See also

bridge set spanning

bridge set spanning forwarddelay

bridge set spanning hellotime

bridge set spanning maxage

bridge show interface

    - Syntax

bridge show interface {<name>|<number>}

    - Description

This command displays the filter type value and portfilter setting of a named bridge interface.

Note - This command does not shows the current contents of the bridge’s filter table. See the console command bridge filter

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing bridge interface. To display interface names, use the bridge list interfaces command. | N/A |
| number | A number that identifies an existing bridge interface. To display interface numbers, use the bridge list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Examples

prompt> bridge show interface bridge1

Bridge Interface: bridge1

Filter Type: Pppoe

Port Filter: All

    - See also

bridge set interface filtertype

bridge set interface portfilter