- DHCP Client CLI commands

This chapter describes the DHCP client CLI commands.

Summary

    - DHCP client CLI commands

The table below lists the DHCP client commands provided by the CLI:

| Command | Reference |
| --- | --- |
| dhcpclient add interfaceconfig | dhcpclient add interfaceconfig |
| dhcpclient clear interfaceconfigs | dhcpclient clear interfaceconfigs |
| dhcpclient delete interfaceconfig | dhcpclient delete interfaceconfig |
| dhcpclient interfaceconfig add requested option | dhcpclient interfaceconfig add requested option |
| dhcpclient interfaceconfig add required option | dhcpclient interfaceconfig add required option |
| dhcpclient interfaceconfig add sent option | dhcpclient interfaceconfig add sent option |
| dhcpclient interfaceconfig clear sent options | dhcpclient interfaceconfig clear sent options |
| dhcpclient interfaceconfig clear requested options | dhcpclient interfaceconfig clear requested options |
| dhcpclient interfaceconfig delete requested option | dhcpclient interfaceconfig delete requested option |
| dhcpclient interfaceconfig delete sent option | dhcpclient interfaceconfig delete sent option |
| dhcpclient interfaceconfig list requested options | dhcpclient interfaceconfig list requested options |
| dhcpclient interfaceconfig list sent options | dhcpclient interfaceconfig list sent options |
| dhcpclient list interfaceconfigs | dhcpclient list interfaceconfigs |
| dhcpclient release | dhcpclient release |
| dhcpclient set backoff | dhcpclient set backoff |
| dhcpclient set interfaceconfig autoip | dhcpclient set interfaceconfig autoip enabled¦disabled |
| dhcpclient set interfaceconfig clientid | dhcpclient set interfaceconfig clientid |
| dhcpclient set interfaceconfig defaultroute enabled¦disabled | dhcpclient set interfaceconfig defaultroute enabled¦disabled |
| dhcpclient set interfaceconfig dhcpinform enabled¦disabled | dhcpclient set interfaceconfig dhcpinform enabled¦disabled |
| dhcpclient set interfaceconfig dhcpserverpoolsize | dhcpclient set interfaceconfig dhcpserverpoolsize |
| dhcpclient set interfaceconfig dhcpserverinterface | dhcpclient set interfaceconfig dhcpserverinterface |
| dhcpclient set interfaceconfig givednstoclient enabled¦disabled | dhcpclient set interfaceconfig givednstoclient enabled¦disabled |
| dhcpclient set interfaceconfig givednstorelay enabled¦disabled | dhcpclient set interfaceconfig givednstorelay enabled¦disabled |
| dhcpclient set interfaceconfig interface | dhcpclient set interfaceconfig interface |
| dhcpclient set interfaceconfig noclientid | dhcpclient set interfaceconfig noclientid |
| dhcpclient set interfaceconfig requestedleasetime | dhcpclient set interfaceconfig requestedleasetime |
| dhcpclient set interfaceconfig server | dhcpclient set interfaceconfig server |
| dhcpclient set reboot | dhcpclient set reboot |
| dhcpclient set retry | dhcpclient set retry |
| dhcpclient show | dhcpclient show |
| dhcpclient update | dhcpclient update |

    - DHCP client Console commands

The table below lists the dhcp-client console commands and, if available, their equivalent CLI command:

| Command | CLI Equivalent |
| --- | --- |
| dhcpclient config | Replaced by CLI command dhcpclient show |
| dhcpclient help | Usable command, see help |
| dhcpclient status | Replaced by CLI command dhcpclient show |
| dhcpclient pool | Usable command, see pool |
| dhcpclient trace | Usable command, see trace |

    - DHCP client default settings

By default, DHCP client requests the following information from DHCP server:

- the DHCP server IP address and subnet mask

- DNS server addresses. By default, received DNS server addresses are passed on to the DNS relay and not passed to DNS client. To change these default settings, use the commands dhcpclient set interfaceconfig givednstoclient enabled|disabled  and dhcpclient set interfaceconfig givednstorelay enabled|disabled.

- DHCP server default gateway information. By default, DHCP client makes use of default gateway information. To change this default setting, use the command dhcpclient set interfaceconfig defaultroute enabled|disabled.

dhcpclient add interfaceconfig

    - Syntax

dhcpclient add interfaceconfig <name> <ip interface>

    - Description

This command configures DHCP client parameters for negotiation over an existing IP interface. The client interface can only set the IP configuration if the IP interface has DHCP enabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the client interface. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| ip interface | An IP address or a name that identifies an existing IP interface. The interface must have DHCP enabled. To display interface names, use the ip list interfaces command. | N/A |

    - Example

prompt> dhcpclient add interfaceconfig config1 ip1

    - See also

dhcpclient list interfaceconfigs

ip list interfaces

ip set interface dhcp

dhcpclient clear interfaceconfigs

    - Syntax

dhcpclient clear interfaceconfigs

    - Description

This command deletes all existing DHCP client interface configurations.

    - Example

prompt> dhcpclient clear interfaceconfigs

    - See also

dhcpclient list interfaceconfigs

dhcpclient delete interfaceconfig

    - Syntax

dhcpclient delete interfaceconfig {<name>|<number>}

    - Description

This command deletes a single DHCP client interface configuration.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |

    - Example

prompt> dhcpclient delete interfaceconfig config1

    - See also

dhcpclient list interfaceconfigs

dhcpclient interfaceconfig add requested option

    - Syntax

dhcpclient interfaceconfig {<name>|<number>} add requested option <option>

    - Description

This command tells the DHCP client to request a specified option from a DHCP server. The requested option is not compulsory - if the option is not included in a lease offered by DHCP server, the DHCP client will still accept the offer.

Options are detailed in RFC 2132.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| option | A text string that identifies a DHCP server configuration option. | N/A |

    - Example

prompt> dhcpclient interfaceconfig client1 add requested option irc-server

    - See also

dhcpclient interfaceconfig clear requested options

dhcpclient interfaceconfig delete requested option

dhcpclient interfaceconfig list requested options

For information on RFC 2132, see http://www.ietf.org/rfc/rfc2132.txt

dhcpclient interfaceconfig add required option

    - Syntax

dhcpclient interfaceconfig {<name>|<number>} add required option <option>

    - Description

This command tells DHCP client that it requires a specified option from DHCP server. The required option is compulsory - if the option is not included in a lease offered by DHCP server, the DHCP client will ignore the offer.

Options are detailed in RFC 2132 or you can use dhcpserver list options to display the option names..

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| option | A text string that identifies a DHCP server configuration option. | N/A |

    - Example

prompt> dhcpclient interfaceconfig client1 add required option domain-name

    - See also

dhcpclient interfaceconfig add requested option

dhcpclient interfaceconfig add sent option

    - Syntax

dhcpclient interfaceconfig {<name>|<number>} add sent option <option> <value>

    - Description

This command tells the DHCP client to send a value for the given DHCP configuration option to a DHCP server. The DHCP server’s response depends on the type of option being sent out.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| option | A text string that identifies a DHCP server configuration option. | N/A |
| value | The value associated with the option identifier. | N/A |

    - Example

prompt> dhcpclient interfaceconfig client1 add sent option host-name ‘“vancouver”’

This command example tells the DHCP client to send the DHCP host-name option to the DHCP server with the value “vancouver”. Note that for options with string-type values associated with them, the option value must be in double-quotes (“ ”). Also, the entire string including the double quotes must be inside single quotes (‘’) to ensure that the CLI treats the double quotes literally.

    - See also

dhcpclient list interfaceconfigs

dhcpclient interfaceconfig clear sent options

dhcpclient interfaceconfig delete sent option

dhcpclient interfaceconfig list sent options

dhcpclient interfaceconfig clear sent options

    - Syntax

dhcpclient interfaceconfig {<name>|<number>} clear sent options

    - Description

This command deletes all options that were previously added to an interfaceconfig using the dhcpclient interfaceconfig add sent option command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |

    - Example

prompt> dhcpclient interfaceconfig client1 clear sent options

    - See also

dhcpclient list interfaceconfigs

dhcpclient interfaceconfig list sent options

dhcpclient interfaceconfig add sent option

dhcpclient interfaceconfig delete sent option

dhcpclient interfaceconfig clear requested options

    - Syntax

dhcpclient interfaceconfig {<name>|<number>} clear requested options

    - Description

This command deletes all options that were previously added to an interfaceconfig using the dhcpclient interfaceconfig add requested/required option commands.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| Number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |

    - Example

prompt> dhcpclient interfaceconfig client1 clear requested options

    - See also

dhcpclient list interfaceconfigs

dhcpclient interfaceconfig add requested option

dhcpclient interfaceconfig add required option

dhcpclient interfaceconfig delete requested option

dhcpclient interfaceconfig delete requested option

    - Syntax

dhcpclient interfaceconfig {<name>|<number>} delete requested option <option number>

    - Description

This command deletes a single option that was previously added to an interfaceconfig using the dhcpclient interfaceconfig add requested/required option commands.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| option number | A number that identifies an option that is requested from the DHCP server by the DHCP client. To display option numbers, use the dhcpclient interfaceconfig list requested options command. | N/A |

    - Example

prompt> dhcpclient interfaceconfig client1 delete requested option 1

    - See also

dhcpclient list interfaceconfigs

dhcpclient interfaceconfig add requested option

dhcpclient interfaceconfig add required option

dhcpclient interfaceconfig clear requested options

dhcpclient interfaceconfig delete sent option

    - Syntax

dhcpclient interfaceconfig {<name>|<number>} delete sent option <option number>

    - Description

This command deletes a single option that was previously added to an interfaceconfig using the dhcpclient interfaceconfig add sent option command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| option number | A number that identifies an option that is sent from the DHCP client to the DHCP server. To display option numbers, use the dhcpclient interfaceconfig list sent options command. | N/A |

    - Example

prompt> dhcpclient interfaceconfig client1 delete sent option 5

    - See also

dhcpclient list interfaceconfigs

dhcpclient interfaceconfig list sent options

dhcpclient interfaceconfig add sent option

dhcpclient interfaceconfig clear sent options

dhcpclient interfaceconfig list requested options

    - Syntax

dhcpclient interfaceconfig {<name>|<number>} list requested options

    - Description

This command lists the options that the DHCP client requests and/or requires from the DHCP server. These options were set using the dhcpclient interfaceconfig add requested/required option commands. The following information is displayed:

- Option identification number

- Option identifier (name)

- Requirement status - true for options that were added using the dhcpclient interfaceconfig add required option command, false for options added using the dhcpclient interfaceconfig add requested option command.

Options and their values are detailed in RFC2132.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |

    - Example

prompt> dhcpclient interfaceconfig client1 list requested options

ID  |       Identifier   | Is option required?

-----|--------------------|---------------------

1 | host-name          | true

2 | domain-name        | false

------------------------------------------------

    - See also

dhcpclient interfaceconfig add requested option

dhcpclient interfaceconfig add required option

dhcpserver subnet add option

dhcpclient interfaceconfig list sent options

    - Syntax

dhcpclient interfaceconfig {<name>|<number>} list sent options

    - Description

This command displays a list of the options that the DHCP client sends to the DHCP server. These options were set using the dhcpclient interfaceconfig add sent option command. The following information is displayed:

- Option identification number

- Option identifier (name)

- Suggested value

Options and their values are detailed in RFC2132.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |

    - Example

prompt> dhcpclient interfaceconfig client1 list sent options

DHCP client options to be sent to server for client1:

ID  |       Identifier   | Suggested Value

-----|--------------------|---------------------

1 | host-name          | vancouver

2 | domain-name        | globespanvirata

------------------------------------------------

    - See also

dhcpclient interfaceconfig add sent option

dhcpclient interfaceconfig clear sent options

dhcpclient interfaceconfig delete sent option

dhcpserver subnet add option

dhcpclient list interfaceconfigs

    - Syntax

dhcpclient list interfaceconfigs

    - Description

This command lists the following information about existing DHCP client interfaces:

- interface identification number

- interface name

- IP interface configured by the client interface

- requested lease time (in seconds)

- client identifier (if set)

- Status of IP address auto-configuration (true or false)

- Whether the DHCP client has used DHCPINFORM message type

    - Example

prompt> dhcpclient list interfaceconfigs

DHCP Client Declarations:

Requested

ID  |   Name     | Interface  | Lease Time | Client ID         | AutoIP | DHCPINFORM

-----|------------|------------|------------|-------------------|--------|------------

1 | client1    | ip1        | 9000       | 00:11:22:33:44:5a | true   | false

----------------------------------------------------------------|--------|------------

    - See also

dhcpclient show

dhcpclient set interfaceconfig requestedleasetime

dhcpclient set interfaceconfig clientid

dhcpclient set interfaceconfig autoip enabled|disabled

dhcpclient release

    - Syntax

dhcpclient release

    - Description

This command will force all of DHCP clients to release the acquired IP address.

    - Example

prompt> dhcpclient release

    - See also

dhcpclient update

dhcpclient set backoff

    - Syntax

dhcpclient set backoff <backofftime>

    - Description

This command sets the global maximum time (in seconds) that a DHCP client interface will ‘back off’ between issuing individual DHCP requests. This prevents many clients trying to configure themselves at the same time, and sending too many requests at once.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| backofftime | The maximum number of seconds that the DHCP client can pause for between unsuccessful DHCP negotiations. | 120 |

    - Example

prompt> dhcpclient set backoff 200

    - See also

dhcpclient show

dhcpclient set interfaceconfig autoip enabled|disabled

dhcpclient set interfaceconfig {<name>|<number>} autoip {enabled | disabled}

    - Description

This command enables/disables IP address auto-configuration (Auto-IP).

Auto-IP automatically configures an IP address when a DHCP client fails to contact a DHCP server and cannot obtain a lease. An IP address on the 169.254.x.x subnet is automatically created, and ARP requests are issued for the suggested IP address. The address is abandoned if it already exists on the network or if any other host on the network issues an ARP probe for that IP address.

Once an IP address has been automatically configured, the DHCP client continues to check whether or not it can contact a DHCP server. If the client can contact a DHCP server and obtain a legitimate lease, the legitimate lease will supersede the auto-configured IP address.

Note - Even if you have enabled Auto-IP using this command, you will not be able to use IP address auto-configuration if a DHCP server on the same network does not allow it. See the dhcpserver subnet add option command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| enabled | Enables Auto-IP on a specified dhcp client. | enabled |
| disabled | Disables Auto-IP on a specified dhcp client. |  |

    - Example

prompt> dhcpclient set interfaceconfig mycfg autoip enabled

    - See also

dhcpserver subnet add option  (see the specific example given for this command)

For further information on the RFC standard for DHCP IP address auto-configuration, see http://www.ietf.org/rfc/rfc2563.txt.

dhcpclient set interfaceconfig clientid

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} clientid <clientid>

    - Description

This command sets a unique client identifier that DHCP server uses to identify the client.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| client id | A unique identifier that DHCP server can use to identify the client. For Microsoft DHCP servers, the client ID should be the MAC address of the card that DHCP is running on. For other DHCP servers, the client ID can be a MAC address or a text string such as the hostname. | N/A |

    - Example

prompt> dhcpclient set interfaceconfig client1 clientid 00:11.22.33.44.5a

    - See also

dhcpclient list interfaceconfigs

dhcpclient set interfaceconfig defaultroute enabled|disabled

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} defaultroute {enabled|disabled}

    - Description

This command enables/disables whether DHCP client makes use of default gateway information received from a DHCP server. If no DHCP interfaceconfigs have been added to the system, by default DHCP client will use default gateway information received from DHCP server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| enabled | DHCP client uses default gateway information it receives from DHCP server. | enabled |
| disabled | DHCP client does not use default gateway information it receives from DHCP server. |  |

    - Example

prompt> dhcpclient set interfaceconfig client1 defaultroute disabled

    - See also

dhcpclient list interfaceconfigs

dhcpclient set interfaceconfig dhcpinform enabled|disabled

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} dhcpinform {enabled|disabled}

    - Description

This command enables/disables whether DHCP client uses the dhcpinform message type. This DHCP message type is used whenever a client has obtained an IP address or subnet mask (for example, the address has been manually configured or obtained through PPP/IPCP), but wishes to obtain extra configuration parameters (such as DNS servers or default gateway) from a DHCP server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| enabled | Enables the dhcpinform message type. IP address and subnet mask will not be negotiated if this mode is selected. | disabled |
| disabled | Disables the dhcpinform message type. |  |

    - Example

prompt> dhcpclient set interfaceconfig client1 dhcpinform disabled

    - See also

dhcpclient list interfaceconfigs

dhcpclient set interfaceconfig server

dhcpclient set interfaceconfig dhcpserverinterface

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} dhcpserverinterface <interface name>

    - Description

This command allows the user to specify an existing IP interface on which the automatically configured DHCP server can be created. If the interface name does not correspond with an existing IP interface, or no interface name is given, the DHCP server will be placed on the first LAN interface that it finds.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| interface name | The name that identifies an existing IP interface. To display IP interface names, use the ip list interfaces command. | N/A |

    - Example

prompt> dhcpclient set interfaceconfig client1 dhcpserverinterface ip2

    - See also

dhcpclient list interfaceconfigs

dhcpclient set interfaceconfig dhcpserverpoolsize

ip list interfaces

dhcpclient set interfaceconfig dhcpserverpoolsize

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} dhcpserverpoolsize <pool size>

    - Description

This command tells DHCP client to configure a DHCP server on the LAN if the given address pool size is set to a number greater than 0. The LAN DHCP server is configured using parameters received by a DHCP client interface on the WAN. Information such as DNS server addresses can then be distributed to LAN clients. The new DHCP server gives out the default gateway address as its LAN IP address.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| pool size | The number of DHCP client addresses in a pool. The first address in the pool is the address immediately after the LAN DHCP address. For example, if the LAN DHCP address is 192.168.102.3, the first address in the pool will be 192.168.102.4. | N/A |

    - Example

prompt> dhcpclient set interfaceconfig client1 dhcpserverpoolsize 5

    - See also

dhcpclient list interfaceconfigs

dhcpclient set interfaceconfig givednstoclient enabled|disabled

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} givednstoclient {enabled|disabled}

    - Description

This command enables/disables whether DHCP client passes received DNS server addresses to DNS client. If no DHCP interfaceconfigs have been added to the system, by default DHCP client will not pass DNS server addresses to DNS client.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| enabled | DHCP client passes DNS server addresses to DNS client. | disabled |
| disabled | DHCP client does not pass DNS server addresses to DNS client. |  |

    - Example

prompt> dhcpclient set interfaceconfig client1 givednstoclient disabled

    - See also

dhcpclient list interfaceconfigs

dhcpclient set interfaceconfig givednstorelay enabled|disabled

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} givednstorelay {enabled|disabled}

    - Description

This command enables/disables whether DHCP client passes received DNS server addresses to DNS relay. If no DHCP interfaceconfigs have been added to the system, by default DHCP client will pass DNS server addresses to DNS relay.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| enabled | DHCP client passes DNS server addresses to DNS relay. | enabled |
| disabled | DHCP client does not pass DNS server addresses to DNS relay. |  |

    - Example

prompt> dhcpclient set interfaceconfig client1 givednstorelay disabled

    - See also

dhcpclient list interfaceconfigs

dhcpclient set interfaceconfig interface

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} interface <ipinterface>

    - Description

This command sets the IP interface that will have its configuration set by the DHCP client interface. The client interface can only set the IP configuration if the IP interface has DHCP enabled, using the ip set interface dhcp command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| ipinterface | A name that identifies an existing IP interface. The interface must have DHCP enabled. To display interface names, use the ip list interfaces command. | N/A |

    - Example

prompt> dhcpclient set interfaceconfig client1 interface ip2

    - See also

dhcpclient list interfaceconfigs

ip list interfaces

ip set interface dhcp

dhcpclient set interfaceconfig noclientid

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} noclientid

    - Description

This command deletes a client identifier from a DHCP client. The DHCP server must have ‘allowunknownclients’ enabled in order to work with DHCP clients that are not specifically named in DHCP server configuration or its lease database.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |

    - Example

prompt> dhcpclient set interfaceconfig client1 noclientid

    - See also

dhcpclient set interfaceconfig clientid

dhcpserver set allowunknownclients

dhcpclient set interfaceconfig requestedleasetime

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} requestedleasetime <requestedleasetime>

    - Description

The DHCP client requests a specific lease time from the DHCP server for the allocated IP addresses. This command determines the length of lease time requested. The DHCP server will ‘cap’ a requested lease time if it is too large.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| requested lease time | The lease time (in seconds) that a DHCP client requests from the DHCP server. | 86400 |

    - Example

prompt> dhcpclient set interfaceconfig client1 requestedleasetime 70000

    - See also

dhcpclient list interfaceconfigs

dhcpserver set maxleasetime

dhcpserver set defaultleasetime

dhcpclient set interfaceconfig server

    - Syntax

dhcpclient set interfaceconfig {<name>|<number>} server <ipaddress>

    - Description

If dhcpclient set dhcpinform has been set to enabled, this command will unicast the first DHCPINFORM message to the specific DHCP server at the specified IP address. If the first unicast fails, the DHCPINFORM will default to broadcasting its messages.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Name | A name that identifies an existing DHCP client interface. To display client interface names, use the dhcpclient list interfaceconfigs command. | N/A |
| Number | A number that identifies an existing DHCP client interface. To display client interface numbers, use the dhcpclient list interfaceconfigs command. | N/A |
| Ipaddress | The IP address of a DHCP server that DHCP client can use to obtain configuration parameters. The IP address is displayed in the following format: 192.168.102.3 | N/A |

    - Example

prompt> dhcpclient set interfaceconfig client1 server 192.168.101.2

    - See also

dhcpclient set interfaceconfig dhcpinform enabled|disabled

dhcpclient set reboot

    - Syntax

dhcpclient set reboot <reboottime>

    - Description

When the DHCP client is restarted, it tries to reacquire the last address that it had. This command sets the time between the client trying to reacquire its last address and giving up then trying to discover a new address.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Reboottime | The time (in seconds) after a client tries to reacquire the last IP address it had and before the client gives up then tries to discover a new address. | 10 |

    - Example

prompt> dhcpclient set reboot 5

    - See also

dhcpclient show

dhcpclient set retry

    - Syntax

dhcpclient set retry <retrytime>

    - Description

This command sets the time that must pass after the client has determined that no DHCP server is present before it tries again to contact a DHCP server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| retrytime | The time (in seconds) that must pass after the client has determined that no DHCP server is present before it tries again to contact a DHCP server. | 300 |

    - Example

prompt> dhcpclient set retry 150

    - See also

dhcpclient show

dhcpclient show

    - Syntax

dhcpclient show

    - Description

This command displays the following global configuration information about DHCP client:

- reboot time

- retry time

- maximum backoff time

    - Example

prompt> dhcpclient show

Global DHCP Client Configuration:

Reboot time: 10

Retry time: 300

Max. backoff time: 120

    - See also

dhcpclient set reboot

dhcpclient set retry

dhcpclient set backoff

dhcpclient update

    - Syntax

dhcpclient update

    - Description

This command updates the DHCP client configuration. Changes made to the client configuration are not updated until this command has been entered.

    - Example

prompt> dhcpclient update

dhcpclient: Reset request acknowledged. Reset imminent.

    - See also

dhcpclient release