- SNMP CLI commands

This chapter describes CLI support for SNMP.

Summary

Configuring view-based access control is a process that requires three steps:

- Define a family of view subtrees.

- Define a group and its associated access rights.

- Assign a SNMPv3 user to the group defined in step 2.

Configuration notification is a process that requires three steps:

- Define a notification

- Define a set of network addresses to which a notification should be sent.

- Define parameters to use when sending notifications to each of the target addressed identified in step 2.

    - SNMP CLI commands

The table below lists the SNMP commands provided by the CLI:

| Command | Reference |
| --- | --- |
| snmp add community | snmp add community |
| snmp delete community | snmp delete community |
| snmp clear communities | snmp clear communities |
| snmp list communities | snmp list communities |
| snmp add trap | snmp add trap |
| snmp delete trap | snmp delete trap |
| snmp clear traps | snmp clear traps |
| snmp list traps | snmp list traps |
| snmp add view | snmp add view |
| snmp set view | snmp set view |
| snmp delete view | snmp delete view |
| snmp clear views | snmp clear views |
| snmp list views | snmp list views |
| snmp add access | snmp add access |
| snmp set access | snmp set access |
| snmp delete access | snmp delete access |
| snmp clear accesses | snmp clear accesses |
| snmp list accesses | snmp list accesses |
| snmp add user | snmp add user |
| snmp set user | snmp set user |
| snmp delete user | snmp delete user |
| snmp clear users | snmp clear users |
| snmp list users | snmp list users |
| snmp add group | snmp add group |
| snmp set group | snmp set group |
| snmp delete group | snmp delete group |
| snmp clear group | snmp clear group |
| snmp list group | snmp list group |
| snmp add notify | snmp add notify |
| snmp set notify | snmp set notify |
| snmp delete notify | snmp delete notify |
| snmp clear notifications | snmp clear notifications |
| snmp list notifications | snmp list notifications |
| snmp add targetaddr | snmp add targetaddr |
| snmp set targetaddr | snmp set targetaddr |
| snmp delete targetaddr | snmp delete targetaddr |
| snmp clear targetaddresses | snmp clear targetaddresses |
| snmp list targetaddresses | snmp list targetaddresses |
| snmp add targetparams | snmp add targetparams |
| snmp set targetparams | snmp set targetparams |
| snmp delete targetparams | snmp delete targetparams |
| snmp clear targetparams | snmp clear targetparams |
| snmp list targetparams | snmp list targetparams |
| snmp status | snmp status |

snmp add community

    - Syntax

snmp add community read <Community> [<IPaddr>]

snmp add community write <Community> [<IPaddr>]

    - Description

Create a read-only or read-write community for SNMPv1 or SNMPv2c. If <IPaddr> option is omitted, there will no check IP address for SNMP manager.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Community | community or password | N/A |
| IPaddr | the IP address designated for SNMP manager | 0.0.0.0 |

    - Example

prompt> snmp add community read public

prompt> snmp add community write password

    - See also

snmp list communities

snmp delete community

snmp clear communities

snmp delete community

    - Syntax

snmp delete community <Community>

    - Description

Destory a community, either read-only or read-write .

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Community | community or password | N/A |

    - Example

prompt> snmp delete community public

    - See also

snmp add community

snmp list communities

snmp clear communities

snmp clear communities

    - Syntax

snmp clear communities

    - Description

Delete all of snmp communities.

    - Example

prompt> snmp clear communities

    - See also

snmp add community

snmp delete community

snmp list communities

snmp list communities

    - Syntax

snmp list communities

    - Description

Display the configuration of all communities.

    - Example

prompt> snmp list communities

SNMP Accesses:

ID  |    Community   |    Ipaddress   | Access

------|----------------|----------------|---------

1  |public          |0.0.0.0         | read

2  |password        |0.0.0.0         | write

--------------------------------------------------

    - See also

snmp add community

snmp delete community

snmp clear communities

snmp add trap

    - Syntax

snmp add trap <Community> <IPaddr>

    - Description

Create a SNMPv1 trap receiver.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Community | community or password | N/A |
| IPaddr | the IP address designated for SNMP manager | N/A |

    - Example

prompt> snmp add trap public 192.168.1.1

    - See also

snmp delete trap

snmp clear traps

snmp list traps

snmp delete trap

    - Syntax

snmp delete trap <Community>

    - Description

Destroy a trap receive.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Community | community or password | N/A |

    - Example

prompt> snmp delete trap public

    - See also

snmp add trap

snmp clear traps

snmp list traps

snmp clear traps

    - Syntax

snmp clear traps

    - Description

Delete all of configured trap receivers.

    - Example

prompt> snmp clear traps

    - See also

snmp add trap

snmp delete trap

snmp list traps

snmp list traps

    - Syntax

Snmp list traps

    - Description

Display the configuration of trap receivers.

    - Example

prompt> snmp list traps

SNMP Traps:

ID  |    Community   |    Ipaddress

------|----------------|----------------

1  |public          |192.168.1.1

----------------------------------------

    - See also

snmp add trap

snmp clear traps

snmp delete  traps

snmp add view

    - Syntax

snmp add view <viewname> <oidsubtree>

    - Description

Create a view tree family.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| viewname | a human readable string representing the name of this family of view subtrees | N/A |
| oidsubtree | an OID that identifies a subtree of the MIB | N/A |

    - Example

Defines a subtree for the view named “All” that includes the entire set of 1.3.6.1(iso.org.dod.internet).

prompt> snmp add view All 1.3.6.1

    - See also

snmp set view

snmp delete view

snmp clear views

snmp list views

snmp set view

    - Syntax

snmp set view <ID> oidmask <oidmask>

snmp set view <ID> type included | excluded

    - Description

oidmask: oidmask is built using octets that correspond to the OIDSubtree being restricted .

type: Set the view type to “included” or “excluded” and indicates if the OIDSubtree is explicity accessible or not accessible in this family of view subtrees.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ID | A number that identifies an existing MIB view. | N/A |
| oidmask | OIDMask is a series of ones and zeros used for masking out parts of the OIDSubtree. A zero indicates a “wild card”, and a one indicates an exact match be made. ”don’t care” is represented with a dash(-). | - |
| included | Included the MIB view | N/A |
| excluded | Excluded the MIB view | N/A |

    - Example

prompt> snmp set view 1 oidmask -

prompt> snmp set view 1 type included

For example, one may wish to restrict a user’s view of the ifTable(1.3.6.1.2.1.2.2.1) to only the second row, all columns.

|  | Byte 1 |  |  |  |  |  |  |  | Byte 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OIDSubtree | 1 | 3 | 6 | 1 | 2 | 1 | 2 | 2 | 1 | 0 | 2 |  |  |  |  |  |
| Original_mask | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |  |  |  |  |  |
| Padded with 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| Hex value | ff |  |  |  |  |  |  |  | bf |  |  |  |  |  |  |  |

So set the MIB view config as below:

ID  |     Name    |    OIDSubtree   | OIDMask |    Type

------|-------------|-----------------|---------|-------------

1  | interface2  | 1.3.6.1.2.1.2.2.1.0.2| ff:bf   | included

--------------------------------------------------------------

    - See also

snmp add view

snmp delete view

snmp clear views

snmp list views

snmp delete view

    - Syntax

snmp delete view <ID>

    - Description

Destroy a view tree family.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ID | A number that identifies an existing MIB view. | N/A |

    - Example

prompt> snmp delete view All

    - See also

snmp add view

snmp set view

snmp clear views

snmp list views

snmp clear views

    - Syntax

snmp clear views

    - Description

Delete all of view tree families.

    - Example

prompt> snmp clear views

    - See also

snmp add view

snmp set view

snmp delete view

snmp list views

snmp list views

    - Syntax

snmp list views

    - Description

Display the configuration of view tree families.

    - Example

prompt> snmp list views

SNMP ViewTable:

ID  |   ViewName  |    OIDSubtree   | OIDMask |    Type

------|-------------|-----------------|---------|-------------

1  | All         | 1.3.6.1         | -       | included

--------------------------------------------------------------

    - See also

snmp add view

snmp set view

snmp delete view

snmp clear views

snmp add access

    - Syntax

snmp add access <groupname> v3 authNoPriv| noAuthNoPriv

    - Description

Create an access right for a group.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| groupname | A human readable string which is the group name. | N/A |
| authNoPriv | Security level for no authentication and no privacy. | N/A |
| noAuthNoPriv | Security level for MD5 authentication with no privacy. | N/A |

    - Example

prompt> snmp add access Admin v3 authNoPriv

    - See also

snmp set access

snmp delete access

snmp clear accesses

snmp list accesses

snmp set access

    - Syntax

snmp set access <ID> readview|writeview|notifyview <viewname>

    - Description

Set read, write or notification access right for the group.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ID | A number that identifies an existing access right. | N/A |
| readview | Identifying the view subtrees accessible for Get, GetNext and GetBulk requests. | - |
| writeview | Identifying the view subtrees accessible for Set requests. | - |
| notifyview | Identifying the view subtrees from which objects may be included as VarBinds in Trap messages and Inform requests. | - |
| viewname | A MIB view name define in ViewTable. ‘NULL’ is represented with a dash(-). | N/A |

    - Example

prompt> snmp set access 1 readview All

prompt> snmp set access 1 writeview All

prompt> snmp set access 1 notifyview -

    - See also

snmp add access

snmp delete access

snmp clear accesses

snmp list accesses

snmp delete access

    - Syntax

snmp delete access <ID>

    - Description

Destroy an access right for the group.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ID | A number that identifies an existing access right. | N/A |

    - Example

prompt> snmp delete access 1

    - See also

snmp add access

snmp set access

snmp clear accesses

snmp list accesses

snmp clear accesses

    - Syntax

snmp clear accesses

    - Description

Delete all of access rights.

    - Example

prompt> snmp clear accesses

    - See also

snmp add access

snmp set access

snmp delete accesses

snmp list accesses

snmp list accesses

    - Syntax

snmp list accesses

    - Description

Display the configuration of access rights.

    - Example

prompt> snmp list accesses

SNMP AccessTable:

ID  |                        Details

------|---------------------------------------------------------

1  | GroupName  Admin            ReadView   All

| WriteView  All              NotifyView -

| Security   authNoPriv       Model      snmpv3

------|---------------------------------------------------------

    - See also

snmp add access

snmp set access

snmp delete accesses

snmp clear accesses

snmp add user

    - Syntax

snmp add user <username>

    - Description

Create a SNMPv3 user.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| username | A human readable string repressenting the name of the user. | N/A |

    - Example

prompt> snmp add user private

    - See also

snmp set user

snmp delete user

snmp clear users

snmp list users

snmp set user

    - Syntax

snmp set user <username> targettag <targettagname>

snmp set user <username> authProtoco md5Auth|noAuth

snmp set user <username> authKey <authkey>

    - Description

targettag: A readable string that is used to select a set of entries in the targetaddr for source address checking or send trap.

authProtoco: Idicates whether messages sent on behalf of this user to or from the SNMP engine identified by EngineID can be authenticated, and if so, the type of authentication which is used.

authKey: A readable string representing the user’s authentication password. If authProtoco is noAuth, then this user does not have an authKey.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| username | A human readable string repressenting the name of the user. | N/A |
| targettagname | A readable string that is used for source address checking. If the SNMP entity should not perform source address checking, then this field should set to dash( - ). | - |
| md5Auth | Using the Hash-based Message Authentication Code (HMAC) with MD5 Message-Digest Algorithm(MD5) for authentication. | N/A |
| noAuth | No authentication. | N/A |
| authkey | A readable string representing the user’s authentication password. | password |

    - Example

prompt> snmp set user private targettag -

prompt> snmp set user private authProtoco md5Auth

prompt> snmp set user private authKey password

    - See also

snmp add user

snmp delete user

snmp clear users

snmp list users

snmp delete user

    - Syntax

snmp delete user <username>

    - Description

Destroy a  user.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| username | A human readable string repressenting the name of the user. | N/A |

    - Example

prompt> snmp delete user private

    - See also

snmp add user

snmp set user

snmp clear users

snmp list users

snmp clear users

    - Syntax

snmp clear users

    - Description

Delete all of users

    - Example

prompt> snmp clear users

    - See also

snmp add user

snmp set user

snmp delete user

snmp list users

snmp list users

    - Syntax

snmp list users

    - Description

Display the configuration of all users

    - Example

prompt> snmp list users

SNMP UserTable:

Name      |   TargetTag    |  AuthProtocol  |    AuthKey

----------------|----------------|----------------|----------------

private         |-               |MD5Auth         |password

-------------------------------------------------------------------

    - See also

snmp add user

snmp set user

snmp delete user

snmp clear users

snmp add group

    - Syntax

snmp add group <username> v3

    - Description

To assign a user to a group.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| username | A human readable string repressenting the name of the user that define in UseTable. | N/A |

    - Example

prompt> snmp add group private v3

    - See also

snmp set group

snmp delete group

snmp clear groups

snmp list groups

snmp set group

    - Syntax

snmp set group <ID> group <groupname>

    - Description

Assign the user to a group that is define in AccessTable.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ID | A number that identifies an existing GroupTable. | N/A |
| groupname | A human readable string repressenting the name of the group that define in AccessTable. | - |

    - Example

prompt> snmp set group 1 group Admin

    - See also

snmp add group

snmp delete group

snmp clear groups

snmp list groups

snmp delete group

    - Syntax

snmp delete group <ID>

    - Description

Destroy a  entry of GroupTable.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ID | A number that identifies an existing exnty of GroupTable. | N/A |

    - Example

prompt> snmp delete group 1

    - See also

snmp add group

snmp set group

snmp clear groups

snmp list groups

snmp clear groups

    - Syntax

snmp clear groups

    - Description

Delete all entries of GroupTable.

    - Example

prompt> snmp clear groups

    - See also

snmp add group

snmp set group

snmp delete groups

snmp list groups

snmp list groups

    - Syntax

snmp list groups

    - Description

Display the configuration of GroupTable.

    - Example

prompt> snmp list groups

SNMP GroupTable:

ID  |    UserName    |   GroupName    |  Model

------|----------------|----------------|--------

1  |private         |Admin           |snmpv3

-------------------------------------------------

    - See also

snmp add group

snmp set group

snmp delete groups

snmp clear groups

snmp add notify

    - Syntax

snmp add notify <notifyname>

    - Description

Create a notification for generate Trap or Inform message.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| notifyname | A readable string representing the name of this notification. | N/A |

    - Example

prompt> snmp add notify myTrap

    - See also

snmp set notify

snmp delete notify

snmp clear notifications

snmp list notifications

snmp set notify

    - Syntax

snmp set notify <notifyname> targettag <targettagname>

snmp set notify <notifyname> type trap|inform

    - Description

targettag: Used to select a set of entries in the TargetAddrTable.

type: Define the notification type (trap or inform).

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| notifyname | A readable string representing the name of this notification. | N/A |
| targettagname | A readable string that is used to select a set of entries in the TargetAddrTable | - |
| trap | SNMP trap | N/A |
| inform | SNMP inform | N/A |

    - Example

prompt> snmp set notify myTrap targettag wheremyTrapGo

prompt> snmp set notify myTrap type trap

    - See also

snmp add notify

snmp delete notify

snmp clear notifications

snmp list notifications

snmp delete notify

    - Syntax

snmp delete notify <notifyname>

    - Description

Destroy the entry of NotifyTable.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| notifyname | A readable string representing the name of this notification. | N/A |

    - Example

prompt> snmp delete notify myTrap

    - See also

snmp add notify

snmp set notify

snmp clear notifications

snmp list notifications

snmp clear notifications

    - Syntax

snmp clear notifications

    - Description

Delete all entries of NotifyTable.

    - Example

prompt> snmp clear notifications

    - See also

snmp add notify

snmp set notify

snmp delete notify

snmp list notifications

snmp list notifications

    - Syntax

snmp list notifications

    - Description

Display the configuration of NotifyTable.

    - Example

prompt> snmp list notifications

SNMP NotifyTable:

NotifyName   |   TargetTag    |  Type

----------------|----------------|--------

myTrap          |wheremyTrapGo   |trap

-----------------------------------------

    - See also

snmp add notify

snmp set notify

snmp delete notify

snmp clear notifications

snmp add targetaddr

    - Syntax

snmp add targetaddr <indexname>

    - Description

Create a TargetAddr entry for source address checking or notification’s target address.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| indexname | A readable string representing the name of this entry. | N/A |

    - Example

prompt> snmp add targetaddr index1

    - See also

snmp set targetaddr

snmp delete targetaddr

snmp clear targetaddresses

snmp list targetaddresses

snmp set targetaddr

    - Syntax

snmp set targetaddr <indexname> ipaddress <ipaddr> <mask>

snmp set targetaddr <indexname> targetparams <targetparamsname>

snmp set targetaddr <indexname> targettag <targettagname>

    - Description

ipaddress: IP address.

targetparams: A readable string that is used to select a set of entries in the TargetParamsTable.

targettag: The TargetTag correspond to the value of UserTable and NotifyTable.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| indexname | A readable string representing the name of this entry. | N/A |
| ipaddr | IP address. | 0.0.0.0 |
| mask | IP address mask. | 0.0.0.0 |
| targetparamsname | A readable string representing the name of TargetParams. | - |
| targettagname | A readable string representing the name of TargetTag | - |

    - Example

prompt> snmp set targetaddr index1 ipaddress 192.168.1.1 255.255.255.255

prompt> snmp set targetaddr index1 targetparams –

prompt> snmp set targetaddr index1 targettag -

    - See also

snmp add targetaddr

snmp delete targetaddr

snmp clear targetaddresses

snmp list targetaddresses

snmp delete targetaddr

    - Syntax

snmp delete <indexname>

    - Description

Destroy the entry of TargetAddrTable.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| indexname | A readable string representing the name of this entry. | N/A |

    - Example

prompt> snmp delete targetaddr index1

    - See also

snmp add targetaddr

snmp set targetaddr

snmp clear targetaddresses

snmp list targetaddresses

snmp clear targetaddresses

    - Syntax

snmp clear targetaddresses

    - Description

Delete all entries of TargetAddrTable.

    - Example

prompt> snmp clear targetaddresses

    - See also

snmp add targetaddr

snmp set targetaddr

snmp delete targetaddr

snmp list targetaddresses

snmp list targetaddresses

    - Syntax

snmp list targetaddresses

    - Description

Display the configuration of TargetAddrTable.

    - Example

prompt> snmp list targetaddresses

SNMP TargetAddrTable:

IndexName |   IP Address   |      Mask      | TargetTag  |TargetParams

------------|----------------|----------------|------------|------------

index1      |192.168.1.1     |255.255.255.255 |-           |-

------------------------------------------------------------------------

    - See also

snmp add targetaddr

snmp set targetaddr

snmp delete targetaddr

snmp clear targetaddresses

snmp add targetparams

    - Syntax

snmp add targetparams <targetparamsname>

    - Description

Create a TargetParams to be used when sending notifications.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| targetparamsname | A readable string representing the name of this entry. | N/A |

    - Example

prompt> snmp add targetparams params

    - See also

snmp set targetparams

snmp delete targetparams

snmp clear targetparams

snmp list targetparams

snmp set targetparams

    - Syntax

snmp set targetparams <targetparamsname> security authNoPriv|noAuthNoPriv

snmp set targetparams <targetparamsname> user <username>

    - Description

security: Identifies the security level of the notification to send.

user : Which user to be used in the notification

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| targetparamsname | A readable string representing the name of this entry. | N/A |
| authNoPriv | Security level for no authentication and no privacy. | N/A |
| noAuthNoPriv | Security level for MD5 authentication with no privacy. | N/A |
| username | A readable string representing the name of UserTable | - |

    - Example

prompt> snmp set targetparams params security noAuthNoPriv

prompt> snmp set targetparams params user private

    - See also

snmp add targetparams

snmp delete targetparams

snmp clear targetparams

snmp list targetparams

snmp delete targetparams

    - Syntax

snmp delete targetparams <targetparamsname>

    - Description

Destroy the entry of TargetParamsTable

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| targetparamsname | A readable string representing the name of this entry. | N/A |

    - Example

prompt> snmp delete targetparams params

    - See also

snmp add targetparams

snmp set targetparams

snmp clear targetparams

snmp list targetparams

snmp clear targetparams

    - Syntax

snmp clear targetparams

    - Description

Delete all entries of TargetParamsTable

    - Example

prompt> snmp clear targetparams

    - See also

snmp add targetparams

snmp set targetparams

snmp delete targetparams

snmp list targetparams

snmp list targetparams

    - Syntax

snmp list targetparams

    - Description

Display the configuration of TargetParamsTable

    - Example

prompt> snmp list targetparams

SNMP TargetParamsTable:

TargetParamsName|  Model |      User      |    Security

----------------|--------|----------------|----------------

params          |snmpv3  |private         |noAuthNoPriv

------------------------------------------------------------

    - See also

snmp add targetparams

snmp set targetparams

snmp delete targetparams

snmp clear targetparams

snmp status

    - Syntax

snmp status

    - Description

Display the information about SNMP version

    - Example

prompt> snmp status

SNMP Research SNMP Agent Resident Module Version 15.3.1.7

    - See also

None

Example:

You can refer the following examples to configuring your SNMP agent using SNMP CLI.

- Configuring a SNMPv1 or SNMPV2c user with community(public for read-only and password for read-write) and no source address checking.

SNMP Communities:

ID  |    Community   |    Ipaddress   | Access

------|----------------|----------------|---------

1  |public          |0.0.0.0         | read

2  |password        |0.0.0.0         | write

--------------------------------------------------

- Configuring a SNMPv1 trap receiver with ip address(192.168.1.1) and community is “public”.

SNMP Traps:

ID  |    Community   |    Ipaddress

------|----------------|----------------

1  |public          |192.168.1.1

----------------------------------------

- Configuring a SNMPV3 user(private) using MD5 for authentication, password is “password”, read-write for internet MIB(1.3.6.1) and only from 192.168.1.1

* Step1: Define a family of view subtrees(called internet).

SNMP ViewTable:

ID  |   ViewName  |    OIDSubtree   | OIDMask |    Type

------|-------------|-----------------|---------|-------------

1  | internet    | 1.3.6.1         | -       | included

--------------------------------------------------------------

* Step2: Define a group(called Admin) and its associated access rights(both of ReadView and WriteView access rights are internet).

SNMP AccessTable:

ID  |                        Details

------|---------------------------------------------------------

1  | GroupName  Admin            ReadView   internet

| WriteView  internet         NotifyView -

| Security   authNoPriv       Model      snmpv3

------|---------------------------------------------------------

* Step3: Assign a SNMPv3 user(called private) to the group defined in step 2.

SNMP UserTable:

Name      |   TargetTag    |  AuthProtocol  |    AuthKey

----------------|----------------|----------------|----------------

private         |checkIP         |MD5Auth         |password

-------------------------------------------------------------------

SNMP GroupTable:

ID  |    UserName    |   GroupName    |  Model

------|----------------|----------------|--------

1  |private         |Admin           |snmpv3

-------------------------------------------------

* Step4: For source address checking (192.168.1.1)

SNMP TargetAddrTable:

IndexName |   IP Address   |      Mask      | TargetTag  |TargetParams

------------|----------------|----------------|------------|------------

index1      |192.168.1.1     |255.255.255.255 |checkIP     |-

------------------------------------------------------------------------

- Configuring a SNMPv3 notification(trap) with ip address(192.168.1.1) and user name is private.

* Step1: Define a notification.

SNMP NotifyTable:

NotifyName   |   TargetTag    |  Type

----------------|----------------|--------

index           |TrapTag         |trap

-----------------------------------------

* Step2: Define a set of network addresses to which a notification should be sent.

SNMP TargetAddrTable:

IndexName |   IP Address   |      Mask      | TargetTag  |TargetParams

------------|----------------|----------------|------------|------------

index1      |192.168.1.1     |255.255.255.255 |TrapTag     |TrapParams

------------------------------------------------------------------------

* Step3: Define parameters to use when sending notifications to each of the target addressed identified in step 2.

SNMP TargetParamsTable:

TargetParamsName|  Model |      User      |    Security

----------------|--------|----------------|----------------

TrapParams      |snmpv3  |private         |noAuthNoPriv

------------------------------------------------------------

SNMP ViewTable:

ID  |   ViewName  |    OIDSubtree   | OIDMask |    Type

------|-------------|-----------------|---------|-------------

1  | internet    | 1.3.6.1         | -       | included

--------------------------------------------------------------

SNMP AccessTable:

ID  |                        Details

------|---------------------------------------------------------

1  | GroupName  TrapGroup        ReadView   -

| WriteView  -                NotifyView internet

| Security   noAuthNoPriv     Model      snmpv3

------|---------------------------------------------------------

SNMP GroupTable:

ID  |    UserName    |   GroupName    |  Model

------|----------------|----------------|--------

1  |private         |TrapGroup       |snmpv3

-------------------------------------------------

SNMP UserTable:

Name      |   TargetTag    |  AuthProtocol  |    AuthKey

----------------|----------------|----------------|----------------

private         |-               |NoAuth          |password

-------------------------------------------------------------------