- IGMP CLI commands

This chapter describes the Internet Group Management Protocol (IGMP) CLI commands.

Summary

    - IGMP CLI commands

The table below lists the IGMP commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| igmp set forwardall | igmp set forwardall |
| igmp set upstreaminterface | igmp set upstreaminterface |
| igmp show upstreaminterface | igmp show upstreaminterface |
| igmp show forwardall | igmp show forwardall |
| igmp show status | igmp show status |

igmp set forwardall

    - Syntax

igmp set forwardall {enabled|disabled}

    - Description

This command allows you to enable/disable your router’s ability to forward multicast traffic to ALL interfaces. By default, multicast traffic is only forwarded to interfaces on which there is IGMP Proxy group membership.

Note – This command setting is only effective if you have already enabled IGMP Proxy using igmp set upstreaminterface.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | Your router forwards multicast traffic to all interfaces |  |
| disabled | Your router only forwards multicast traffic to interfaces on which there are IGMP Proxy grou members. | N/A |

    - Example

prompt> igmp set forwardall enabled

    - See also

igmp show forwardall

igmp set upstreaminterface

    - Syntax

igmp set upstreaminterface {<ip_interface> | none}

    - Description

This command enables the router’s IGMP Proxy, and sets one of the router’s existing IP interfaces as the upstream interface; all other router interfaces are designated downstream interfaces. The upstream interface implements the Host portion of the IGMP protocol, and the downstream interfaces implement the Router portion of the IGMP protocol. The IGMP Proxy may be disabled by setting upstream interface to none.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ip_interface | The name of an existing router interface that you want to set as the upstreaminterface. | N/A |
| none | Disables IGMP proxy | N/A |

    - Example

prompt> igmp set upstreaminterface ip1

    - See also

igmp show upstreaminterface

    - Syntax

igmp show upstreaminterface

    - Description

This command displays the status of the upstream interface. If an upstream interface has been set using the igmp set upstreaminterface command, this command displays the current setting.

    - Example

prompt> igmp show upstreaminterface

IGMP Proxy configuration

Upstream If : ip1

    - See also

igmp set upstreaminterface

igmp show forwardall

    - Syntax

igmp show forwardall

    - Description

This command displays status information about the igmp set forwardall command setting. It ells you whether or not the router is set to forward multicast traffic to all interfaces.

    - Example

prompt> igmp show forwardall

IGMP Forwarder::

Forward All : false

    - See also

igmp show upstreaminterface

igmp show status

    - Syntax

igmp show status

    - Description

This command displays the following information about the status of IGMP:

- IGMP Proxy group membership per interface details

- Interface name and querier status

- Group address

    - Example

prompt> igmp show status

Multicast group membership:

Interface (querier) | Group address

---------------------|-----------------

eth0          (yes) | 239.255.255.250

r1483         (yes) | 224.0.1.101

r1483         (yes) | 224.0.1.102

r1483         (yes) | 224.0.1.103

---------------------------------------

    - See also

igmp show upstreaminterface