- Transports CLI commands

This chapter describes the Transports CLI commands

Summary

This module allows you to clear, delete, list and display information about existing transports that were created using the <transport_module> add transport commands. To carry out more detailed configuration of transports, see the corresponding transport module chapter:

For RFC1483 commands, see RFC1483 CLI commands

For PPPoA commands, see PPPoA CLI commands

For PPPoE commands, see PPPoE CLI commands

For Ethernet commands, see Ethernet CLI commands

For IPoA commands, see IPoA CLI commands

    - Transports CLI commands

The table below lists the Transports commands provided by the CLI:

| Command | Reference |
| --- | --- |
| transports clear | transports clear |
| transports delete | transports delete |
| transports list | transports list |
| transports set classifer disabled | transports set classifier disabled |
| transports set classifer profile | transports set classifier profile |
| transports show | transports show |

transports clear

    - Syntax

transports clear

    - Description

This command deletes all transports that were created using the <transport_module> add transport command.

    - Example

prompt> transports clear

    - See also

transports delete

transports delete

    - Syntax

transports delete {<name>|<number>}

    - Description

This command deletes a single transport that was created using the <transport_module> add transport command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the transports list command. | N/A |
| number | A number that identifies an existing transport. To display transport numbers, use the transports list command. | N/A |

    - Example

prompt> transports delete eth1

    - See also

transports clear

transports list

transports list

    - Syntax

transports list

    - Description

This command lists all transports created during a session. It displays the following information about the transports:

- transport identification number

- transport name

- transport type (RFC1483, PPP, Ethernet, or IPoA)

- Number of transmitted/received packets for each transport

- VPI/VCI setting (RFC1483, PPP and IPoA transports only)

    - Example

prompt> transports list

Services:

ID | Name    | Type

---|---------|-------------------------------------------------

1 | rfc1483 | RFC1483 | TxPkts:0/0  RxPkts:0/0  VPI/VCI:0/100

2 | pppoh2  | PPP     | TxPkts:0/0  RxPkts:0/0  VPI/VCI:0/101

3 | pppoh1  | PPP     | TxPkts:0/0  RxPkts:0/0  VPI/VCI:0/102

4 | pppoa2  | PPP     | TxPkts:0/0  RxPkts:0/0  VPI/VCI:0/103

5 | eth0    | Ethernet| TxPkts:0/0  RxPkts:0/0

---------------------------------------------------------------

    - See also

transports show

transports set classifier disabled

    - Syntax

transports set {<name>|<number>} classifier disabled

    - Description

This command disables packet classification previously set on a specified transport using the transports set classifier profile command. The classifier is removed from the data path.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the transports list command. | N/A |
| number | A number that identifies an existing transport. To display transport numbers, use the transports list command. | N/A |

    - Example

prompt> transports set myrfc classifier disabled

    - See also

transports set classifier profile

transports list

transports set classifier profile

    - Syntax

transports set {<name>|<number>} classifier profile{<profile name>|<profile number>}

    - Description

This command sets an existing classifier profile on an existing transport. All rules that exist in the profile will test incoming packets on the specified transport.

The same profile can be added to more than one transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the transports list command. | N/A |
| number | A number that identifies an existing transport. To display transport numbers, use the transports list command. | N/A |
| profile name | A name that identifies an existing profile. To display profile names, use the classifier list profiles command. | N/A |
| profile number | A number that identifies an existing profile. To display profile numbers, use the classifier list profiles command. | N/A |

    - Example

prompt> transports set myrfc classifier profile p1

    - See also

transports set classifier disabled

transports list

classifier add profile

classifier list profiles

transports show

    - Syntax

transports show {<name>|<number>}

    - Description

This command displays detailed information about an existing transport. The information displayed depends on the transport type selected. See below for examples of PPP and RFC1483 transport information.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the transports list command. | N/A |
| number | A number that identifies an existing transport. To display transport numbers, use the transports list command. | N/A |

    - Examples

Example One - PPP transport

prompt> transports show pppoa1

PPP Status

PPP

Summary              : disabled

Server               : true

Create Route         : true

Specific Route       : false

Subnet Mask          : 0.0.0.0

Route Mask           : 0.0.0.0

Hdlc                 : false

LLC                  : false

Lcp Max Configure    : 10

Lcp Max Failure      : 5

Lcp Max Terminate    : 2

Dialin Auth          : none

Dialout Username     :

Dialout Password     :

Dialout Auth         : none

Interface ID         : 2

Magic Number         : 0

MRU                  : 0

SVC                  : false

Remote Atm           :

Ip Addr From IPCP    : true

Give DNSto Relay     : true

Give DNSto Client    : true

Lcp Echo Every       : 10

If In Octets         : 0

If Out Octets        : 0

If In Errors         : 0

If Out Errors        : 0

Packets Sent         : 0

Good Packets Received : 0

Enabled              : false

Termination          :

Hdlc Channel

Port                 : hdlc

Example Two - RFC1483 transport

prompt> transports show myrfc1483

RFC1483 Status

RFC1483

Mode                 : LlcBridged

If In Octets         : 0

If Out Octets        : 0

If In Errors         : 0

If Out Errors        : 0

Packets Sent         : 0

Good Packets Received : 0

Enabled              : true

Atm Channel

Tx Vci               : 600

Rx Vci               : 600

Peak Cell Rate       : 2000

Class                : UBR

Port                 : a1

    - See also

transports list