- System CLI commands

This chapter describes the System CLI commands.

Summary

    - System CLI commands

The table below lists the system commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| system add user | system add user |
| system add login | system add login |
| system config backup | system config backup |
| system config clear | system config clear |
| system config restore | system config restore |
| system config save | system config save |
| system delete login | system delete login |
| system delete user | system delete user |
| system info | system info |
| system legal | system legal |
| system list errors | system list errors |
| system list openfiles | system list openfiles |
| system list users | system list users |
| system list logins | system list logins |
| system log enable¦disable | system log enable¦disable |
| system log dump¦flush | system log dump¦flush |
| system log save¦remove | system log save¦remove |
| system log set | system log set |
| system log list | system log list |
| system mail | system mail |
| system restart | system restart |
| system set login access | system set login access |
| system set login mayconfigure | system set login mayconfigure |
| system set login maydialin | system set login maydialin |
| system set user access | system set user access |
| system set user mayconfigure | system set user mayconfigure |
| system set user maydialin | system set user maydialin |
| system set user password | system set user password |
| system syslog set | system syslog set |
| system syslog show | system syslog show |
| system trace enable¦disable | system trace enable¦disable |
| system trace dump¦flush | system trace dump¦flush |
| system trace list | system trace list |
| system trace savt | system trace save |
| system trace show¦unshow | system trace show¦unshow |

    - System Console commands

The table below lists the top-level and configuration console commands and, if available, their equivalent CLI command:

| Command | CLI Equivalent |
| --- | --- |
| restart | Replaced by CLI command system restart |
| version | Usable command, see version |
| event | Usable command, see event … |
| . (history mechanism) | Usable command, see . (history mechanism) |
| list | Usable command, see list |
| echo | Usable command, see echo … |
| buildid | Usable command, see buildid |
| help | Usable command, see help |
| update | Usable command |
| @ commands | Blacklisted command, see @ commands |
| <process>,<process>,<co mmand> | Blacklisted command, see <process>, <process> <command> |
| tell | Blacklisted command, see tell <process> … |
| exit, exit! | Blacklisted command, see exit, exit! |
| debug | Blacklisted command, see debug |
| crlf, nocrlf | Blacklisted command, see crlf, nocrlf |
| bind, unbind | Blacklisted command, see bind <process>, unbind |
| config list | Blacklisted command, see list |
| config print | Blacklisted command |
| config reset | Replaced by CLI command system config restore |
| config save | Replaced by CLI command system config save |
| config resource | Blacklisted command |
| config version | Blacklisted command, see version |

system add user

    - Syntax

system add user <name> [“comment”]

    - Description

This command adds a user (typically a PPP user) to the system. Only a Super user can use this command.

    - Default setting

The default setting in the table below are applied to new accounts added using the system add user command. (A different set of defaults are applied to a new account added using the system add login command.)

| Option | Default Setting |
| --- | --- |
| dialin to the system | enabled |
| login to the system | disabled |
| configuration permissions | disabled |
| access permissions | default user |

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A unique user name made up of more than one character that identifies an individual user and lets the user access the system. | N/A |
| comment | An optional comment about the user that is displayed when you type the command system list users and system list logins. | No comment added |

    - Example

prompt> system add user simon

    - See also

system set user access

system set user maydialin

system set user mayconfigure

system list users

system delete user

system add login

    - Syntax

system add login <name> [“comment”]

    - Description

This command adds a user to the system. Only a Super user can use this command.

    - Default setting

The default setting in the table below are applied to new accounts added using the system add login command. (A different set of defaults are applied to a new account added using the system add user command.)

| Option | Default Setting |
| --- | --- |
| dialin to the system | disabled |
| login to the system | enabled |
| configuration permissions | enabled |
| access permissions | default user |

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A unique login name made up of more than one character that identifies an individual user and lets the user access the system. | N/A |
| comment | An optional comment about the user that is displayed when you type the command system list users and system list logins. | Blank (No comment added) |

    - Example

prompt> system add login ckearns “temporary contractor”

    - See also

system delete login

system list logins

system config backup

    - Syntax

system config backup [filename]

    - Description

This command saves the system configuration to a file. To specify the file that you want to save configuration information in, type //isfs/ or //flashfs/ (depending on which directory the backup file is stored in) followed by a filename value. If you do not specify a filename, the configuration is saved in the //isfs/im.conf.backup file by default.

To prevent a user from overwriting the system with their own configuration, only a Super user can use this command. You had better not to save to //flashfs/ if possible, due to flash ROM is scarce resource.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| filename | The name of an existing file that you want to save your configuration in. The filename MUST be preceded by either //isfs/ or //flashfs/. | If a filename is not specified, the configuration is saved in //isfs/im.conf.backup. |

    - Examples

prompt> system config backup

Saving to backup configuration //isfs/im.conf.backup

prompt> system config backup //flashfs/mybackup

Saving to backup configuration //flashfs/mybackup

    - See also

system config restore

system config save

system config clear

    - Syntax

system config clear

    - Description

This command clears your current configuration. Attributes are set to their defaults, and all interfaces and transports are deleted. The default admin user is not affected, so you can still login to the system.

This command has the same effect as entering the system config restore minimal command.

    - Example

prompt> system config clear

dhcpserver: Reset request acknowledged. Reset imminent.

Restoring backup configuration //isfs/im.conf.minimal

    - See also

system config restore

system config restore

    - Syntax

system config restore {backup [filename]|factory|minimal}

    - Description

This command tries to restore all system modules; if you do not have all modules installed, the CLI will display a message telling you which modules could not be restored. The following options are available:

- Super users, Engineers and Default users can restore their backup configuration from the //isfs/im.conf.backup file.

- Super users can restore their backup configuration from a different file by typing //isfs/ or //flashfs/ (depending on which directory the backup file is stored in) followed by a filename value. See system config backup .

- Super users can restore the factory defaults from //isfs/im.conf.factory.

- Super users can clear their current configuration by restoring the //isfs/im.conf.minimal. Attributes are set to their defaults, and all interfaces and transports are deleted. The default admin user is not affected, so you can still login to the system.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| backup | Restores the backup configuration from the //isfs/im.conf.backup file. | N/A |
| filename | The name of a file containing an alternative backup configuration to that stored in the //isfs/im.conf.backup file. The filename MUST be preceded by either //isfs/ or //flashfs/. Only Super users can use this command. | N/A |
| factory | Restores the factory default configuration from the //isfs/im.conf.factory file. Only Super users can use this command. | N/A |
| minimal | Clears the current configuration by resetting attributes to their defaults and deleting interfaces and transports. Only Super users can use this command. | N/A |

    - Example

prompt> system config restore backup

Restoring backup configuration //isfs/im.conf.backup

    - See also

system config backup

system config save

system config save

    - Syntax

system config save

    - Description

This command saves the system configuration in the im.conf file in FlashFS. This allows all users to create their own backup files. Default, Engineer and Super users can use this command.

    - Example

prompt> system config save

    - See also

system config backup

system config restore

Super users can list all configuration files using the console command fm ls.

system delete login

    - Syntax

system delete login <name>

    - Description

This command deletes a user that has been added to the system using the system add login command. Only a Super user can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing user. | N/A |

    - Example

prompt> system delete login ckearns

    - See also

system add login

system delete user

    - Syntax

system delete user <name>

    - Description

This command deletes a user that has been added to the system using the system add user command or the system add login command. Only a Super user can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing user. | N/A |

    - Example

prompt> system delete user ckearns

    - See also

system add user

system add login

system info

    - Syntax

system info

    - Description

This command displays the vendor ID, URL, base MAC address and hardware and software version details of the system that you are using.

    - Example

prompt> system info

Global System Configuration:

Hardware ver:	DSL Multiple-function PCB (Annex A), 1.00

Software ver:	4.30

Product Name:	DSL Firewall Router

Hostname:	home.gateway

Language:	en

Vendor:	company

Vendor URL:	http://www.company.com

MAC address:	##:##:##:##:##:##

Note:

    - See also

system legal

    - Syntax

system legal

    - Description

This command displays copyright information about the software that you are using.

    - Example

prompt> system legal

Check your distributor.

system list errors

    - Syntax

system list errors

    - Description

This command displays a system error log. The error log contains the following information:

- the time (in minutes) that an error was made, calculated from the start of your session

- the module that was affected by the error

- a brief overview of the error itself

    - Example

prompt> system list errors

Error log:

When    |    Who     |    What

------------|------------|-------------------------------------------------

104        | webserver  | webserver:Failed to create node type 'ImRfc1483'

104        | webserver  | webserver:Invalid argument:Failed to open port a4 (may already be in use, or invalid port name)

---------------------------------------------------------------------------

    - See also

system list users

system list logins

system trace

system list openfiles

    - Syntax

system list openfiles <name>

    - Description

This command allows you to display low-level debug information about specific open file handles.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of a file which has open file handles associated with it. | N/A |

    - Example

prompt> system list openfiles bun

qid        devuse   appuse   colour   flags    lasterrno

console    0000004b 00000000 00400000 3        0

console    00000027 00000000 00400000 5        0

console    00000003 00000000 00400000 5        0

    - See also

system log enable|disable

system list users

    - Syntax

system list users

    - Description

This command displays a list of users and logins added to the system using the system add user and system add login commands. The same information is displayed by the system list logins command.

The list contains the following information:

- user ID number

- user name

- configuration permissions (enabled or disabled)

- dialin permissions (enabled or disabled)

- access level (default, engineer or super user)

- comment (any comments that were included when the user was added to the system)

    - Example

prompt> system list users

Users:

May        May       Access

ID  |   Name     |  Conf.   |  Dialin  |   Level    |  Comment

-----|------------|----------|----------|------------|---------------------

1 | admin      | ENABLED  | disabled | superuser  | Default admin user

---------------------------------------------------------------------------

    - See also

system list errors

system list logins

system list logins

    - Syntax

system list logins

    - Description

This command displays a list of logins and users added to the system using the system add login and system add user commands. The same information is displayed by the system list users command.

The list contains the following information:

- user ID number

- user name

- configuration permissions (enabled or disabled)

- dial in permissions (enabled or disabled)

- access level (default, engineer or super user)

- comment (any comments that were included when the user was added to the system)

    - Example

prompt> system list logins

Users:

May        May       Access

ID  |   Name     |  Conf.   |  Dialin  |   Level    |  Comment

-----|------------|----------|----------|------------|--------------------

1 | admin      | ENABLED  | disabled | superuser  | Default admin user

--------------------------------------------------------------------------

    - See also

system list errors

system list users

system log

    - Description

The processes (or said modules) in system will generate event logs. These event logs are classified into different log levels. This system log command group is used control the system log feature.

system log enable|disable

    - Syntax

system log {enable|disable} <processname> to {console|syslog}

    - Description

This command control where the logging output desinated to. By default, all of logging output will stored in system memory buffer. You can check the latest logs using system log dump command. Besides writing to system memory buffer, user can display the logging output in console port or send to a designated syslog daemon at the same time.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enable | Enables the duplication of logging to specified output. | N/A |
| disable | Disables the duplication of logging to the specified output. |  |
| processname | The name of process that generate log. You can find out the process supported in system log list command. |  |
| console | The output is Console Port |  |
| syslog | The output is syslog daemon |  |

    - Examples

prompt> system log enable ppp to console

enabled log to console

    - See also

system log list

system log set

system syslog

system log dump|flush

    - Syntax

system log dump

system log flush

    - Description

The system log dump command will display the logs stored in system memory buffer in console port. The information is displayed as below format:

<system time> <router name>:<process name>:<log level>: <message content>

While the system log flush command will clear all logs in system memory buffer

    - Examples

Example One

prompt> system log dump

----------- system log buffer head ---------------

Jan 01 00:01:48 home.gateway:gs7070:none: ADSL line is DOWN!

----------- system log buffer tail ---------------

Example Two

prompt> system log flush

system log buffer flushed

    - See also

system log list

system log enable|disable

system log save|remove

    - Syntax

system log save [<filename>]

system log remove <filename>

    - Description

The system log save command will save the current content of system log buffer into a temporary file in //isfs/. If the filename is not provided, this command will use the default filename, eventlog. These files will disappear when router power off. You can display the file content using fm cat console command.

The system log remove command will the file from //isfs/. Please be careful about the filename; otherwise, might delete some important, un-related file.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| filename | The file name will be accessed in //isfs/ filing system. | N/A |

    - Examples

Example One – save to file with default file name as ‘eventlog’

prompt> system log save

Example Two – save to file as mylog

prompt> system log save mylog

Example Three – delete the file named ‘mylog’

prompt> system log remove mylog

    - See also

system log list

system log set

system log set

    - Syntax

system log set <processname> {all|error|info|nothing|warning}

    - Description

This command controls the level of logging information will be output for a sepcified process. Process will generate logs in different information level and the logs will writeto system log buffer only when the log level is greater than or equal to current level. Users can use this command to control the details of logging he need.

‘info’ level will output the logs with info/warning/error types. Currently, it is same as ‘all’.

‘warning’ level will output the logs with warning/error types.where the logging output

‘error’ level will only output the logs with error type.

‘nothing’ will disable all of log output except for the unconditional log type.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| processname | The name of process that generate log. You can find out the process supported in system log list command. |  |
| all | All type of logs. | nothing |
| error | Error type of logs. |  |
| info | Info type of logs. |  |
| nothing | Disable logs. |  |
| warning | Warning type of logs. |  |

    - Example

prompt> system log set ppp info

    - See also

system log list

system log enable|disable

system log list

    - Syntax

system log list

    - Description

The system log list command displays the event log options for the modules available in the current image that you are using. It displays the process name, log level and log destination.

    - Example

prompt> system log list

ProcessName         Level        LogTo

snmpr               none         none

ppp                 info         console

firewall            none         syslog

ddns                none         none

    - See also

system log

system log enable|disable

system mail

    - Syntax

system mail file <filename> to <taddr> return-path <raddr>

    - Description

This command is used to send a file located in //isfs/ to a specified email address. Due to the system cannot receive emails by itself, you can specified the return-path whenever the mail fails to deliver.

Normally, it is used accompanied with event log and trace log commands, but no limited to. You had better to check the internet access is working when using this command for sending email successfully.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| filename | The file located is //isfs/ and will be send out. | N/A |
| taddr | The ‘To’ address of sending email and it is full email address, like user@mail.com. | N/A |
| raddr | This email address will embedded in the email header and used when the email fail to deliver. | N/A |

    - Example

prompt> system mail file eventlog to user@company.com return-path backup@company.com

    - See also

system log

system trace

system restart

    - Syntax

system restart [factory]

    - Description

This command restarts your system. It has the same effect as pressing the reset button on your system. The optional factory keyword will force the system to delete current saved configuration file such that the next restart will be with factory setting.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| factory | Delete the saved configuration file in //flashfs/ and then restart the system with factory setting. | N/A |

    - Example

prompt> system restart

    - See also

system config restore

system set login access

    - Syntax

system set login <name> access {default|engineer|superuser}

    - Description

This command sets the access permissions of a user who has been added to the system using the system add login command. Only a Super user can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing user. | N/A |
| default/ engineer/ superuser | Access permissions for a user. | default |

    - Example

prompt> system set login ckearns access engineer

    - See also

system set login mayconfigure

system set login maydialin

system set login mayconfigure

    - Syntax

system set login <name> mayconfigure {enabled|disabled}

    - Description

This command sets configuration permissions for a user who has been added to the system using the add system login or the add system user command. Only a Super user can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing user. | N/A |
| enabled/ disabled | Determines whether or not a user can configure the system. | enabled |

    - Example

prompt> system set login ckearns mayconfigure disabled

    - See also

system set login access

system set login maydialin

system set login maydialin

    - Syntax

system set login <name> maydialin {enabled|disabled}

    - Description

This command sets dial in permissions for a user who has been added to the system using the system add login command. Only a Super user can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing user. | N/A |
| enabled/ disabled | Determines whether or not a user can dialin to the system. | disabled |

    - Example

prompt> system set login ckearns maydialin enabled

    - See also

system set login access

system set login mayconfigure

system set nickname

    - Syntax

system set nickname <ident>

    - Description

This command sets the system nickname. The nickname will appear in syslog, event log, trace log, and can be used as home URL when you start to access the system embedded web server via ‘http://nickname’ if you use system DNS relay feature.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ident | The unique name for the system. | Home.gateway |

    - Example

prompt> system set nickname router@home

    - See also

system info

system set user access

    - Syntax

system set user <name> access {default|engineer|superuser}

    - Description

This command sets the access permissions of a user who has been added to the system using the system add user command. Only a Super user can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing user. | N/A |
| default/ engineer/ superuser | Allows you to set the access permissions for a user. | default |

    - Example

prompt> system set user ckearns access default

    - See also

system set user mayconfigure

system set user maydialin

system set user mayconfigure

    - Syntax

system set user <name> mayconfigure {enabled|disabled}

    - Description

This command sets configuration permissions for a user who has been added to the system using the add system user command. Only a Super user can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing user. | N/A |
| enabled/ disabled | Determines whether or not a user can configure the system. | disabled |

    - Example

prompt> system set user ckearns mayconfigure enabled

    - See also

system set user access

system set user maydialin

system set user maydialin

    - Syntax

system set user <name> maydialin {enabled|disabled}

    - Description

This command sets dial in permissions for a user who has been added to the system using the system add user command. Only a Super user can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing user. | N/A |
| enabled/ disabled | Determines whether or not a user can dialin to the system. | enabled |

    - Example

prompt> system set user ckearns maydialin enabled

    - See also

system set user access

system set user mayconfigure

system set user password

    - Syntax

system set user <name> password <password>

    - Description

This command sets the user password that was previously created using the user password command. Only a Super user can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | The name of an existing user. | N/A |
| password | A unique password made up of more than one character that identifies an individual user and lets the user access PPP services. | N/A |

    - Example

prompt> system set user ckearns password sydney

    - See also

user password

system syslog set

    - Syntax

system syslog set daemon <host>

    - Description

This command configures the syslog daemon’s IP address. It is used with system log command group.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| host | The IP address of syslog daemon in the format as 80.21.32.00. | 0.0.0.0 |

    - Example

prompt> system syslog set daemon 80.12.32.1

    - See also

system log set

system syslog show

system syslog show

    - Syntax

system syslog show

    - Description

This command displays the current syslog daemon’s IP address and the ident used in syslog message. The ident is configured in system set nickname command.

    - Example

prompt> system syslog show

syslog daemon: 81.23.21.1

syslog ident: home.gateway

    - See also

system log se

system set nicknamet

system syslog set

system trace

    - Description

This command group is designed for debugging purpose. Process may generate trace logs and stores in system trace buffer. Using this command group, you can control trace buffer, trace level and related processes

A similar command group is system log, but it is used for different purpose.

system trace enable|disable

    - Syntax

system trace {enable|disable} <module> <category>

    - Description

This command enables/disables which module, what kinds of trace category can be generated. When enabled, the trace log will be output to system trace buffer in ring-buffer manner. You can find out module name and category name using system trace list command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| module | Module name or process name in the system image. | N/A |
| category | Variant category names used for each module. Need to check system trace list comand for details. |  |

    - Examples

prompt> system trace enable ip rawip

enabled logging for ip rawip

    - See also

system trace list

system trace dump|flush

    - Syntax

system trace dump <dumpsize>

system trace flush

    - Description

The system trace dump command will display the trace logs stored in system trace buffer in console port. Due to the buffer could be very large, you can use <dumpsize> to display the latest kilobytes data in trace buffer. If <dumpsize> option is omitted, the whole trace buffer content will be display in console port.

The information is displayed as below format:

<system time> <router name>:

<process name>:<category> <message content>

While the system trace flush command will clear all trace logs in system trace buffer

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| dumpsize | The maximum data size (in kilobytes) you want to display in console port.. | N/A |

    - Examples

Example One

prompt> system trace dump

----------- system trace buffer head ---------------

Jan 01 00:01:48 home.gateway:

Iprecv: rawip i/f 1: rx 58b from 192.168.1.101 to 192.168.1.255 proto UDP

----------- system trace buffer tail ---------------

Example Two

prompt> system trace flush

system trace buffer flushed

    - See also

system trace list

system trace enable|disable

system trace list

    - Syntax

system trace list

    - Description

The system trace list command displays the available modules and corresponding category types in the system image and the current trace configuration for each module

    - Example

prompt> system trace list

Module         Category        (Status)

upload         info            (disabled)

upload         preserve        (disabled)

upload         get             (disabled)

ip             socket          (disabled)

ip             config          (disabled)

ip             rawip           (ENABLED)

    - See also

system trace enable|disable

system trace show|unshow

system trace save

    - Syntax

system trace save

    - Description

The system trace save command will save the current content of system trace buffer into //isfs/tracelog. These file will disappear when router power off. You can display the file content using fm cat console command.

    - Example

prompt> system trace save

    - See also

system trace enable|disable

system trace list

system trace show|unshow

    - Syntax

system trace show

system trace unshow

    - Description

This command controls whether the trace logs displayed in the console port or not when writing to system trace buffer. It is used for real-time monitoring.

    - Example

prompt> system trace show

    - See also

system trace list

system trace enable|disable