- Console Access CLI commands

This chapter describes the console access CLI commands.

Summary

    - Console access CLI commands

The table below lists the console access commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| console enable | console enable |
| console process | console process |

    - Console Commands

The table below lists the access console commands:

| Command | CLI status |
| --- | --- |
| exit | Blacklisted command, see Console command - exit |

console enable

    - Syntax

console enable

    - Description

This command allows you to enter console mode in order to use the console commands. Only Super users can use this command.

    - Example

prompt> console enable

Switching from CLI to console mode - type ‘exit’ to return

    - See also

console process

Console command - exit

console process

    - Syntax

console process <console command>

    - Description

This command allows you to enter a single usable console command without switching to console mode. You cannot enter blacklisted console commands using this CLI command. Users with Engineer or Super user access can use this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| console command | A usable console command. You can find a list of usable commands with a link to further information about each usable command at the start of each chapter in this manual. | N/A |

    - Example

The following console process example enters the usable console command, bridge portfilter:

prompt> console process bridge portfilter

portfilter 2 all

portfilter 3 all

    - See also

console enable if you have Super user access and want to enter more than one console command or use blacklisted commands.

The console commands that you can use with the console process command are listed in atmos/products/include/flashfs/cliconsole.

Console command - exit

Note - This console command has not been replaced by a CLI command. This is a special console command to allow Super users to return to the CLI from the console.

    - Syntax

exit

    - Description

This console command allows you to return to the CLI after you have entered console mode using the command console enable. When you want to exit console mode and return to the CLI, you need to type exit in the root of the console.

Only Super users can use this command.

    - Example

prompt> exit

Returning to CLI from console

    - See also

console enable