- PPPoA CLI commands

This chapter describes the PPP over ATM CLI commands.

Summary

    - PPP CLI commands

The table below lists the PPP commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| pppoa add transport dialin pvc | pppoa add transport dialin pvc |
| pppoa add transport dialout pvc | pppoa add transport dialout pvc |
| pppoa clear transports | pppoa clear transports |
| pppoa delete transport | pppoa delete transport |
| pppoa list transports | pppoa list transports |
| pppoa set transport atmaddress | pppoa set transport atmaddress |
| pppoa set transport autoconnect | pppoa set transport autoconnect |
| pppoa set transport bt | pppoa set transport bt |
| pppoa set transport createroute | pppoa set transport createroute |
| pppoa set transport dialin pvc | pppoa set transport dialin pvc |
| pppoa set transport dialout pvc | pppoa set transport dialout pvc |
| pppoa set transport discoverdns primary | pppoa set transport discoverdns primary |
| pppoa set transport discoverdns secondary | pppoa set transport discoverdns secondary |
| pppoa set transport discovernbns primary | pppoa set transport discovernbns primary |
| pppoa set transport discovernbns secondary | pppoa set transport discovernbns secondary |
| pppoa set transport discoversubnetmask | pppoa set transport discoversubnetmask |
| pppoa set transport enabled¦disabled | pppoa set transport enabled¦disabled |
| pppoa set transport givedns client | pppoa set transport givedns client enabled¦disabled |
| pppoa set transport givedns dhcpserver | pppoa set transport givedns dhcpserver enabled¦disabled |
| pppoa set transport givedns relay | pppoa set transport givedns relay enabled¦disabled |
| pppoa set transport givesubnetmask dhcpserver | pppoa set transport givesubnetmask dhcpserver enabled¦disabled |
| pppoa set transport headers hdlc | pppoa set transport headers hdlc |
| pppoa set transport headers llc | pppoa set transport headers llc |
| pppoa set transport interface | pppoa set transport interface |
| pppoa set transport lcpechoevery | pppoa set transport lcpechoevery |
| pppoa set transport lcpmaxconf | pppoa set transport lcpmaxconf |
| pppoa set transport lcpmaxfail | pppoa set transport lcpmaxfail |
| pppoa set transport lcpmaxterm | pppoa set transport lcpmaxterm |
| pppoa set transport localip | pppoa set transport localip |
| pppoa set transport mbs | pppoa set transport mbs |
| pppoa set transport mcr | pppoa set transport mcr |
| pppoa set transport password | pppoa set transport password |
| pppoa set transport pcr | pppoa set transport pcr |
| pppoa set transport port | pppoa set transport port |
| pppoa set transport prilevels | pppoa set transport prilevels |
| pppoa set transport qosclass | pppoa set transport qosclass |
| pppoa set transport remotedns | pppoa set transport remotedns |
| pppoa set transport remoteip | pppoa set transport remoteip |
| pppoa set transport routemask | pppoa set transport routemask |
| pppoa set transport scr | pppoa set transport scr |
| pppoa set transport specificroute | pppoa set transport specificroute |
| pppoa set transport subnetmask | pppoa set transport subnetmask |
| pppoa set transport theylogin | pppoa set transport theylogin |
| pppoa set transport username | pppoa set transport username |
| pppoa set transport vci | pppoa set transport vci |
| pppoa set transport vpi | pppoa set transport vpi |
| pppoa set transport welogin | pppoa set transport welogin |
| pppoa show transport | pppoa show transport |

    - PPP Console commands

The table below lists the ppp console commands and, if available, their equivalent CLI command:

| Command | CLI Equivalent |
| --- | --- |
| ppp <channel> clear | Blacklisted command, see <channel> clear |
| ppp <channel> disable | Blacklisted command, see <channel> disable |
| ppp <channel> discard | Usable command, see <channel> discard |
| ppp <channel> echo | Usable command, see <channel> echo |
| ppp <channel> echo every | Usable command, see <channel> echo every |
| ppp <channel> enable | Blacklisted command, see <channel> enable |
| ppp <channel> event | Usable command, see <channel> event |
| ppp <channel> hdlc | Replaced by CLI command pppoa set transport headers hdlc, using the hdlc option. |
| ppp <channel> info | Replaced by CLI command pppoa show transport . |
| ppp <channel> interface | Replaced by CLI command pppoa set transport interface |
| ppp <channel> lcpmaxconfigure | Replaced by CLI command pppoa set transport lcpmaxconf |
| ppp <channel> lcpmaxfailure | Replaced by CLI command pppoa set transport lcpmaxfail |
| ppp <channel> lcpmaxterminate | Replaced by CLI command, see pppoa set transport lcpmaxterm |
| ppp <channel> llc | Replaced by CLI command pppoa set transport headers llc, using the llc option. |
| ppp <channel> pvc | Replaced by CLI command pppoa set transport dialin pvc and pppoa set transport dialout pvc |
| ppp <channel> qos | Replaced by CLI command pppoa set transport qosclass, pppoa set transport pcr, pppoa set transport bt |
| ppp <channel> remoteip | Replaced by CLI command pppoa set transport remoteip. |
| ppp <channel> theylogin | Replaced by CLI command pppoa set transport theylogin |
| ppp <channel> tunnel | Blacklisted command, see <channel> tunnel <n> <tunnel protocol> <dial direction> |
| ppp <channel> welogin | Replaced by CLI command pppoa set transport welogin |
| ppp bcp | Replaced by CLI command pppoa set transport headers hdlc, using the hdlc option |
| ppp interface - localip | Replaced by CLI command pppoa set transport localip |
| ppp interface - stats | Replaced by CLI command pppoa show transport |
| ppp user | Blacklisted command, see user |
| ppp version | Replaced by CLI command pppoa show transport |

pppoa add transport dialin pvc

    - Syntax

pppoa add transport <name> dialin pvc <interface> <port> <vpi> <vci>

    - Description

This command creates a PPPoA transport that accepts dialin connections over a PVC (Permanent Virtual Circuit). It allows you to specify the following information:

- the PPP interface to the channel that the PVC will use

- the ATM port that will transport data

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the transport. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interface | The PPP interface to a channel that transports PPPoA data. A single interface can be used by multiple channels. The interface value can be any positive integer. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over.The VCI can be any value between 1 and 65535. | N/A |

    - Example

prompt> pppoa add transport pppoa1 dialin pvc 1 a1 0 800

    - See also

pppoa list transports

pppoa set transport vci

pppoa set transport vpi

port list

pppoa add transport dialout pvc

    - Syntax

pppoa add transport <name> dialout pvc <interface> <port> <vpi> <vci>

    - Description

This command creates a PPPoA transport that performs dialout over a PVC (Permanent Virtual Circuit). It allows you to specify the following PVC information:

- the PPP interface to the channel that the PVC will use

- the ATM port that will transport data

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the transport. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interface | The PPP interface to a channel that transports PPPoA data. A single interface can be used by multiple channels. The interface value can be any positive integer. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over.The VCI can be any value between 1 and 65535. | N/A |

    - Example

prompt> pppoa add transport pppoa1 dialout pvc 1 a1 0 800

    - See also

pppoa list transports

port list

pppoa clear transports

    - Syntax

pppoa clear transports

    - Description

This command deletes all PPPoA transports that were created using the pppoa add transport commands.

    - Example

prompt> pppoa clear transports

    - See also

pppoa delete transport

pppoa delete transport

    - Syntax

pppoa delete transport {<name>|<number>}

    - Description

This command deletes a single PPPoA transport. The PVC attached to the transport is also deleted.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |

    - Example

prompt> pppoa delete transport pppoa1

    - See also

pppoa list transports

pppoa list transports

    - Syntax

pppoa list transports

    - Description

This command lists PPPoA transports that have been created using the pppoa add transport commands. It displays the following information about the transports:

- transport identification number

- transport name

- ATM port used (if applicable)

- Virtual Circuit Identifier (VCI) used (if applicable)

- Virtual Path Identifier (VPI) used (if applicable)

    - Example

prompt> pppoa list transports

PPPoA transports:

ID  |    Name    | Port      | Vci       | Vpi

-----|------------|-----------|-----------|-----------

1 | p2         | N/A       | N/A       | N/A

2 | p1         | a1        | 800       | 0

------------------------------------------------------

    - See also

pppoa show transport

pppoa set transport atmaddress

    - Syntax

pppoa set transport {<name>|<number>} autoconnect <atmaddr>

    - Description

This command allows you to set  the PPPoA remote ATM address?.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| atmaddr | Remote ATM address. | N/A |

    - Example

prompt> pppoa set transport pppoa1 atmaddress 6

    - See also

pppoa list transports

pppoa set transport autoconnect

    - Syntax

pppoa set transport {<name>|<number>} autoconnect {enabled|disabled

    - Description

This command allows you to enable/disable the PPPoA autoconnect function. If enabled, PPPoA automatically connects to TCP/IP whenever a user requests TCP/IP packets from a public destination.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | Enables PPPoA autoconnect. | disabled |
| disabled | Disables PPPoA autoconnect. |  |

    - Example

prompt> pppoa set transport pppoa1 autoconnect enabled

    - See also

pppoa list transports

pppoa set transport bt

    - Syntax

pppoa set transport {<name>|<number>} bt <burst tolerance>

    - Description

This command applies to existing PVC transports - it does not apply  to SVC transports. This command sets the burst tolerance (bt) for an existing PPPoA transport. This command is only valid if you set VBR or VBR RT as the QoS Class using the pppoa set transport qosclass command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| burst tolerance | Controls the duration of traffic bursts on VBR (Variable Bit Rate) and VBR RT (VBR Real Time) channels. This value overrides an existing MBS value (if set). The BT can be any value between 0 and 100. | 0 |

    - Example

prompt> pppoa set transport pppoa1 bt 5

    - See also

pppoa set transport mbs

pppoa set transport createroute

    - Syntax

pppoa set transport {<name>|<number>} createroute {enabled|disabled}

    - Description

This command specifies whether a route is added to the system after IPCP (Internet Protocol Control Protocol) negotiation is completed. If set to enabled, a route will be created which directs packets to the remote end of the PPP link.

This route can either be a default route or a specific route, depending on the value set using the pppoa set transport specificroute command.

To display the createroute setting, use the pppoa show transport command. The route is removed when the PPP link is disconnected.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| enabled | Adds a route to the system after IPCP negotiation. | enabled |
| disabled | Does not add a route to the system after IPCP negotiation. |  |

    - Example

prompt> pppoa set transport pppoa1 createroute disabled

    - See also

pppoa show transport

pppoa set transport specificroute

pppoa set transport dialin pvc

    - Syntax

pppoa set transport {<name>|<number>} dialin pvc <port> <vpi> <vci>

    - Description

This command sets an existing PPPoA transport to accept dialin connections over a PVC (Permanent Virtual Circuit). This replaces the transports existing dialin/dialout setting over PVC. The PVC uses the interface that was specified when the transport was created.

The command allows you to specify the following PVC information:

- the ATM port that will transport data

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over.The VCI can be any value between 1 and 65535. | N/A |

    - Example

prompt> pppoa set transport pppoa2 dialin pvc a1 0 800

    - See also

pppoa list transports

pppoa set transport dialout pvc

port list

pppoa set transport dialout pvc

    - Syntax

pppoa set transport {<name>|<number>} dialout pvc <port> <vpi> <vci>

    - Description

This command sets a PPPoA transport to perform dialout over a PVC (Permanent Virtual Circuit). This replaces the transports existing dialin/dialout setting over PVC. The PVC uses the interface that was specified when the transport was created.

The command allows you to specify the following PVC information:

- the ATM port that will transport data

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over.The VCI can be any value between 1 and 65535. | N/A |

    - Example

prompt> pppoa set transport pppoa2 dialout pvc a1 0 800

    - See also

pppoa list transports

pppoa set transport dialin pvc

port list

pppoa set transport discoverdns primary

    - Syntax

pppoa set transport {<name>|<number>} discoverdns primary {enabled|disabled}

    - Description

Note - You must enable one of the pppoa set transport givedns commands in order for this command setting to work. See pppoa set transport givedns client enabled|disabled, or pppoa set transport givedns relay enabled|disabled

This command enables/disables whether the primary DNS server address is requested from a remote PPP peer using IPCP. The default setting for this command is enabled. The default setting for the pppoa set transport givedns commands is also enabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | A primary DNS server IP address is requested. | enabled |
| disabled | A primary DNS server IP address is not requested. |  |

    - Example

prompt> pppoa set transport pppoa3 discoverdns primary enabled

    - See also

pppoa set transport discoverdns secondary

pppoa set transport givedns client enabled|disabled

pppoa set transport givedns relay enabled|disabled

pppoa set transport remotedns

pppoa set transport discoverdns secondary

    - Syntax

pppoa set transport {<name>|<number>} discoverdns secondary {enabled|disabled}

    - Description

Note - You must enable one of the pppoa set transport givedns commands in order for this command setting to work. See pppoa set transport givedns client enabled|disabled, or pppoa set transport givedns relay enabled|disabled

This command enables/disables whether the secondary DNS server address is requested from a remote PPP peer using IPCP. The default setting for this command is enabled. The default setting for the pppoa set transport givedns commands is also enabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | A secondary DNS server IP address is requested. | enabled |
| disabled | A secondary DNS server IP address is not requested. |  |

    - Example

prompt> pppoa set transport pppoa3 discoverdns secondary enabled

    - See also

pppoa set transport discoverdns primary

pppoa set transport givedns client enabled|disabled

pppoa set transport givedns relay enabled|disabled

pppoa set transport remotedns

pppoa set transport discovernbns primary

    - Syntax

pppoa set transport {<name>|<number>} discovernbns primary {enabled|disabled}

    - Description

This command enables/disables whether the primary NBNS (NetBIOS Name Server) server address is requested from a remote PPP peer using IPCP. The default setting for this command is disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | A primary NBNS server IP address is requested. | disabled |
| disabled | A primary NBNS server IP address is not requested. |  |

    - Example

prompt> pppoa set transport pppoa3 discovernbns primary enabled

    - See also

pppoa set transport discovernbns secondary

pppoa set transport discovernbns secondary

    - Syntax

pppoa set transport {<name>|<number>} discovernbns secondary {enabled|disabled}

    - Description

This command enables/disables whether the secondary NBNS server address is requested from a remote PPP peer using IPCP. The default setting for this command is disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | A secondary NBNS server IP address is requested. | disabled |
| disabled | A secondary NBNS server IP address is not requested. |  |

    - Example

prompt> pppoa set transport pppoa3 discovernbns secondary enabled

    - See also

pppoa set transport discovernbns primary

pppoa set transport discoversubnetmask

    - Syntax

pppoa set transport {<name>|<number>} discoversubnetmask {enabled|disabled}

    - Description

This command enables/disables whether the netmask option is requested from a remote PPP peer using IPCP. The default setting for this command is disabled. Some of PPP peer can assign an IP pool instead of a single IP address.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | A netmask option is requested. | disabled |
| disabled | A netmask option is not requested. |  |

    - Example

prompt> pppoa set transport pppoa3 discoversubnetmask enabled

    - See also

pppoa set transport discoverdns primary

pppoa set transport enabled|disabled

    - Syntax

pppoa set transport {<name>|<number>} {enabled|disabled}

    - Description

This command explicitly enables/disables a PPPoA transport. Attaching a transport to an interface implicitly enables it, but for cases where no attach is performed (for example, multiple channels on an interface, a PPP session that is not attached but needed for testing purposes) the transport must be enabled explicitly.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | Enables a PPPoA transport. | disabled |
| disabled | Disables a PPPoA transport. |  |

    - Example

prompt> pppoa set transport pppoa1 enabled

    - See also

pppoa list transports

pppoa set transport givedns client enabled|disabled

    - Syntax

pppoa set transport {<name>|<number>} givedns client {enabled | disabled}

    - Description

This command controls once IPCP has discovered the DNS server IP address, it automatically gives the address to the local DNS client so that a connection can be established.

You must have the DNS client process included in your image build in order to use this protocol.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | IPCP can request a DNS server IP address and then give the address to DNS client. | enabled |
| disabled | IPCP cannot request a DNS server IP address and then give the address to DNS client. |  |

    - Example

prompt> pppoa set transport pppoa1 givedns client enabled

    - See also

pppoa set transport givedns relay enabled|disabled

pppoa set transport remotedns

pppoa set transport discoverdns primary

pppoa set transport discoverdns secondary

DNS Client CLI commands

pppoa set transport givedns dhcpserver enabled|disabled

    - Syntax

pppoa set transport {<name>|<number>} givedns dhcpserver {enabled | disabled}

    - Description

This command controls once IPCP has discovered the DNS server IP address, it automatically gives the address to the DNCP server so that the local DHCP clients can get the DNS server IP address.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | IPCP can request a DNS server IP address and then give the address to DHCP server. | enabled |
| disabled | IPCP cannot request a DNS server IP address and then give the address to DHCP server. |  |

    - Example

prompt> pppoa set transport pppoa1 givedns dhcpserver enabled

    - See also

pppoa set transport givedns relay enabled|disabled

pppoa set transport remotedns

pppoa set transport discoverdns primary

pppoa set transport discoverdns secondary

pppoa set transport givedns relay enabled|disabled

    - Syntax

pppoa set transport {<name>|<number>} givedns relay {enabled | disabled}

    - Description

This command controls once IPCP has discovered the DNS server IP address, it automatically gives the address to the local DNS relay so that a connection can be established.

You must have the DNS relay process included in your image build in order to use this protocol.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing pppoa transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | IPCP can request a DNS server IP address and then give the address to DNS relay. | enabled |
| disabled | IPCP cannot request a DNS server IP address and then give the address to DNS relay. |  |

    - Example

prompt> pppoa set transport pppoa1 givedns relay enabled

    - See also

pppoa set transport givedns client enabled|disabled

pppoa set transport remotedns

pppoa set transport discoverdns primary

pppoa set transport discoverdns secondary

DNS Relay CLI commands

pppoa set transport givesubnetmask dhcpserver enabled|disabled

    - Syntax

pppoa set transport {<name>|<number>} givesubnetmask dhcpserver {enabled | disabled}

    - Description

This command controls once IPCP has discovered the netmask option from PPP peer, it automatically gives the netmask to the local DHCP server so that the DHCP server creates a new IP pools for leasing.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing pppoa transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | Give the netmask to DHCP server. | disabled |
| disabled | Cannot give the address to DHCP server. |  |

    - Example

prompt> pppoa set transport pppoa1 givesubnetmask dhcpserver enabled

    - See also

pppoa set transport discoversubnetmask enabled|disabled

pppoa set transport headers hdlc

    - Syntax

pppoa set transport {<name>|<number>} headers hdlc {enabled|disabled}

    - Description

This command allows you to enable/disable whether your system can transmit and receive packets containing HDLC headers. If you want LLC packets to be transmitted and received instead of/as well as HDLC packets, use the pppoa set transport headers llc enabled command.

When both HDLC and LLC headers are disabled, the default encapsulation method is VC multiplexed (VC Mux). PPP determines which format to use to transmit/receive packets by ‘learning’ the format information from incoming packet headers.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | Packets that have HDLC headers can be transmitted/received. | disabled (if LLC headers are disabled too, the default value is VC Mux) |
| disabled | Packets that have HDLC headers can not be transmitted/received. |  |

    - Example

prompt> pppoa set transport pppoa1 headers hdlc enabled

    - See also

pppoa list transports

pppoa show transport

pppoa set transport headers llc

pppoa set transport headers llc

    - Syntax

pppoa set transport {<name>|<number>} headers llc {enabled|disabled}

    - Description

This command allows you to enable/disable whether your system can transmit and receive packets containing LLC headers. If you want HDLC packets to be transmitted and received instead of/as well as LLC packets, use the pppoa set transport headers hdlc enabled command.

When both LLC and HDLC headers are disabled, the default encapsulation method is VC multiplexed (VC Mux). PPP determines which format to use to transmit/receive packets by ‘learning’ the format information from incoming packet headers.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | Packets that have LLC headers can be transmitted/received. | disabled (if HDLC headers are disabled too, the default value is VC Mux) |
| disabled | Packets that have LLC headers can not be transmitted/received. |  |

    - Example

prompt> pppoa set transport pppoa1 headers llc enabled

    - See also

pppoa list transports

pppoa show transport

pppoa set transport headers hdlc

pppoa set transport idletimeout

    - Syntax

pppoa set transport {<name>|<number>} idletimeout <idletimeout>

    - Description

This command allows you to set an ‘idle’ time out for your LAN connection. If you are connected to an ISP via PPPoA but fail to send a request for data within a specified time limit, the PPPoA session is disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| idletimeout | The length of time ( in minutes) that a PPPoA session connected to an ISP can remain idle before the session is disabled. The time can be any value between 0 and 60. A value of 0 means that no idletimeout is set. | 0 |

    - Example

prompt> pppoa set transport pppoa1 idletimeout 20

    - See also

pppoa list transports

pppoa set transport lcpechoevery

pppoa set transport interface

    - Syntax

pppoa set transport {<name>|<number>} interface <interface>

    - Description

This command sets the PPP interface for an existing PPPoA transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| interface | The PPP interface to a channel that transports PPPoA data. A single interface can be used by multiple channels. The interface value can be any positive integer. | N/A |

    - Example

prompt> pppoa set transport pppoa2 interface 4

    - See also

pppoa show transport

pppoa list transports

pppoa set transport lcpechoevery

    - Syntax

pppoa set transport {<name>|<number>} lcpechoevery <interval>

    - Description

This command tells a specified PPP transport to send an LCP (Link Control Protocol) echo request frame at specified intervals (in seconds). If no reply to the request is received, the PPP connection is torn down. This functionality is also known as ‘keep-alive’.

If you do not want to send LCP echo frames, specify zero (0) in the <interval> attribute.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| interval | The length of time (in seconds) between LCP echo request frames being sent. If you do not want echo request frames to be sent, specify ‘0’ as the interval. | 10 seconds |

    - Example

prompt> pppoa set transport pppoa2 lcpechoevery 0

    - See also

pppoa show transport

pppoa list transports

pppoa set transport lcpmaxconf

    - Syntax

pppoa set transport {<name>|<number>} lcpmaxconf <lcp max configure>

    - Description

This command sets the Link Control Protocol (LCP) maximum configure number for an existing PPPoA transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| lcp max configure | Link Control Protocol; the maximum number of configures that can be transmitted without reply before assuming that the destination address is unable to respond. The LCPmaxconf can be any positive value. | 10 |

    - Example

prompt> pppoa set transport pppoa1 lcpmaxconf 20

    - See also

pppoa show transport

pppoa list transports

pppoa set transport lcpmaxfail

    - Syntax

pppoa set transport {<name>|<number>} lcpmaxfail <lcp max fail>

    - Description

This command sets the Link Control Protocol (LCP) maximum fail number for an existing PPPoA transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| lcp max fail | Link Control Protocol; the maximum number of consecutive negative acknowledgements (indicating that the information received contains errors) that can be transmitted before assuming that parameter negotiation is not converging. The LCPmaxfail can be any positive value. | 5 |

    - Example

prompt> pppoa set transport pppoa1 lcpmaxfail 20

    - See also

pppoa show transport

pppoa list transports

pppoa set transport lcpmaxterm

    - Syntax

pppoa set transport {<name>|<number>} lcpmaxterm <lcp max terminate>

    - Description

This command sets the Link Control Protocol (LCP) maximum terminate number for an existing PPPoA transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| lcp max term | Link Control Protocol; the maximum number of consecutive Terminate Requests that will be sent without reply before assuming that the destination address is unable to respond. The LCPfailterm can be any positive value. | 2 |

    - Example

prompt> pppoa set transport pppoa1 lcpmaxterm 20

    - See also

pppoa show transport

pppoa list transports

pppoa set transport localip

    - Syntax

pppoa set transport {<name>|<number>} localip <ip-address>

    - Description

This command is only applicable to dialin PVC transports that provide the server- end of a connection. The command tells the PPP process the local IP address to be associated with the client-end of an interface. This allows remote users to have dialin access via the channel(s) that the interface is attached to.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| ip-address | The ip address of the local ‘client-end’ of an interface displayed in the following format: 111.222.254.4 | 0.0.0.0 |

    - Example

prompt> pppoa set transport pppoa1 localip 192.168.103.2

    - See also

pppoa show transport

pppoa list transports

pppoa set transport remoteip

pppoa set transport mbs

    - Syntax

pppoa set transport {<name>|<number>} mbs <max burst size>

    - Description

This command applies to existing PVC transports - it does not apply  to SVC transports. It sets the maximum burst size (mbs) for the PPPoA transport.

This command is only valid if you set VBR or VBR RT as the QoS Class using the pppoa set transport quosclass command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| maximum burst size | Controls the maximum burst size for VBR (Variable Bit Rate) and VBR RT (VBR Real Time) channels. This value overrides an existing BT value (if set). The MBS can be any value between 0 and 100. | 0 |

    - Example

prompt> pppoa set transport pppoa3 mbs 10

    - See also

pppoa set transport bt

pppoa set transport qosclass

pppoa set transport mcr

    - Syntax

pppoa set transport {<name>|<number>} mcr <min cell rate>

    - Description

This command applies to existing PVC transports - it does not apply  to SVC transports. This command sets the minimum cell rate for an existing PPPoA transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| minimum cell rate | Determines the minimum rate at which ATM cells are allowed to be sent along the PPPoA transport. | 0 |

    - Example

prompt> pppoa set transport pppoa2 mcr 0

    - See also

pppoa set transport pcr

pppoa set transport password

    - Syntax

pppoa set transport {<name>|<number>} password <password>

    - Description

This command sets a dial-out password on a named transport. The password is required when PPP negotiation takes place and is supplied to the remote PPP server for authentication.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| password | An arbitrary word that acts as a dialout password enabling you to login to the remote end. The password will be required by the PPP server when you want to login remotely. It can be made up of one or more characters and/or digits. To display the password, use the pppoa show transport command. | N/A |

    - Example

prompt> pppoa set transport pppoa2 password mercury

    - See also

pppoa list transports

pppoa show transport

pppoa set transport username

pppoa set transport pcr

    - Syntax

pppoa set transport {<name>|<number>} pcr <peak cell rate>

    - Description

This command applies to existing PVC transports - it does not apply  to SVC transports. This command sets the peak cell rate (pcr) for an existing PPPoA transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| peak cell rate | Determines the maximum rate at which ATM cells are allowed to be sent along the PPPoA transport. The PCR can be any value from 0 up to the maximum PortSpeed parameter set when the port was created (using the initbun file in FlashFS or the CLI command port list). | 0 |

    - Example

prompt> pppoa set transport pppoa2 pcr 50000

    - See also

pppoa set transport mcr

pppoa set transport port

    - Syntax

pppoa set transport {<name>|<number>} port <port>

    - Description

This command applies to existing PVC transports - it does not apply  to SVC transports. This command sets the port that an existing transport uses to transport PPPoA data.

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| port | The system port that is used to transport ATM data. | N/A |

    - Example

prompt> pppoa set transport pppoa4 port a1

    - See also

pppoa list transports

port list

pppoa set transport prilevels

    - Syntax

pppoa set transport {<name>|<number>} prilevels <prilevels>

    - Description

This command enables support for multiple packet priority levels on the same ATM VC. By default, the system supports two prilevel values:

- 1: there are no multiple priority levels enabled so the feature is disabled.

- 2: packets with different priorities set (such as best effort and hight priority traffic) are prioritized on the same ATM VC.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| prilevels | The number of priority levels available on an ATM transport. | 1 |

    - Example

prompt> pppoa set transport pppoa4 prilevels 2

    - See also

pppoa show transport

pppoa set transport pvc

    - Syntax

pppoa set transport {<name>|<number>} pvc <port> <vpi> <vci>

    - Description

This command sets the PVC information fro an existing PPPoA transport PVC. The PVC uses the interface that was specified when the transport was created.

The command allows you to specify the following PVC information:

- the ATM port that will transport data

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| Vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |
| vci | Part of ATM header. The VCI is a tag that identifies which channel a cell will travel over. The VCI can be any value between 1 and 65535. | N/A |

    - Example

prompt> pppoa set transport pppoa4 pvc a1 0 800

    - See also

pppoa list transports

pppoa set transport dialin pvct

pppoa set transport dialout pvc

port list

pppoa set transport qosclass

    - Syntax

pppoa set transport {<name>|<number>} qosclass {ubr|cbr|vbr|vbrrt|abr|qfc}

    - Description

This command applies to existing PVC transports - it does not apply  to SVC transports. It sets the quality of service class for the transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| ubr | Unspecified Bit Rate; non-constant and unpredictable data transport rate. PCR (Peak Cell Rate) is the average and maximum speed of transmission. | UBR |
| cbr | Constant Bit Rate; constant demand and predictable data transport rate. PCR is the average and maximum speed of transmission. |  |
| vbr | Variable Bit Rate; non-constant but predictable data transport rate that uses Non-Real-Time (NRT). You can specify the PCR, SCR, BT and MBS for VBR traffic. |  |
| vbrrt | Variable Bit Rate Real-Time; non- constant but predictable data transport rate that uses Real-Time (RT). You can specify the PCR, SCR, BT and MBS for VBRRT traffic. |  |
| abr | Available Bit Rate; non-constant and unpredictable data transport rate that provides ATM-layer feedback and flow control. |  |
| qfc | QFC; ATM flow control protocol that supports ABR. |  |

    - Example

prompt> pppoa set transport pppoa3 qosclass abr

    - See also

pppoa show transport

pppoa set transport qosclass

pppoa set transport remotedns

    - Syntax

pppoa set transport {<name>|<number>} remotedns <ipaddress> [<ipaddress2>]

    - Description

This command is a PPP server function.

This command sets the primary and secondary local DNS server addresses that will be given to a remote PPP peer when the peer requests a primary or secondary DNS server IP address using IPCP. Setting the secondary IP address is optional.

If you want to delete an IP address, set the IP address to 0.0.0.0.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing pppoa transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing pppoa transport. To display transport numbers, use the pppoa list transports command. | N/A |
| ipaddress | The ip address of the primary local DNS server displayed in the following format: 192.168.102.3 | 0.0.0.0 (no primary address set) |
| ipaddress2 | The ip address of the secondary local DNS server displayed in the following format: 192.168.102.3 | 0.0.0.0 (no secondary address set) |

    - Examples

Example One - setting a primary address

prompt> pppoa set transport pppoa1 remotedns 192.168.102.3

Example Two - setting primary and secondary addresses

To set primary and secondary addresses, use this command syntax:

prompt> pppoa set transport pppoa1 remotedns 192.168.102.3 192.168.105.1

Example Three - deleting an address

To delete an address, set it to 0.0.0.0. The example below deletes the secondary address that was set in Example Two:

prompt> pppoa set transport pppoa1 remotedns 192.168.102.3 0.0.0.0

    - See also

pppoa set transport givedns client enabled|disabled

pppoa set transport givedns relay enabled|disabled

pppoa set transport discoverdns primary

pppoa set transport discoverdns secondary

pppoa set transport remoteip

    - Syntax

pppoa set transport {<name>|<number>} remoteip <ip-address>

    - Description

This command sets the IP address supplied to the remote end of the PPP connection during negotiation. This is particularly important for PPP dialin transports.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| ip-address | The ip address of the local ‘server-end’ of an interface displayed in the following format: 111.222.254.4 | 0.0.0.0 |

    - Example

prompt> pppoa set transport pppoa1 remoteip 192.168.103.2

    - See also

pppoa show transport

pppoa list transports

pppoa set transport localip

pppoa set transport remotenbns

    - Syntax

pppoa set transport {<name>|<number>} remotenbns <ipaddress> [<ipaddress2>]

    - Description

This command is a PPP server function.

This command sets the primary and secondary local NBNS server addresses that will be given to a remote PPP peer when the peer requests a primary or secondary NBNS server IP address using IPCP. Setting the secondary IP address is optional.

If you want to delete an IP address, set the IP address to 0.0.0.0.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing pppoa transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing pppoa transport. To display transport numbers, use the pppoa list transports command. | N/A |
| ipaddress | The ip address of the primary local NBNS server displayed in the following format: 192.168.102.3 | 0.0.0.0 (no primary address set) |
| ipaddress2 | The ip address of the secondary local NBNS server displayed in the following format: 192.168.102.3 | 0.0.0.0 (no secondary address set) |

    - Examples

Example One - setting a primary address

prompt> pppoa set transport pppoa1 remotenbns 192.168.102.3

Example Two - setting primary and secondary addresses

To set primary and secondary addresses, use this command syntax:

prompt> pppoa set transport pppoa1 remotenbns 192.168.102.3 192.168.105.1

Example Three - deleting an address

To delete an address, set it to 0.0.0.0. The example below deletes the secondary address that was set in Example Two:

prompt> pppoa set transport pppoa1 remotenbns 192.168.102.3 0.0.0.0

    - See also

pppoa set transport discovernbns primary

pppoa set transport discovernbns secondary

pppoa set transport routemask

    - Syntax

pppoa set transport {<name>|<number>} routemask <mask>

    - Description

This command sets the subnet mask used by the route that is created when a PPP link comes up. If it is set to 0.0.0.0, the subnet mask is determined by the IP address of the remote end of the link. The class of the IP address is obtained during IPCP (Internet Protocol Control Protocol) negotiation.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| mask | The subnet mask that is used for the route that is created when a PPP link comes up. 0.0.0.0 | 0.0.0.0 |

    - Example

prompt> pppoa set transport pppoa1 routemask 0.0.0.0

    - See also

pppoa show transport

pppoa list transports

pppoa set transport scr

    - Syntax

pppoa set transport {<name>|<number>} scr <sustainable cell rate>

    - Description

This command applies to existing PVC transports - it does not apply  to SVC transports. This command is only valid if you set VBR or VBR RT as the QoS Class using the pppoa set transport qosclass command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| sustainable cell rate | Sustainable Cell Rate; the average cell rate for a VBR or VBR RT connection. The SCR can be any positive value that is less than the PortSpeed and the PCR for the channel. | 0 |

    - Example

prompt> pppoa set transport pppoa2 scr 25000

    - See also

pppoa set transport qosclass

pppoa set transport specificroute

    - Syntax

pppoa set transport {<name>|<number>} specificroute {enabled | disabled}

    - Description

Note - This command is only valid if the pppoa set transport createroute command is set to enabled. If the createroute command is set to disabled, no route is created and therefore the pppoa set transport specificroute command is ignored.

This command specifies whether the route created when a PPP link comes up is a specific or default route. If set to enabled, the route created will only apply to packets for the subnet at the remote end of the PPP link. The address of this subnet is obtained during IPCP negotiation.

The mask for the route is calculated from the class of the remote subnet unless an alternative has been specified using the pppoa set transport routemask command. If specificroute is set to disabled, a default route to the subnet at the remote end of the PPP link is created.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| enabled | Allows the created route to apply to packets for the subnet at the remote end of the PPP link. | disabled |
| disabled | A default route to the subnet at the remote end of the PPP link is created. |  |

    - Example

prompt> pppoa set transport pppoa1 specificroute disabled

    - See also

pppoa set transport routemask

pppoa set transport createroute

pppoa list transports

pppoa show transport

pppoa set transport subnetmask

    - Syntax

pppoa set transport {<name>|<number>} subnetmask <mask>

    - Description

This command sets the subnet mask used for the local IP interface connected to the PPP transport. If the value 0.0.0.0 is supplied, the netmask will be calculated from the class of the IP address obtained during IPCP negotiation.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| mask | The subnet mask used by the local IP interface connected to the PPP transport. | 0.0.0.0 |

    - Example

prompt> pppoa set transport pppoa1 subnetmask 255.255.255.0

    - See also

pppoa list transports

pppoa set transport theylogin

    - Syntax

pppoa set transport {<name>|<number>} theylogin {none|pap|chap}

    - Description

This command sets the authentication method that remote PPP clients must use to dialin to the server. If authentication is used, clients must use the specified authentication method and provide the username set using the system add user command.

This command is only valid if the user has maydialin set using the system set login maydialin command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| none | No authentication method is set. | None |
| pap | Password Authentication Protocol; the server sends an authentication request to the remote user dialing in. PAP passes the unencrypted username and password and identifies the remote end. |  |
| chap | Challenge Handshake Authentication Protocol; the server sends an authentication request to the remote user dialing in. PAP passes the encrypted username and password and identifies the remote end. |  |

    - Example

prompt> pppoa set transport pppoa2 theylogin pap

    - See also

pppoa list transports

pppoa show transport

system add user

system set user maydialin

pppoa set transport username

    - Syntax

pppoa set transport {<name>|<number>} username <username>

    - Description

This command sets a (dial-out) username on a named transport. The username is required when PPP negotiation takes place and is supplied to the remote PPP server for authentication.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| username | A name that identifies a user and, together with the dialout password, enables a user to login to the remote end. The username will be required by the PPP server when the user wants to login remotely. It can be made up of one or more characters and/or digits. To display the username, use the pppoa show transport command. | N/A |

    - Example

prompt> pppoa set transport pppoa2 username jsmith

    - See also

pppoa set transport password

pppoa set transport vci

    - Syntax

pppoa set transport {<name>|<number>} vci <vci>

    - Description

This command applies to existing PVC transports - it does not apply  to SVC transports.This command sets the Virtual Circuit Identifier.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over. The VCI can be any value between 1 and 65535. | N/A |

    - Example

prompt> pppoa set transport pppoa4 vci 800

    - See also

pppoa list transports

pppoa show transport

pppoa set transport vpi

pppoa set transport vpi

    - Syntax

pppoa set transport {<name>|<number>} vpi <vpi>

    - Description

This command applies to existing PVC transports - it does not apply  to SVC transports.This command sets the Virtual Path Identifier.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |

    - Example

prompt> pppoa set transport pppoa3 vpi 0

    - See also

pppoa list transports

pppoa show transport

pppoa set transport vci

pppoa set transport welogin

    - Syntax

pppoa set transport {<name>|<number>} welogin {none|auto|pap|chap}

    - Description

This command sets the authentication protocol used to connect to external PPP servers (dial- out).

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |
| none | No authentication method is used. | None |
| auto | The authentication protocol used by the remote PPP server is discovered and used. |  |
| pap | Password Authentication Protocol; the server sends an authentication request to the remote user dialing in. PAP passes the unencrypted username and password and identifies the remote end. |  |
| chap | Challenge Handshake Authentication Protocol; the server sends an authentication request to the remote user dialing in. PAP passes the encrypted username and password and identifies the remote end. |  |

    - Example

prompt> pppoa set transport pppoa2 theylogin pap

    - See also

pppoa set transport theylogin

pppoa show transport

pppoa list transports

pppoa show transport

    - Syntax

pppoa show transport {<name>|<number>}

    - Description

This command displays the following information about an existing PPPoA transport:

- Description

- Summary - the connection state

- Server - dialin status

- Headers - the data format that the transport can accept or receive

- SVC status (true or false)

- Local IP address

- Subnet mask

- Remote IP address

- Remote DNS

- Give DNS to Client status

- Give DNS to Relay status

- Create Route status

- Specific Route status

- Route Mask

- Dialout Username

- Dialout Password

- Dialout Authentication method

- Dialin Authentication method

- LCP Max Configure

- LCP Max Failure

- LCP Echo Every

- ATM address (for SVC transports only)

- Auto-connect status

- Idletime status

- ATM port (for PVC transports only)

- Rx VPI (for PVC transports only)

- Rx VCI (for PVC transports only)

- Quality of Service (QoS) class (for PVC transports only)

- Burst tolerance (for PVC transports only)

- Sustainable Cell Rate (SCR) (for PVC transports only)

- Maximum burst size (MBS) (for PVC transports only)

- Maximum Cell Rate (MCR) (for PVC transports only)

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoA transport. To display transport names, use the pppoa list transports command. | N/A |
| number | A number that identifies an existing PPPoA transport. To display transport numbers, use the pppoa list transports command. | N/A |

    - Example

prompt> pppoa show transport pppoa2

PPP Transport: pppoa2

Description : poa

Summary : enabled, down

Summary Err :

Server : true

NCPRemote Addr : N/A

Hdlc : false

Llc : false

Local Ip : 192.168.100.1

Subnet Mask : 255.255.255.0

Remote Ip : 192.168.100.2

Remote DNS : N/A

Give DNSto Client : true

Give DNSto Relay : true

Create Route : true

Specific Route : false

Route Mask : 255.0.0.0

Dialout Username :

Dialout Password :

Dialout Auth : none

Dialin Auth : none

Lcp Max Configure : 10

Lcp Max Failure : 5

Lcp Max Terminate : 2

Lcp Echo Every : 10

Auto Connect : false

Idle Timeout : 0

Port : a1

Rx Vpi : N/A

Rx Vci : 100

Class : UBR

Burst Tolerance : N/A

Sustainable Cell Rate : N/A

MBS : N/A

MCR : N/A

    - See also

pppoa list transports