- Introduction to the CLI

This chapter provides some basic instructions on how to use the CLI, and gives details of how the information in this manual is organized.

About the CLI

The CLI is the Command Line Interface. It largely replaces the console commands that were used in earlier releases.

Some console commands are available for use if you have appropriate access permissions set. For details of access permissions, see Access permissions to the CLI .

The CLI provides a command line interface to the software modules that are modelled in the unified Information Model. For more details on using the command line interface, see Using CLI and Console Commands.

Using CLI and Console Commands

There are two types of command available for use in the product:

- CLI commands - the CLI command replaces the majority of legacy console commands. For example, the console command:

bridge device add

is now the CLI command:

bridge add interface

- Console commands - the console command has not been replaced by a CLI command. Users with appropriate access permissions (see Access permissions to the CLI ) can enter console mode from the CLI and use the console commands. For details of how to use console commands, see Console Access CLI commands.

There are two types of console command, and different access permissions exist for each type of command:

- Usable commands - using these commands will not lead to inconsistencies between the Information Model and the underlying system.

- Blacklisted commands - using blacklisted console commands can lead to inconsistencies between the Information Model and the underlying system and should be used with extreme caution.

    - Status of console commands

The appendices at the back of this guide describe the status of the console commands. One of the following notes is included at the beginning of each console command description to explain the recommended usage of the console command with the CLI:

Note - This console command has been replaced by the CLI command <cross- reference to CLI command>. Use the CLI command instead of the console command. Users with any of the three CLI user access settings (Default, Engineer or Super user) can use this CLI command.

Note - This console command has not been replaced by a CLI command. If used, this command will not create inconsistencies between the Information Model and the underlying system. It is a usable console command.

Note - This console command has not been replaced by a CLI command. If used, this command will create inconsistencies between the Information Model and the underlying system. It is a blacklisted console command that can only be used with extreme caution.

Each console command description also contains details of which CLI access permissions a user needs in order to use the command. For more information, see Access permissions to the CLI.

    - Access permissions to the CLI and Console commands

There are three access level options for CLI users:

- default user - can use CLI commands; cannot use usable console commands or blacklisted console commands.

- engineer - can use CLI commands and usable console commands; cannot use blacklisted console commands.

- super user - can use CLI commands, usable console commands and blacklisted console commands. Can also set up user login accounts, save backup configuration and restore factory settings.

CLI Command Groups

    - Functionalities access supported by the CLI

The implemented modules which are supported by the CLI are listed below:

- Bridge configuration, bridge module; see Bridge CLI commands.

- Console access; see Console Access CLI commands.

- DHCP Client configuration, dhcpclient module; see DHCP Client CLI commands.

- DHCP Relay configuration, dhcprelay module; see DHCP Relay CLI commands.

- DHCP Server configuration, dhcpserver module; see DHCP Server CLI commands.

- DNS Client configuration, dnsclient module; see DNS Client CLI commands.

- DNS Relay configuration, dnsrelay module, see DNS Relay CLI commands.

- Ethernet configuration, ethernet module; see Ethernet CLI commands.

- Firewall configuration, firewall module; see Firewall CLI commands.

- IGMP configuration, igmp module; see IGMP CLI commands

- IPoA configuration, ipoa module; see IPoA CLI commands.

- MAC Spoofing, macspoof module; see MAC Spoofing CLI commands.

- NAT configuration, nat module; see NAT CLI commands.

- Port configuration, port module; see Port CLI commands.

- PPPoA configuration, pppoa module; see PPPoA CLI commands.

- PPPoE configuration, pppoe module; see PPPoE CLI commands.

- PPTP configuration, pptp module; see PPTP CLI commands.

- RFC1483 configuration, rfc1483 module; see RFC1483 CLI commands.

- Security configuration, security module; see Security CLI commands.

- SNTP configuration, sntp module; see SNTP CLI commands.

- System configuration; system module; see System CLI commands.

- Routing configuration - IP stack, ip module; see TCP/IP CLI commands.

- Transports configuration, transports module; see Transports CLI commands.

- User configuration, user module: see User CLI commands.

- WebServer administration, webserver; see Web Server CLI commands.

    - Functionalities access NOT supported by the CLI

The functionalities access which are not supported by the CLI are listed below. For these modules, the Console interface can be used for configuration:

- BUN; see BUN Console Commands.

- TFTP; see TFTP Console Commands.

- Chips; see Chips Console Commands

There are other support modules that are not supported by the CLI, including:

| atm buffer chips flashfs fm | isfs led oam oamcli portcli |
| --- | --- |

CLI Description

This section describes the conventions used in this manual for describing the CLI commands.

    - Understanding CLI/Console command tables

Each chapter of this manual describes a CLI command group. A chapter about an implemented module that is supported by the CLI has two tables at the beginning of it:

- The first table in the chapter lists all the CLI commands available in that CLI command group.

- The second table contains the legacy console commands. The status of each console command is confirmed as one of the following:

- replaced by a CLI command <link to further information on the CLI command>

- usable <link to information on the console command in the relevant Appendix>

- blacklisted <link to information on the console command in the relevant Appendix>

A chapter about an implemented module that is not supported by the CLI only has the console commands table at the beginning of it. Similarly, if there are no console commands for a particular module, that chapter has no CLI command table.

A typical extract from a console/CLI command table is shown below:

| Console command | Status of command in this release |
| --- | --- |
| event | Usable command, see event … |
| restart | Replaced by CLI command restart |
| tell | Blacklisted command, see tell <process> … |

    - Using the Index

The Index chapter at the end of this manual provides you with a quick reference guide to all of the CLI and console commands. Each console command also displays the module that the command belongs to, and its status. For example:

console process60

cpu (Chips) (usable)618

    - Identification in the CLI

Existing CLI interfaces, transports, tunnels and so on can be identified by their name or number. You assign a name when you create the interface (or transport, tunnel etc) using the add interface command. Once created, the CLI adds this interface to a list of IP interfaces which you can display using the ip list interfaces command. Interfaces are given identification numbers that appear under the first column under the heading ID:

prompt> ip list interfaces

IP Interfaces:

ID  |    Name      |    IP Address    |   DHCP   |   Transport

-----|--------------|------------------|----------|-------------

1 | ip4          | 0.0.0.0          | disabled | Not attached

2 | ppp_device   | 192.168.102.2    | disabled | pppoe1

3 | ip2          | 192.168.102.3    | disabled | eth1

4 | ip1          | 0.0.0.0          | disabled | ipoa1

----------------------------------------------------------------

Identification numbers only reflect the order that they appear in the interface list. If you delete the IP interface called ip2 with the identification number 3, the IP interface ip1 moves up the list and inherits the identification number 3.

Using the source CLI command

The source <filename> command allows you to run a list of predefined commands stored in an existing file. This saves you having to retype lengthy configurations that you will want to use again.

Before you can use this command, you need to create a file containing the command list and save it in your ISFS directory. Once you specify the filename in the source command, the file is located and the commands are executed. For example:

prompt> source //isfs/myconfiguration.txt

Sourcing file ‘//isfs/myconfiguration.txt’...

prompt> ip clear interfaces

prompt> ip clear routes

prompt> transports clear

prompt> ethernet add transport eth1 ethernet

prompt> ip add interface ip1 192.168.101.2

prompt> ip attach ip1 eth1

prompt> ipoa add transport ipoa1 pvc a1 0 700

prompt> ip add interface ip2 192.168.102.2

prompt> ip attach ip2 ipoa1

prompt> ipoa transport ipoa1 set pvc 1 pcr 50000

prompt> ip add route default 0.0.0.0 0.0.0.0 gateway 192.168.102.3

prompt> ip list interfaces

IP Interfaces:

ID  |    Name      |    IP Address    |   DHCP   |   Transport

-----|--------------|------------------|----------|----------------

1 | ip2          | 192.198.102.2    | disabled | ipoa1

2 | ip1          | 192.168.101.2    | disabled | eth1

-------------------------------------------------------------------

prompt--> transports list

Services:

ID  |    Name      | Type

-----|--------------|------------------------------------------------------

1 | ipoa1        | IPoA      | TxPkts:   19/0    RxPkts:    0/0

2 | eth1         | Ethernet  | TxPkts:  152/0    RxPkts:  152/0

---------------------------------------------------------------------------

IP routes:

prompt> ip list routes

IP routes:

ID  |    Name    | Destination      | Netmask          | Gateway/Interface

-----|------------|------------------|------------------|------------------

1 | default    | 0.0.0.0          | 0.0.0.0          | 192.168.102.3

---------------------------------------------------------------------------

Using the hotkeys in CLI command mode

During CLI command mode, the system provides several hotkey to accelerate or help the input of CLI command. Entering the question mark (‘?’) any time will provide context-sensitive help or possible options for your reference. If you enter TAB key, the system will automatically filled the remaining characters if it is the only one keyword matching the initial inputs. The most useful hotkey is SPACEBAR, it combines the functions of ‘?’and TAB, to help you finish the command speedily.

Ctrl-P will automatically pop-up the previous CLI command and Ctrl-N is the next CLI command in command history.

You can find more information by enter ‘help’ command.