Meter CLI commands

This chapter describes the meter CLI commands.

| 1 | Summary |
| --- | --- |
|  | 1.1 Meter CLI commands |
| The table below lists the meter commands provided by the CLI: |  |

| Command | Description/Console command |
| --- | --- |
| meter add profile srtcm | meter add profile srtcm |
| meter add profile tokenbucket | meter add profile tokenbucket |
| meter add profile trtcm | meter add profile trtcm |
| meter clear profiles | meter clear profiles |
| meter delete profile | meter delete profile |
| meter list profiles | meter list profiles |
| meter set profile action drop | meter set profile action drop |
| meter set profile action mark dscp | meter set profile action mark dscp |
| meter set profile action pass | meter set profile action pass |
| meter show profile | meter show profile |

| 2 | meter add profile srtcm |  |
| --- | --- | --- |
|  | 2.1 Syntax |  |
|  | meter add profile <name> srtcm <cir> <cbs> <ebs> |  |
|  | 2.2 Description |  |
|  |  | This command creates a meter profile that uses srTCM algorithm for metering. |
|  |  | • If a packet stream’s average rate is within CIR and the burst size is within CBS, then that packet is in-profile (green). |
|  |  | • If a packet stream’s average rate is within CIR and the burst size is not within CBS but is within CBS+EBS, then that packet is partially in profile (yellow). |
|  |  | • All other packets are out of profile (red). |
|  |  | Note that only IP packet size (IP header and payload) is considered for calculations. |
|  |  | You can specify the treatment or actions for green, yellow and red packets using the meter set profile action commands. You can then set the profile to create meter instance on a transport using the transports set meter instance profile command. |
|  | 2.3 Options |  |
|  | The following table gives the range of values for each option which can be specified with this command and a default value (if applicable). |  |

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the profile. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| cir | Committed Information Rate in kbps. | N/A |
| cbs | Committed Burst Size in bytes. | N/A |
| ebs | Excess Burst Size in bytes. | N/A |

| 2.4 | Example |
| --- | --- |
|  | prompt> meter add profile mp1 srtcm 80 10000 25000 |

| 2.5 | See also |
| --- | --- |
|  | meter add profile tokenbucket |
|  | meter add profile trtcm |
|  | meter list profiles |

3 meter add profile tokenbucket

3.1 Syntax

meter add profile <name> tokenbucket <cir> <cbs>

3.2 Description

This command creates a meter profile that uses the token-bucket algorithm for metering.

• If a packet stream’s average rate is within CIR and the burst size is within CBS, then the packet is in profile (green).

• All other packets are out of profile (red).

Note that only IP packet size (IP header and payload) is considered for calculations. You can specify the treatment or actions for green, yellow and red packets using the meter set profile action commands. You can then set the profile to create meter instance on a transport using the transports set meter instance profile command.

3.3 Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the profile. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| cir | Committed Information Rate in kbps. | N/A |
| cbs | Committed Burst Size in bytes. | N/A |

3.4 Example

prompt> meter add profile mp1 tokenbucket 80 10000

3.5 See also

meter add profile srtcm

meter add profile trtcm

4 meter add profile trtcm

4.1 Syntax

meter add profile <name> trtcm <cir> <cbs> <pir> <pbs>

This command creates a meter profile that uses the trTCM algorithm for metering.

• If a packet stream’s average rate is within CIR and the burst size is within CBS, then the packet is in profile (green).

• If a packet stream’s average rate is within PIR and the burst size is within PBS, then the packet is partially in profile (yellow).

• All other packets are out of profile (red).

Note that only IP packet size (IP header and payload) is considered for calculations. You can specify the treatment or actions for green, yellow and red packets using the meter set profile action commands. You can then set the profile to create meter instance on a transport using the transports set meter instance profile command.

4.2 Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the profile. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| cir | Committed Information Rate in kbps. | N/A |
| cbs | Committed Burst Size in bytes. | N/A |
| pir | Peak Information Rate in kbps. | N/A |
| pbs | Peak Burst Size in bytes. | N/A |

4.3 Example

prompt> meter add profile mp1 trtcm 80 10000 120 15000

4.4 See also

meter add profile srtcm

meter add profile tokenbucket

meter list profiles

5 meter clear profiles

5.1 Syntax

meter clear profiles

5.2 Description

This command allows you to delete all meter profiles that were previously created using the meter add profile commands.

Note that this command does not delete the profiles that are associated with meter instances created using the transports set meter instance profile command.

5.3 Example

prompt> meter clear profiles

24.5.4 See also

meter add profile srtcm

meter add profile tokenbucket

meter add profile trtcm

6 meter delete profile

6.1 Syntax

meter delete profile <name>

6.2 Description

This command deletes a single meter profile that was previously created using one of the meter add profile commands.

Note that this command does not delete the profiles that are associated with meter instances created using the transports set meter instance profile command.

6.3 Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display names, use the meter list profiles command. | N/A |

6.4 Example

prompt> meter delete profile mp1

6.5 See also

meter add profile srtcm

meter add profile tokenbucket

meter add profile trtcm

7 meter list profiles

7.1 Syntax

meter list profiles

7.2 Description

This command lists all of the meter profiles that were created using the meter add profile commands. It displays the following information about meter profiles:

• Name

• Type of algorithm used

• CIR value (in kbps)

• CBS value (in bytes)

• EBS value (for algorithm type srtcm only)

• PIR value (for algorithm type trtcm only)

• PBS value (for algorithm type trtcm only)

• Green action

• Yellow action (for algorithm types trtcm and srtcm only)

• Red action

7.3 Example

prompt> meter list profiles

Meter Profiles:

7.4 See also

meter add profile srtcm

meter add profile tokenbucket

meter add profile trtcm

8 meter set profile action drop

8.1 Syntax

meter set profile <name> {green|red|yellow} action drop

8.2 Description

This command configures an existing profile to drop packets depending on their metering result. Note that if this command is not applied, by default the green and yellow packets are passed and red packets are dropped.

8.3 Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display names, use the meter list profiles command. | N/A |
| green | Sets packets with green (in profile) metering results to be dropped. | pass |
| red | Sets packets with red (out of profile) metering results to be dropped. | drop |
| yellow | Sets packets with yellow (partially in profile) metering results to be dropped. | pass |

8.4 Example

prompt> meter set profile mp1 green action drop

8.5 See also

meter list profiles

9 meter set profile action mark dscp

9.1 Syntax

meter set profile <name> {green|red|yellow} action mark dscp <dscp>

9.2 Description

This command configures a profile to set the DSCP value in the IP header of packets depending on their metering result (green, yellow or red). The DSCP value is written into the DS field in the header of the IP packet.

9.3 Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display names, use the meter list profiles command. | N/A |
| green | Sets the DSCP value in packets with green (in profile) metering results. | N/A |
| red | Sets the DSCP value in packets with red (out of profile) metering results. | N/A |
| yellow | Sets the DSCP value in packets with yellow (partially in profile) metering results to be dropped. | N/A |
| dscp | The Differentiated Service Code Point (DSCP). This can be in hexadecimal format (0xNN) or as six binary digits (NNNNNN). If specified in hex, only the six most significant bits specify the DSCP value and the two least significant bits are ignored. | N/A |

9.4 Example

prompt> meter set profile mp1 yellow action mark dscp 011001

9.5 See also

meter list profiles

10 meter set profile action pass

10.1 Syntax

meter set profile <name> {green|red|yellow} action pass

10.2 Description

This command configures an existing profile to pass packets depending on their metering result. The DSCP value in the IP header of packets is not set.

Note that if this command is not applied, by default the green and yellow packets are passed and red packets are dropped.

10.3 Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display names, use the meter list profiles command. | N/A |
| green | Sets packets with green (in profile) metering results to pass. | pass |
| red | Sets packets with red (out of profile) metering results to pass. | drop |
| yellow | Sets packets with yellow (partially in profile) metering results to pass. | pass |

24.10.4 Example

prompt> meter set profile mp1 green action pass

24.10.5 See also

meter list profiles

11 meter show profile

11.1 Syntax

meter show profile <name>

11.2 Description

This command displays information about a meter profile that was created using one of the meter add profile commands. It displays the following information about the specified profile:

• Name

• Type of algorithm used

• CIR value (in kbps)

• CBS value (in bytes)

• EBS value (for algorithm type srtcm only)

• PIR value (for algorithm type trtcm only)

• PBS value (for algorithm type trtcm only)

• Green action

• Yellow action (for algorithm types trtcm and srtcm only)

• Red action

11.3 Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing profile. To display names, use the meter list profiles command. | N/A |

11.4 Example

prompt> meter show profile mp3

Meter Profile : mp3

Type : trtcm

CIR (kbps) : 80