- SNTPClient CLI commands

This chapter describes the SNTP (Simple Network Time Protocol) Client CLI commands.

Summary

    - SNTPClient CLI commands

The table below lists the SNTP Client commands provided by the CLI:

| Command | Reference |
| --- | --- |
| sntpclient set timezone | sntpclient set timezone |
| sntpclient set dst | sntpclient set mode |
| sntpclient set mode | sntpclient set mode |
| sntpclient set server | sntpclient set server |
| sntpclient set poll-interval | sntpclient set pollintv |
| sntpclient sync | sntpclient sync |
| sntpclient set timeout | sntpclient set timeout |
| sntpclient set retries | sntpclient set retries |
| sntpclient show association | sntpclient show association |
| sntpclient show status | sntp show status |
| sntpclient set clock | sntpclient set clock |

sntpclient set timezone

    - Syntax

sntpclient set timezone <abbreviation>

    - Description

This command sets the local time zone abbreviation as a parameter and configures the local system to be up to + 13 hours of the Universal Time Coordinate (UTC). Sixty-four of the worlds most prominent time zones are represented (including those using standard time and summer/daylight savings time).

    - Options

The following table gives the 64 time zone abbreviations that you can use in this command. to set the timezone difference for the system timer. The table also contains the difference in time (in hours and minutes) from the UTC, and a description of the area of the world (from west to east) where the time difference is calculated from:

| Abbreviation | + UTC | World Area of Time Zone |
| --- | --- | --- |
| IDLW | -1200 | International Date Line West |
| NT | -1100 | Nome |
| HST | -1000 | Hawaii Standard |
| AKST | -0900 | Alaska Standard |
| YST | -0900 | Yukon Standard |
| YDT | -0800 | Yukon Daylight |
| PST | -0800 | US Pacific Standard |
| PDT | -0700 | US Pacific Daylight |
| MST | -0700 | US Mountain Standard |
| MDT | -0600 | US Mountain Daylight |
| CST | -0600 | US Central Standard |
| CDT | -0500 | US Central Daylight |
| EST | -0500 | US Eastern Standard |
| EDT | -0400 | US Eastern Daylight |
| AST | -0400 | Atlantic Standard |
| NFST | -0330 | Newfoundland Standard |
| NFT | -0330 | Newfoundland |
| BRA | -0300 | Brazil Standard |
| ADT | -0300 | Atlantic Daylight |
| NDT | -0230 | Newfoundland Daylight |
| AT | -0200 | Azores |
| WAT | -0100 | West Africa |
| GMT | +0000 | Greenwich Mean |
| UTC | +0000 | Universal (Coordinated) |
| WET | +0000 | Western European |
| CET | +0100 | Central European |
| FWT | +0100 | French Winter |
| MET | +0100 | Middle European |
| MEWT | +0100 | Middle European Winter |
| SWT | +0100 | Swedish Winter |
| BST | +0100 | British Summer |
| EET | +0200 | Eastern Europe |
| FST | +0200 | French Summer |
| MEST | +0200 | Middle European Summer |
| SST | +0200 | Swedish Summer |
| IST | +0200 | Israeli Standard |
| IDT | +0300 | Israeli Daylight |
| BT | +0300 | Baghdad |
| IT | +0330 | Iran |
| USZ3 | +0400 | Russian Volga |
| USZ4 | +0500 | Russian Ural |
| INST | +0530 | Indian Standard |
| USZ5 | +0600 | Russian West-Siberian |
| NST | +0630 | North Sumatra |
| WAST | +0700 | West Australian Standard |
| USZ6 | +0700 | Russian Yenisei |
| JT | +0730 | Java |
| CCT | +0800 | China Coast |
| WADT | +0800 | West Australian Daylight |
| ROK | +0900 | Korean Standard |
| KST | +0900 | Korean Standard |
| JST | +0900 | Japan Standard |
| CAST | +0930 | Central Australian Standard |
| KDT | +1000 | Korean Daylight |
| EAST | +1000 | Eastern Australian Standard |
| GST | +1000 | Guam Standard |
| CADT | +1030 | Central Australian Daylight |
| EADT | +1100 | Eastern Australian Daylight |
| IDLE | +1200 | International Date Line East |
| NZST | +1200 | New Zealand Standard |
| NZT | +1200 | New Zealand |
| NZDT | +1300 | New Zealand Daylight |

    - Example

In the example below, the time zone is set to Unites States Eastern Standard Time, which is five hours earlier than UTC (-0500):

prompt> sntpclient set timezone EST

sntpclient set dst

    - Syntax

sntpclient set dst {auto|none}

    - Description

- This command enables/disables the Daylight Saving Time (DST) capability in the STNP client.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| auto | Enable Daylight Saving Time (DST).. | auto |
| none | Disable DST function. |  |

    - Example

prompt> sntpclient set dst none

    - See also

sntpclient set timezone

sntpclient set mode

    - Syntax

sntpclient set mode {unicast|broadcast|anycast} {enable|disable}

    - Description

This command enables/disables the STNP client in a particular time synchronous access mode. There are three modes to choose from, and each mode has enable and disable options:

- Unicast mode

- Enable - the mode uses a unicast server and the IP address or hostname in the SNTP server association list is used to synchronize the client time with the server. The SNTP client attempts to contact the specific server in the association in order to receive a timestamp when the sntpclient sync command is issued.

- Disable - the unicast server is removed from the association list.

- Broadcast mode

- Enable - allows the SNTP client to accept time synchronization broadcast packets from an SNTP server located on the network, and updated the local system time accordingly.

- Disable - stops synchronization via broadcast mode

- Anycast mode

- Enable - the SNTP client sends time synchronized broadcast packets to the network and subsequently expects a reply from a valid timeserver. The client then uses the first reply it receives to establish a link for future sync operations in unicast mode. This server will then be added to the server association list. The client ignores any later replies from servers after the first one is received.

The enabled anycast mode takes precedence over any entries currently in the associations list when the sntpclient sync command is issued. The entry will then be substituted for any existing entry in the unicast association list.

- Disable - stops synchronization via anycast mode.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Unicast | Sets the time synchronous access mode to use the unicast server. | N/A |
| Broadcast | Sets the time synchronous access mode to use the broadcast server. | N/A |
| Anycast | Sets the time synchronous access mode to use the anycast server. | N/A |
| Enable | Enables the selected time synchronous access mode. | N/A |
| Disable | Enables the selected time synchronous access mode. | N/A |

    - Example

prompt> sntpclient set mode anycast enable

    - See also

sntpclient set server

sntp show status

sntpclient show association

sntpclient set server

    - Syntax

sntpclient set server {ipaddress <IP address> | hostname <hostname>}

    - Description

This command sets the dedicated unicast server for which the SNTP client can synchronize its time. You can set the server either by specifying the IP address or the hostname.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| IP address | The IP address of the dedicated unicast server that SNTP can use to synchronize its time. | N/A |
| hostname | The hostname of the dedicated unicast server that SNTP can use to synchronize its time. | N/A |

    - Examples

Example One - IP address

prompt> sntpclient set server ipaddress 129.6.15.28

Example Two - hostname

prompt> sntpclient set server hostname time-a.nist.gov

sntpclient set pollintv

    - Syntax

sntpclient set pollintv <0-9999>

    - Description

This command sets the SNTP client to automatically send a time synchronization request (specific to the mode) to the network at a specific interval. If the poll-interval is set to 0, the polling mechanism will be disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| 0-9999 | Sets the polling interval (in minutes) that SNTP client will sync with a designated server. This can be any value between 0 and 9999. | 0 (disabled) |

    - Example

prompt> sntpclient set pollintv 10

sntpclient sync

    - Syntax

sntpclient sync

    - Description

This command forces the SNTP client to immediately synchronize the local time with the server located in the association list (if unicast) or, if anycast is enabled, initiate an anycast sequence to the network.

    - Example

prompt> sntpclient sync

    - See also

sntpclient set server

sntpclient set timeout

    - Syntax

sntpclient set timeout <0-30>

    - Description

This command sets the received packet response timeout value (in seconds) upon sync request initiation. After timeout, if the sntpclient retry command value is set, an attempt will be retried.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| 0-30 | Sets the received packet response timeout value (in seconds). This can be any value between 0 and 30. | 5 seconds |

    - Example

prompt> sntpclient set timeout 10

    - See also

sntpclient set retries

sntpclient set retries

    - Syntax

sntpclient set retries <0-10>

    - Description

This command sets the number of packet retry attempts when no response is received from a timeserver. The SNTP client will send another packet for synchronization after a timeout.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| 0-10 | Sets the number of packet retry attempts made when no response is received from a timeserver. | 2 |

    - Example

prompt> sntpclient set retries 4

    - See also

sntpclient set timeout

sntpclient show association

    - Syntax

sntpclient show association

    - Description

This command lists the server configuration for the SNTP client with the timeserver address and displays whether or not the client is synchronized with the association server.

    - Examples

Example One - IP address

prompt> sntpclient show association

Time Reference Server IP addr: 129.6.15.28

Time Reference Server Hostname:

** Local clock synchronized with this server.

Example Two - hostname

prompt> sntpclient show association

Time Reference Server IP addr:

Time Reference Server Hostname: time-a.nist.gov

** Local clock synchronized with this server.

    - See also

sntpclient set server

sntp show status

    - Syntax

sntpclient show status

    - Description

This command displays the SNTP client status information.

    - Example

prompt> sntpclient show status

Clock Synchronized	TRUE

SNTP Standard Version Number:	4

SNTP Mode(s) Configured:	Unicast Broadcast

Local Time:	Tuesday, 28 Aug, 2001 - 14:39:25

Local Timezone:	EDT, Eastern Daylight Time

Time Difference +-VTC:	-4:00

Daylight Saving Time:         Automatically

Server Stratum:               0

Precision:	1/16384 of a second

Root Delay:                   +0.0 second(s)

Dispersion:	+0.2342 second(s)

Server Reference ID:	GPS.

Round Trip Delay:	2 second(s)

Local Clock Offset:	-1 second(s)

Resync Poll Interval	15 minute(s)

Packet Retry Timeout:	5 seconds

Packet Retry Attempts:	3

    - See also

sntpclient show association

sntpclient set clock

    - Syntax

sntpclient set clock <yyyy:mm:dd:hh:mm:ss>

    - Description

This command sets the system clock to a specific time and date. This command can be used as an alternative to synchronizing the local system clock via internal or external timeservers.

    - Example

The following command sets the system clock to 11:10:13pm, 2nd November 2001:

prompt> sntpclient set clock 2001:11:02:23:10:13