- 802.1x CLI commands

This chapter describes 802.1x CLI commands.

Summary

    - 802.1x CLI commands

The table below lists the 802.1x commands provided by the CLI:

| Command | Reference |
| --- | --- |
| 802.1x authenticator set authentication enabled¦disabled | 802.1x authenticator set authentication enabled¦disabled |
| 802.1x authenticator set authentication local¦radius | 802.1x authenticator set authentication local¦radius |
| 802.1x authenticator set identity | 802.1x authenticator set identity |
| 802.1x authenticator set rekey-timeout | 802.1x authenticator set rekey-timeout |
| 802.1x authenticator show | 802.1x authenticator show |

802.1x authenticator set authentication enabled|disabled

    - Syntax

802.1x authenticator set authentication {enabled|disabled}

    - Description

This command enables/disables authentication on your wireless device. Enabling this command sets the Access Point (AP) on your device as the authenticator in your network configuration.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | Enables authentication and sets the AP on your wireless device as the authenticator in your network configuration. | disabled |
| disabled | Disables authentication on your device |  |

    - Example

prompt> 802.1x authenticator set authentication enabled

    - See also

802.1x authenticator show

802.1x authenticator set authentication local|radius

    - Syntax

802.1x authenticator set authentication {local|radius}

    - Description

This command sets the method of authentication used by your authenticator (AP).

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| local | Sets a local server as your authentication method (using MD5) | N/A |
| radius | Sets RADIUS as your authentication method (using EAP) | N/A |

    - Example

prompt> 802.1x authenticator set authentication radius

    - See also

802.1x authenticator show

For details of RADIUS client commands, see RADIUS Client CLI commands

802.1x authenticator set identity

    - Syntax

802.1x authenticator set identity <string>

    - Description

This command allows you to set a unique identity string for your device.

The authenticator will use this string to identify your device.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| string | A unique string that identifies your device. If you want to use spaces in your string, you need to enclose the entire argument in straight double quotes. | N/A |

    - Example

prompt> 802.1x authenticator set identity “DSL Router”

    - See also

802.1x authenticator show

802.1x authenticator set rekey-timeout

    - Syntax

802.1x authenticator set rekey-timeout <seconds>

    - Description

This command sets the WEP rekey timeout that causes the issue of a new WEP key for the session. Once set to a suitable timeout, the keys will rotate at a rate fast enough to prevent an attacker from sniffing enough packets in order to discover the WEP key.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| seconds | The time (in seconds) before a WEP key is rotated. | 600 |

    - Example

prompt> 802.1x authenticator set rekey-timeout 800

    - See also

802.1x authenticator show

802.1x authenticator show

    - Syntax

802.1x authenticator show

    - Description

This command displays the following information about your 802.1x configuration:

• Authentication status (true/false)

• Authentication method set

• Identity string set

• Rekey timeout set

This command also displays the following information about the supplicants in your wireless network:

• Supplicant MAC address

• Authenticated status; whether the supplicant is authenticated (granted access) (true/false)

• Re_auth-when; the time since the system was booted when the supplicant will be forced to re-authenticate

• Pae_stated; the current Port Access Entity (PAE) state (AUTHENTICATED, DISCONNECTING or CONNECTING).

• Re_auth-count; the number of times the supplicant has attempted to authenticate since the system was booted

• Auth_state; the current state of the authentication state machine for that supplicant

• Idletime; the number of seconds since the supplicant last sent any traffic

    - Example

prompt> 802.1x authenticator show

Ser802.1x Authenticator

Auth Control Enabled : true

Auth Server : RADIUS

Identity String : test

Rekey Timeout : 600

Supplicants:

MacAddress |Authenticated|Re-auth-when| Pae-stated  | Re_auth_count | Auth_state | Ideltime

-----------------------------------------------------------------------------------------------

00:20:2b:01:02:03| true                    |8347                   | AUTHENTICATED | 0                      | IDLE                  | 0

-----------------------------------------------------------------------------------------------------------------------------------------------------

    - See also

802.1x authenticator