- VPN/PPTP CLI commands

This chapter describes the PPTP CLI commands used for VPN connections.

Summary

    - VPN/PPTP CLI commands

The table below lists the CLI commands for manipulating VPN/PPTP transports:

| Command | Reference |
| --- | --- |
| pptp add transport | pptp add transport |
| pptp clear transports | pptp clear transports |
| pptp delete transport | pptp delete transport |
| pptp list transports | pptp list transports |
| pptp set transport autoconnect | pptp set transport autoconnect |
| pptp set transport enabled¦disabled | pptp set transport enabled¦disabled |
| pptp set transport encryption mppe | pptp set transport encryption mppe |
| pptp set transport idletimeout | pptp set transport idletimeout |
| pptp set transport lantolan | pptp set transport lantolan |
| pptp set transport localip | pptp set transport localip |
| pptp set transport mppe | pptp set transport mppe |
| pptp set transport natinternalif | pptp set transport natinternalif |
| pptp set transport password | pptp set transport password |
| pptp set transport peer | pptp set transport peer |
| pptp set transport peernetworkip | pptp set transport peernetworkip |
| pptp set transport remoteaccess | pptp set transport remoteaccess |
| pptp set transport remoteip | pptp set transport remoteip |
| pptp set transport theylogin | pptp set transport theylogin |
| pptp set transport username | pptp set transport username |
| pptp set transport welogin | pptp set transport welogin |
| pptp show transport | pptp show transportl |

pptp add transport

    - Syntax

pptp add transport <name> {dialin|dialout} ipinterface <interface>

    - Description

This command creates a VPN/PPTP connection. The system can act as client using dialout; as server using dialin. User also needs to specifiy the IP interface the VPN connection will carry through.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the transport. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interface | The IP interface that VPN/PPTP wors through. The ip list interfaces displays the available IP interfaces. | N/A |

    - Example

prompt> pptp add transport vpn1 dialout ipinterface ipwan

    - See also

pptp clear transports

pptp delete transport

pptp list transports

pptp clear transports

    - Syntax

pptp clear transports

    - Description

This command deletes all transports that were created using the pptp add transport command.

    - Example

prompt> pptp clear transports

    - See also

pptp add transport

pptp delete transport

pptp list transports

pptp delete transport

    - Syntax

pptp delete transport {<name>|<number>}

    - Description

This command deletes a single PPTP transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| number | A number that identifies an existing transport. To display transport numbers, use the pptp list transports command. | N/A |

    - Example

prompt> pptp delete transport vpn1

    - See also

pptp add transport

pptp clear transports

pptp list transports

pptp list transports

    - Syntax

pptp list transports

    - Description

This command lists all PPTP transports that have been created using the pptp add transport command. It displays the following information about PPTP transports:

- transport ID number

- transport name

- type (dialin or dialout)

- remote IP address (if applicable)

- enabled or disabled

    - Example

prompt> pptp list transports

PPTP Transports:

ID  |   Name    |    Type    |    Remote IP   | Enabled

-----|-----------|------------|----------------|-----------

1 | vpn1      | dialin     | 192.168.102.3  | false

-----------------------------------------------------------

    - See also

pptp show transport

pptp set transport autoconnect

    - Syntax

pptp set transport <name> autoconnect {enabled|disabled}

    - Description

This command determines whether PPTP transport is dial-on-demand or not.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | Set autoconnect to true. | enabled |
| disabled | Set autoconnect to false. |  |

    - Example

prompt> pptp set transport vpn1 autoconnect disabled

    - See also

pptp add transport

pptp show transports

pptp set transport enabled|disabled

    - Syntax

pptp set transport <name> {enabled|disabled}

    - Description

This command enables (or disables) the specified PPTP transport. When a PPTP transport is disabled, it is inactive for either dialout or dialin. Moreover, disables a transport that is connected will force it to disconnect. Enabling a disconnected transport will try to connect.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| enabled | Enable (and try to connect) the specified transport. | disabled |
| disabled | Disable (and/or disconnect) the specified transport. |  |

    - Example

prompt> pptp set transport vpn1 disabled

    - See also

pptp add transport

pptp show transports

pptp set transport encryption mppe

    - Syntax

pptp set transport <name> encryption mppe {enabled|disabled|auto}

    - Description

This command controls whether the data encrypted or not in the VPN connection. The encryption algorithm is MPPE (Microsoft Point-to-Point Encryption.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| enabled | Enable data encryption and encryption is mandatory | disabled |
| disabled | Disable the data encryption.. |  |
| auto | Negotiate whether enabling encryption or not with peer |  |

    - Example

prompt> pptp set transport vpn1 encryption mppe enabled

    - See also

pptp set transport mppe

pptp show transports

pptp set transport idletimeout

    - Syntax

pptp set transport <name> idletimeout <time>

    - Description

This command configures the idle time for disconnecting the VPN connection.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| time | The idle time (in minutes) to disconnect the connection. Valid range is 0 to 60. If time is 0, it means never timeout. | 0 |

    - Example

prompt> pptp set transport vpn1 idletimeout 10

    - See also

pptp set transport autoconnect

pptp show transports

pptp set transport lantolan

    - Syntax

pptp set transport <name> lantolan {enabled|disabled}

    - Description

This command configures the transport is used for LAN-to-LAN application. In this application, packets is transferred according to routing mechanism. Please note, when a transport created using pptp add transport command, it is neither LAN-to-LAN nor RemoteAccess. User has to configure it to right mode before using. This command also overwrites the mode even the original mode is RemoteAccess.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| enabled | Set the transport as LAN-to-LAN application. | N/A |
| disabled | Not for LAN-to-LAN application |  |

    - Example

prompt> pptp set transport vpn1 lantolan enabled

    - See also

pptp add transport

pptp set transport remoteaccess

pptp show transports

pptp set transport localip

    - Syntax

pptp set transport <name> localip <ip address>

    - Description

This command configures the local IP address when a remote PPTP client dial-in. This IP address will give to client as his remote IP address during connection setup.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| ip address | The IP address of the local side of the transport, displayed in the following format: 192.168.102.3 | 0.0.0.0 |

    - Example

prompt> pptp set transport vpn1 localip 192.168.0.101

    - See also

pptp set transport remoteip

pptp show transports

pptp set transport mppe

    - Syntax

pptp set transport <name> mppe keylength {40|128|auto} {stateful|stateless}

    - Description

This command configures the key length of mode used in MPPE encryption.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| 40 | 40-bit session key length | 40 |
| 128 | 128-bit session key length |  |
| auto | Negotiate with the peer |  |
| stateful | Stateful mode, session key will update when the low order octet of the coherency count equals to 0xFF. | stateless |
| 128 | Stateless mode, the session key is changed every packet. |  |

    - Example

prompt> pptp set transport vpn1 mppe auto stateful

    - See also

pptp set transport mppe

pptp show transports

pptp set transport natinternalif

    - Syntax

pptp set transport <name> natinternalif {ipinterface}

    - Description

When a dialout type transport is configured as Remote Access, you can specify which local IP sunet can share the transport with NAT/PAT. Normally, it is useful when you have multiple local IP subnets.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| ipinterface | The interface name of a local IP subnet . | N/A |

    - Example

prompt> pptp set transport vpn1 natinternalif iplan

    - See also

pptp set transport remoteaccess

pptp show transports

pptp set transport password

    - Syntax

pptp set transport <name> password <pwd>

    - Description

This command sets the password when login the remote PPTP server or uses to authenticate the connection when a remote PPTP client dial-in.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| pwd | The password of a login account | N/A |

    - Example

prompt> pptp set transport vpn1 password private

    - See also

pptp set transport username

pptp show transports

pptp set transport peer

    - Syntax

pptp set transport <name> peer {ipaddress <address>|hostname <hostname>}

    - Description

This command configures the IP address or Host Domain Name of the PPTP server when the PPTP client dial to..

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| address | The IP address of the PPTP server, displayed in the following format: 192.168.102.3 | 0.0.0.0 |
| hostname | FQDN (Full-Qualified Domain Name) of PPTP server. It is useful when PPTP server is using Dynamic DNS. | N/A |

    - Example

prompt> pptp set transport vpn1 peer ipaddress 123.0.0.1

prompt> pptp set transport vpn1 peer hostname head.dyn.com

    - See also

pptp add transport

pptp set transport enable|disable

pptp set transport peernetworkip netmask

    - Syntax

pptp set transport <name> peernetworkip <address> netmask <mask>

    - Description

This command specifies the remote subnet for LAN-to-LAN VPN connection. It is used to create the routing entry when the connection is up.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| address | The IP address of the remote subnet, displayed in the following format: 192.168.102.3 | 0.0.0.0 |
| mask | The sunet mask of the remote subnet. | N/A |

    - Example

prompt> pptp set transport vpn1 peernetworkip 192.168.1.1 netmask 255.255.255.0

    - See also

pptp set transport lantolan

pptp show transports

pptp set transport remoteaccess

    - Syntax

pptp set transport <name> remoteaccess {enabled|disabled}

    - Description

This command configures the transport as RemoteAccess. Please note, when a transport created using pptp add transport command, it is neither LAN-to-LAN nor RemoteAccess. User has to configure it to right mode before using. This command also overwrites the mode even the original mode is LAN-to-LAN.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| enabled | Set the transport as RemoteAccess mode | disabled |
| disabled | Unset the transport as RemoteAccess mode |  |

    - Example

prompt> pptp set transport vpn1 remoteaccess enabled

    - See also

pptp set transport lantolan

pptp show transports

pptp set transport remoteip

    - Syntax

pptp set transport <name> remoteip <address>

    - Description

This command configures the IP address that will assign to remote PPTP client dial-in

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| address | The IP address assigned to remote client, displayed in the following format: 192.168.102.3 | 0.0.0.0 |

    - Example

prompt> pptp set transport vpn1 remoteip 192.168.1.50

    - See also

pptp set transport localip

pptp show transports

pptp set transport theylogin

    - Syntax

pptp set transport <name> welogin {auto|chap|pap}

    - Description

This command configures the authentication protocol used when remote PPTP client dial in.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| auto | Negotiate the protocol with remote peer. | chap |
| chap | Using CHAP(Challenge Handshake Authentication Protocol), including standard CHAP and MS-CHAP1. |  |
| pap | Using PAP (Password Authentication Protocol) |  |

    - Example

prompt> pptp set transport vpn1 theylogin chap

    - See also

pptp set transport welogin

pptp show transports

pptp set transport username

    - Syntax

pptp set transport <name> username <username>

    - Description

This command configures the username of a login account that used to login to remote PPTP server or used to authenticate remote PPTP client dial-in.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| username | The usename of a login account. | disabled |

    - Example

prompt> pptp set transport vpn1 username test

    - See also

pptp set transport password

pptp show transports

pptp set transport welogin

    - Syntax

pptp set transport <name> welogin {auto|chap|pap}

    - Description

This command configures the authentication protocol used when connect to remote PPTP server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |
| auto | Negotiate the protocol with remote peer. | chap |
| chap | Using CHAP(Challenge Handshake Authentication Protocol), including standard CHAP and MS-CHAP1. |  |
| pap | Using PAP (Password Authentication Protocol) |  |

    - Example

prompt> pptp set transport vpn1 welogin chap

    - See also

pptp set transport theylogin

pptp show transports

pptp show transport

    - Syntax

pptp show transport <name>

    - Description

This command lists all PPTP transports that have been created using the pptp add transport command. It displays the following information about PPTP transports:

- transport ID name

- type (dialin or dialout)

- login configuration (username, password, authentication protocol)

- Encryption

- Remote PPTP Server IP or Domain name

- Local IP address or remote IP address (if applicable)

- Remote subnet configuration

- NAT internal interface

- enabled or disabled

- Current status: active, session connected, call connected

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing transport. To display transport names, use the pptp list transports command. | N/A |

    - Example

prompt> pptp show transport vpn1

PPTP Transports: vpn1

Type: dialout (none)

Via IP interface: ipwan

User Name: test

Password:

Auth Type: chap

Encryption: disabled (mppe sessionkey length 49bits stateless mode)

Peer IP:          or Peer Hostname:

Server IP:

Client IP:

Perr Network IP: 0.0.0.0

Peer Network Netmask: 0.0.0.0

NAT internal interface:

Enabled: false

Active Status: false

Session Connected: false

Call Connected: false (encryption none)

    - See also

pptp list transports

Application Examples

    - Configure a PPTP Server for Remote User Dial-in

pptp add transport vpn1 dialin ipinterface ipwan

pptp set transport vpn1 remoteacess enabled

pptp set transport vpn1 localip 192.168.1.51

pptp set transport vpn1 remoteip 192.168.1.52

pptp set transport vpn1 username test

pptp set transport vpn1 password private

pptp set transport vpn1 enabled

    - Configure a LAN-to-LAN between two Routers

In Router1, its private LAN subnet is 192.168.1.x/24

pptp add transport vpn1 dialin ipinterface ipwan

pptp set transport vpn1 lantolan enabled

pptp set transport vpn1 peernetworkip 10.0.0.254 netmask 255.255.255.0

pptp set transport vpn1 localip 192.168.0.101

pptp set transport vpn1 remoteip 192.168.0.102

pptp set transport vpn1 username test

pptp set transport vpn1 password private

pptp set transport vpn1 enabled

In Router2, its private LAN subnet is 10.0.0.x/24

pptp add transport vpn1 dialout ipinterface ipwan

pptp set transport vpn1 lantolan enabled

pptp set transport vpn1 peernetworkip 192.168.1.1 netmask 255.255.255.0

pptp set transport vpn1 peer hostname headoffice.com

pptp set transport vpn1 username test

pptp set transport vpn1 password private

pptp set transport vpn1 welogin chap

pptp set transport vpn1 enabled