- IPoA CLI commands

This chapter describes the IP over ATM (IPoA) transport CLI commands

Summary

    - IPoA Tracing commands

You can carry out tracing in the IP stack using the following system commands:

- system trace enable|disable ; enables/disables tracing support output for a specific module and category, including IPoA.

- system trace list ; displays the tracing options for the modules available in your image.

    - IPoA CLI commands

The table below lists the IPoA commands provided by the CLI:

| Command | Reference |
| --- | --- |
| ipoa add transport pvc | ipoa add transport pvc |
| ipoa add transport svc | ipoa add transport svc |
| Ipoa clear transports | ipoa clear transports |
| Ipoa delete transport | ipoa delete transport |
| ipoa list arp | ipoa list arp |
| ipoa list transports | ipoa list transports |
| ipoa set transport arpserver | ipoa set transport arpserver |
| ipoa set transport svc | ipoa set transport svc |
| ipoa set transport lifetime | ipoa set transport lifetime |
| ipoa show transport | ipoa show transport |
| ipoa transport add pvc | ipoa transport add pvc |
| ipoa transport delete pvc | ipoa transport delete pvc |
| ipoa transport set pvc pcr | ipoa transport set pvc pcr |
| ipoa transport set pvc port | ipoa transport set pvc port |
| ipoa transport set pvc vpi | ipoa transport set pvc vpi |
| ipoa transport set pvc vci | ipoa transport set pvc vci |

ipoa add transport pvc

    - Syntax

ipoa add transport <name> pvc <atm-port> <vpi> <vci>

    - Description

This command creates a named IPoA transport that runs over a PVC (Permanent Virtual Circuit). It allows you to specify the following PVC information:

- the ATM port that will transport IP data

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies an IPoA transport. It can be made up of one or more characters or a combination of characters and digits, but it cannot start with a digit. | N/A |
| pvc | A connection-oriented permanent leased line circuit between end stations on a network. An IPoA transport can have multiple PVCs associated with it. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | 0 |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over.The VCI can be any value between 1 and 65535. | 0 |

    - Example

prompt> ipoa add transport ipoa1 pvc a1 0 800

    - See also

ipoa list transports

port list

ipoa clear transports

    - Syntax

ipoa clear transports

    - Description

This command deletes all IPoA transports that were created using the ipoa add transport commands.

    - Example

prompt> ipoa clear transports

    - See also

ipoa delete transport

ipoa delete transport

    - Syntax

ipoa delete transport {<name>|<number>}

    - Description

This command deletes a single IPoA transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |

    - Example

prompt> ipoa delete transport ipoa1

    - See also

ipoa list transports

ipoa list arp

    - Syntax

ipoa list arp

    - Description

This command lists the cached mappings from IP addresses to ATM addresses that were previously set using the ipoa set transport arpserver command. This command is only relevant when using IP over ATM with SVCs.

Note – In current firmware, we did not support SVC transports.

    - Example 1

If there are entries in the ATM ARP table, the output from this command looks like this:

prompt> ipoa list arp

10.11.12.13 47.00.83.00.00.00.00.00.20.2b.00.04.93.00.20.2b.00.04.93.00

    - Example 2

If there are no entries in the ATM ARP table, the output from this command looks like this:

prompt> ipoa list arp

# ATMARP table is empty

    - See also

ipoa set transport arpserver

ipoa list transports

    - Syntax

ipoa list transports

    - Description

This command lists IPoA transports that have been created using the ipoa add transport commands. It displays the following information about the transports:

- transport identification number

- transport name

- SVC status (enabled or disabled), disabled means PVC

- ATM Address Resolution Protocol (ARP) Server information (if set using the ipoa set transport arpserver command).

    - Example

prompt> ipoa list transports

IPoA transports:

ID  |    Name    |   SVC    |                  ATM ARP Server

-----|------------|----------|---------------------------------------------

1 | ipoa1      | disabled | here

--------------------------------------------------------------------------

    - See also

To display more information about an individual transport, use the command ipoa show transport

ipoa set transport arpserver

ipoa set transport arpserver

    - Syntax

ipoa set transport {<name>|<number>} arpserver <server>

    - Description

This command sets the Address Resolution Protocol (ARP) Server that an IPoA transport will use. It only applies to SVCs.

Note – In current firmware, we did not support SVC transports.

The IPoA transport needs to be attached to an IP interface in order to transport data. See TCP/IP CLI commands .

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |
| server | The ATM ARP server you want to use. If you want your local machine to be the ATM ARP server, type ‘here’. If you want another machine to be the ATM ARP server, type the ATM-address of that machine. | N/A |

    - Example

prompt> ipoa set transport ipoa1 arpserver 47.00.83.10.a2.b1.00.00.00.00.00.00.00.00.20.2b.01.00.07.00

    - See also

ipoa list transports

ipoa list arp

ipoa set transport lifetime

    - Syntax

ipoa set transport {<name>|<number>} lifetime <timeout>

    - Description

This command sets idle time-out for IP over ATM SVCs. If there is no traffic on an SVC for this period, then it will be disconnected. (It might be disconnected before this period in order to make room for new connections.)

Note – In current firmware, we did not support SVC transports.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |
| timeout | The idle time-out (in seconds) for IP over ATM SVC connections. | 60 |

    - Example

prompt> ipoa set transport ipoa1 lifetime 120

    - See also

ipoa show transport

ipoa show transport

    - Syntax

ipoa show transport {<name>|<number>}

    - Description

This command displays the following information about an existing IPoA transport:

- transport name

- transport description

- Switched Virtual Channel (SVC) status (true or false)

- ARP Server ATM details (Server IP address or ‘here’)

- details about Permanent Virtual Circuits (PVC) attached to this transport:

- PVC ID number

- Port used by PVC

- Virtual Path Identifier (VPI) setting

- Virtual Circuit Identifier (VCI) setting

- Peak Cell Rate (PCR) setting

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |

    - Example

prompt> ipoa show transport ipoa1

IPoA Transport: ipoa1

Description: ipoa1

SVC: false

Arp server ATM: here

Idle SVC lifetime: 60 secs

PVCs :

ID |  Port  |  Vpi   |  Vci   |  PCR

-----|--------|--------|--------|--------

1 | a1     | 0      | 900    | 59111

2 | a1     | 0      | 600    | 59111

3 | a1     | 0      | 800    | 59111

-----------------------------------------

If the IPoA transport has SVC enabled but also has PVC(s) reserved for future use, this command also displays the following information:

SVC is enabled - the PVC's listed below are NOT in use

    - See also

ipoa list transports

ipoa transport add pvc

    - Syntax

ipoa transport {<name>|<number>} add pvc <port> <vpi> <vci>

    - Description

This command adds a new Permanent Virtual Circuit (PVC) to an existing IPoA transport. You can add new PVCs to existing PVC and SVC IPoA transports. On SVC transports, the PVC(s) are reserved so that you can disable the SVC (using the ipoa set transport svc command) and run over PVC(s) if required.

PVC is a connection-oriented permanent leased line circuit between end stations on a network. This command allows you to specify the following PVC information:

- the ATM port that will transport IP data

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | 0 |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over.The VCI can be any value between 1 and 65535. | 0 |

    - Example

prompt> ipoa transport ipoa1 add pvc a1 0 800

    - See also

ipoa list transports

ipoa show transport

ipoa set transport svc

port list

ipoa transport delete pvc

    - Syntax

ipoa transport {<name>|<number>} delete pvc <pvc number>

    - Description

This command deletes a PVC from an IPoA transport that was added using the ipoa transport add pvc command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |
| pvc number | A number that identifies an existing PVC. The PVC numbers associated with an IPoA transport are displayed using the ipoa show transport command. PVC numbers appear in the ID column of the PVC table. For an example of this table, see ipoa show transport. | N/A |

    - Example

prompt> ipoa transport ipoa1 delete pvc 2

    - See also

ipoa list transports

ipoa show transport

ipoa transport add pvc

ipoa transport set pvc pcr

    - Syntax

ipoa transport {<name>|<number>} set pvc <pvc number> pcr <peak cell rate>

    - Description

This command sets the peak cell rate for an existing PVC.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |
| pvc number | A number that identifies an existing PVC. The PVC numbers associated with an IPoA transport are displayed using the ipoa show transport command. PVC numbers appear in the ID column of the PVC table. For an example of this table, see ipoa show transport. | N/A |
| peak cell rate | Determines the maximum rate at which ATM cells are allowed to be transported into the ATM network. The PCR can be any value from 0 up to the maximum PortSpeed parameter set when the port was created (using the initbun file in FlashFS or the CLI command port list) | 0 |

    - Example

prompt> ipoa transport ipoa1 set pvc 1 pcr 50000

    - See also

ipoa list transports

ipoa transport set pvc port

    - Syntax

ipoa transport {<name>|<number>} set pvc <pvc number> port <port>

    - Description

This command changes the port that an existing PVC uses to transport IPoA data.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |
| pvc number | A number that identifies an existing PVC. The PVC numbers associated with an IPoA transport are displayed using the ipoa show transport command. PVC numbers appear in the ID column of the PVC table. For an example of this table, see ipoa show transport. | N/A |
| port | The system port that is used to transport ATM data. The port/VPI/VCI combination must be unique for each transport. | N/A |

    - Example

prompt> ipoa transport ipoa1 set pvc 1 port a1

    - See also

ipoa list transports

ipoa show transport

port list

ipoa transport set pvc vpi

    - Syntax

ipoa transport {<name>|<number>} set pvc <pvc number> vpi <vpi>

    - Description

This command sets the VPI (Virtual Path Identifier) for an existing PVC. The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |
| pvc number | A number that identifies an existing PVC. The PVC numbers associated with an IPoA transport are displayed using the ipoa show transport command. PVC numbers appear in the ID column of the PVC table. For an example of this table, see ipoa show transport. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | 0 |

    - Example

prompt> ipoa transport ipoa1 set pvc 1 vpi 0

    - See also

ipoa list transports

ipoa show transport

ipoa transport set pvc vci

    - Syntax

ipoa transport {<name>|<number>} set pvc <pvc number> vci <vci>

    - Description

This command sets the VCI (Virtual Circuit Identifier) for an existing PVC.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPoA transport. To display transport names, use the ipoa list transports command. | N/A |
| number | A number that identifies an existing IPoA transport. To display transport numbers, use the ipoa list transports command. | N/A |
| pvc number | A number that identifies an existing PVC. The PVC numbers associated with an IPoA transport are displayed using the ipoa show transport command. PVC numbers appear in the ID column of the PVC table. For an example of this table, see ipoa show transport. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over.The VCI can be any value between 1 and 65535. | 0 |

    - Example

prompt> ipoa transport ipoa1 set pvc 1 vci 800

    - See also

ipoa list transports

ipoa show transport