- DHCP Relay CLI commands

This chapter describes the DHCP Relay CLI commands.

Summary

    - DHCP relay CLI commands

The table below lists the DHCP relay commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| dhcprelay add interface | dhcprelay add interface |
| dhcprelay clear interfaces | dhcprelay clear interfaces |
| dhcprelay delete interface | dhcprelay delete interface |
| dhcprelay list interfaces | dhcprelay list interfaces |
| dhcprelay add server | dhcprelay add server |
| dhcprelay clear servers | dhcprelay clear servers |
| dhcprelay delete server | dhcprelay delete server |
| dhcprelay enable¦disable | dhcprelay enable¦disable |
| dhcprelay list servers | dhcprelay list servers |
| dhcprelay show | dhcprelay show |
| dhcprelay update | dhcprelay update |

    - DHCP-relay Console commands

The table below lists the dhcp relay console commands and, if available, their equivalent CLI command:

| Command | CLI Equivalent |
| --- | --- |
| dhcprelay add | Replaced by CLI command dhcprelay add server |
| dhcprelay config | Replaced by CLI command dhcprelay list servers |
| dhcprelay delete | Replaced by CLI command dhcprelay delete server |
| dhcprelay help | Blacklisted command, see dhcprelay help |
| dhcprelay pool | Blacklisted command, see dhcprelay pool |
| dhcprelay status | Blacklisted command, see dhcprelay status |
| dhcprelay trace/untrace | Blacklisted command, see system log |
| dhcprelay version | Blacklisted command, see dhcprelay version |

dhcprelay add interface

    - Syntax

dhcprelay add interface <ipinterface>

    - Description

This command set DHCP relay to operate on a specific IP interface. The IP interface is defined as a DHCP rely IP interface. By setting DHCP server to operate on other interfaces, you can simultaneously use DHCP relay and server in your configuration.

Note – You cannot set both DHCP relay and DHCP server to operate on the same interface – you must use different interfaces for each.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ipinterface | The name of the existing interface that you want DHCP relay to operate on. To display interface names, use the ip list interfaces command. | N/A |

    - Example

prompt> dhcprelay add interface iplan

    - See also

dhcpserver add interface

ip list interfaces

dhcprelay clear interface

    - Syntax

dhcprelay clear interfaces

    - Description

This command delete all DHCP relay IP interfaces previously defined using the dhcprelay add interface command.

Note – This command does not delete the IP interfaces from the router.

    - Example

prompt> dhcprelay clear interfaces

    - See also

dhcprelay add interface

ip list interfaces

dhcprelay delete interface

    - Syntax

dhcprelay delete interface <ipinterface>

    - Description

This command deletes a single DHCP relay IP interface previously defined using the dhcprelay add interface command.

Note – This command does notdelete the IP interface from the router

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ipinterface | The name of the existing interface that you want DHCP relay to operate on. To display interface names, use the dhcprelay list interfaces command. | N/A |

    - Example

prompt> dhcprelay delete interface iplan

    - See also

dhcpserver add interface

dhcprelay list interfaces

dhcprelay list interfaces

    - Syntax

dhcprelay list interfaces

    - Description

This command list the existing DHCP relay IP interfaces previously defined using the dhcprelay add interface command. It displays the following information:

- DHCP Relay interface ID number

- IP interface name

    - Example

prompt> dhcprelay list interfaces

DHCP Relay Interfaces:

ID  |    Name

--------------------

1   |    iplan

--------------------

    - See also

dhcprelay add interface

dhcprelay add server

    - Syntax

dhcprelay add server <ipaddress>

    - Description

This command adds the IP address of a DHCP server subnet to DHCP relay’s list of server IP addresses. Records of new IP addresses added are not updated until the dhcprelay update command has been entered.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ipaddress | The IP address of a DHCP server that DHCP relay can use. The IP address is displayed in the following format: 192.168.102.3 | N/A |

    - Example

prompt> dhcprelay add server 239.252.197.0

    - See also

dhcpserver list subnets

dhcprelay update

dhcprelay clear servers

    - Syntax

dhcprelay clear servers

    - Description

This command deletes all DHCP server IP addresses stored in DHCP relay’s list of server IP addresses.

    - Example

prompt> dhcprelay clear servers

    - See also

dhcprelay delete server

dhcprelay delete server

    - Syntax

dhcprelay delete server <number>

    - Description

This command deletes a single DHCP server address stored in DHCP relay’s list of server IP addresses.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| number | A number that identifies the DHCP server in the DHCP relay list. To display server numbers, use the dhcprelay list servers command. | N/A |

    - Example

prompt> dhcprelay delete server 3

    - See also

dhcprelay list servers

dhcprelay clear servers

dhcprelay enable|disable

    - Syntax

dhcprelay {enable|disable}

    - Description

This command enables/disables DHCP relay. You must have DHCP relay enabled in order to carry out any DHCP relay configuration. If you try configuring DHCP relay before you’ve entered the dhcprelay disable command, the CLI issues a warning message.

You cannot have DHCP relay and DHCP server enabled at the same time. If you try to configure DHCP relay when DHCP server is enabled, the CLI issues a warning message.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enable | Enables configuration of DHCP relay. | enable |
| disable | Disables configuration of DHCP relay. |  |

    - Example

prompt> dhcprelay enable

    - See also

dhcpserver enable|disable

dhcprelay list servers

    - Syntax

dhcprelay list servers

    - Description

This command displays the DHCP relay’s list of DHCP server IP addresses with their identification numbers.

    - Example

prompt> dhcprelay list servers

DHCP Servers:

ID  | IP Address

-----|------------------

1 | 192.168.102.3

2 | 239.252.197.0

------------------------

    - See also

dhcpserver list subnets

dhcprelay show

    - Syntax

dhcprelay show

    - Description

This command tells you whether DHCP relay is enabled or disabled.

    - Example

prompt> dhcprelay show server

Global DHCP Relay Configuration:

Status: ENABLED

    - See also

dhcprelay enable|disable

dhcprelay update

    - Syntax

dhcprelay update

    - Description

This command updates the DHCP relay configuration. Changes made to the relay configuration will not take effect until this command has been entered.

    - Example

prompt> dhcprelay update

dhcprelay: Reset request acknowledged. Reset imminent.