- Security CLI commands

This chapter describes the Security CLI commands. The Security module is the parent module in the Security package.

Summary

    - About the VMI Security package

The Security package allows you to build security features into your system. The package contains three modules comprising of one parent module and two child modules:

- The Security module is the parent module in the Security package. The Security commands allow you to:

- enable/disable all modules in the Security package (including the child modules; NAT and Firewall)

- add IP interfaces to the Security package to create security interfaces that are used to configure the NAT and Firewall child modules.

- set triggers to allow applications to open secondary port sessions

- The NAT module is a child module in the Security package. The NAT commands allow you to:

- implement NAT based on the security interfaces added to the Security package by the parent Security module

- enable NAT between an interface and an interface type

- create global address pools

- map global IP addresses to individual hosts on the inside network.

- The stateful Firewall module is a child module in the Security package. The Firewall commands allow you to:

- implement stateful Firewall features based on the security interfaces added in the parent Security module

- set security levels and policies that determine whether traffic between interfaces is allowed or blocked

- set portfilters to determine which ports and protocols are allowed or blocked

- set IDSs (Intrusion Detection Settings).

For information on NAT commands, see NAT CLI commands .

For information on stateful Firewall commands, see Firewall CLI commands .

    - Security CLI commands

The table below lists the security commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| security enable¦disable | security |
| security status | security status |
| security add interface | security add interface |
| security clear interfaces | security clear interfaces |
| security delete interface | security delete interface |
| security list interfaces | security list interfaces |
| security show interface | security show interface |
| security add management-station range | security add mgmt-station |
| security delete management-station | security delete mgmt-station |
| security list mangement-station | security list mgmt-stations |
| security set management-station | security set mgmt-station |
| security add trigger tcp¦udp | security add trigger tcp¦udp |
| security add trigger netmeeting | security add trigger netmeeting |
| security clear triggers | security clear triggers |
| security delete trigger | security delete trigger |
| security list triggers | security list triggers |
| security set trigger addressreplacement | security set trigger addressreplacement |
| security set trigger multihost | security set trigger multihost |
| security set trigger binaryaddressreplacement | security set trigger binaryaddressreplacement |
| security set trigger maxactinterval | security set trigger maxactinterval |
| security set trigger endport | security set trigger endport |
| security set trigger startport | security set trigger startport |
| security set trigger secondaryendport | security set trigger secondaryendport |
| security set trigger secondarystartport | security set trigger secondarystartport |
| security set trigger sessionchaining | security set trigger sessionchaining |
| security set trigger UDPsessionchaining | security set trigger UDPsessionchaining |
| security show trigger | security show trigger |

security

    - Syntax

security {enable | disable}

    - Description

This command explicitly enables/disables all modules in the Security package (including the child modules; NAT and Firewall). You must enable the Security package if you want to use the NAT and/or Firewall modules to configure security for your system.

If you disable the Security package during a session, any configuration changes made to the Security, NAT or Firewall modules when the package was enabled remain in the system, so that you can re-enable them later in the session. If you need to reboot your system but want to save the security configuration between sessions, use the system config save command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | Enables all modules in the Security package (Security, NAT and Firewall modules). | disabled |
| disabled | Disables all modules in the Security package (Security, NAT and Firewall modules). |  |

    - Example

prompt> security enable

    - See also

firewall set securitylevel

system config save

security status

    - Syntax

security status

    - Description

This command displays the following information about the Security package:

- Security status (enabled or disabled)

- Firewall status (enabled or disabled)

- Firewall security level setting (none, high, low, or medium)

- Firewall session logging (enabled or disabled)

- Firewall blocking logging (enabled or disabled)

- Firewall intrusion logging (enabled or disabled)

- NAT status (enabled or disabled)

    - Example

prompt> security status

Security enabled.

Firewall disabled.

Firewall security level: none.

Firewall session logging enabled.

Firewall blocking logging enabled.

Firewall intrusion logging disabled.

NAT enabled

    - See also

security

firewall set securitylevel

security add interface

    - Syntax

security add interface <name> {external|internal|dmz}

    - Description

This command adds an existing IP interface to the Security package to create a security interface, and specifies what type of interface it is depending on how it connects to the network.

Once you have added security interfaces, you can use them in the NAT and/or Firewall configurations.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| external | An interface that connects to the external network. | N/A |
| internal | An interface that connects to the internal network | N/A |
| dmz | An interface that connects to the de- militarized zone (DMZ) | N/A |

    - Example

prompt> security add interface ip1 internal

    - See also

ip list interfaces

Firewall CLI commands

NAT CLI commands

security clear interfaces

    - Syntax

security clear interfaces

    - Description

This command removes all security interfaces that were added to the Security package using the security add interface command.

    - Example

prompt> security clear interfaces

    - See also

security delete interface

security delete interface

    - Syntax

security delete interface <name>

    - Description

This command removes a single security interface that was added to the Security package using the security add interface command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing security interface. To display interface names, use the security list interfaces command. | N/A |

    - Example

prompt> security delete interface f1

    - See also

security clear interfaces

security list interfaces

security list interfaces

    - Syntax

security list interfaces

    - Description

This command lists the following information about security interfaces that were added to the Security package using the security add interface command:

- Interface ID number

- Interface name

- Interface type (external, internal or DMZ)

    - Example

prompt> security list interfaces

Security Interfaces:

ID  |   Name   | Type

-----|----------|----------

1 | i1       | internal

2 | i2       | external

3 | i3       | dmz

---------------------------

    - See also

security show interface

security show interface

    - Syntax

security show interface <name>

    - Description

This command displays information about a single interface that was added to the Security package using the security add interface command. The following interface information is displayed:

- Interface name

- Interface type (external, internal or DMZ)

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing security interface. To display all interface names, use the security list interfaces command. | N/A |

    - Example

prompt> security show interface f2

Interface name: f2

Interface type: internal

    - See also

security list interfaces

security add mgmt-station

Note – Management Station support is part of Remote Access feature.

    - Syntax

security add mgmt-station <name> {range <start_addr> <end_addr> | subnet <address> <mask>} <transport_type> <port> <idle_timeout>

    - Description

This command creates Management Station(s) that allows a specific host (or range of hosts) to access your device directly, bypassing NAT and Firewall. IP packets from a Management Station are sent to the external interface (WAN) using a specific transport and port number. The Management Station is not enabled until you enable it using security set mgmt-station command or enable Remote Access using the WEB GUI.

The idle timeout setting configured here is overruled by the idle timeout setting configured using the WEB GUI.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| start_addr | The first remote host IP address in the range allowed. | N/A |
| end_addr | The last remote host IP address in the range allowed. | N/A |
| address | A specific IP address in the remote subnet allowed. | N/A |
| mask | The mask defining the remote subnet allowed. | N/A |
| transport_type | The number of the transport type used, e.g. TCP = 6, UDP = 17, wildcard = 255.. | N/A |
| port | The port number used. This is only effective if the transport_type is set to 6 (TCP) or 17 (UDP). The wildcard is 65535. | N/A |
| Idle_timeout | The idle time (in minutes). If no sessions are created by the Management Station within this setting, the Station is disabled. If a session is created, that session uses the idle time set and the Station is not disabled until the session expired. | N/A |

    - Example

prompt> security add mgmt-station ISP subnet 192.168.1.1 255.255.255.0 17 26 10

    - See also

security set mgmt-station

security delete mgmt-station

Note – Management Station support is part of Remote Access feature.

    - Syntax

security delete mgmt-station <name>

    - Description

This command deletes a single Management Station that was added to the Security module using the security add mgmt-station command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |

    - Example

prompt> security delete mgmt-station ISP

    - See also

security add mgmt-station

security list mgmt-station

security list mgmt-station

Note – Management Station support is part of Remote Access feature.

    - Syntax

security list mgmt-stations

    - Description

This command lists Management Stations that were added to the Security module using security add mgmt-station command. It displays the following information about Management Stations:

- Management Station ID number

- Management Station Name

- subnet status (true/false)

- IP address (of subnet or first address in range)

- subnet mask or last address of range

- transport type

- port number

- idle timeout (minutes)

- enabled status (true/false)

    - Example

prompt> security list mgmt-stations

Management Stations:

ID  |   Name   | Subnet  | IP address | Mask/End Address | Transp | Port  | Idle | Enable

-----|----------|---------|------------|------------------|--------|-------|------|-------

1 | ISP      | true    | 192.168.1.1 | 255.255.255.0   | 6      | 65535 | 15   | true

-----------------------------------------------------------------------------------------

    - See also

security add mgmt-station

security set mgmt-station

Note – Management Station support is part of Remote Access feature.

    - Syntax

security set mgmt-station {enabled|disabled}

    - Description

This command enables a Management Station that was added to the Security module using security add mgmt-station command.

The enable/disable setting configured here is overruled by the enable/disable setting configured using the WEB GUI.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| enabled | Enables the Management Station. Once enabled, Management Station sessions can be created | disabled |
| disabled | Disables the Management Station. |  |

    - Example

prompt> security set mgmt-station ISP enabled

    - See also

security add mgmt-station

security list mgmt-stations

security add trigger tcp|udp

    - Syntax

security add trigger <name> {tcp|udp} <startport> <endport> <maxactinterval>

    - Description

This command adds a trigger to the Security module. A trigger allows an application to open a secondary port in order to transport packets.

Some applications, such as FTP, need to open secondary ports - they have a control session port (21 for FTP) but also need to use a second port in order to transport data. Adding a trigger means that you do not have to define static portfilters to open ports for each secondary session. If you did this, the ports would remain open for potential use (or misuse, see the command firewall set IDS SCANattackblock) until the portfilters were deleted. A trigger opens a secondary port dynamically, and allows you to specify the length of time that it can remain inactive before it is closed.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the trigger. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| tcp | Adds a trigger for a TCP application to the security package. | N/A |
| udp | Adds a trigger for a UDP application to the security package. | N/A |
| startport | Sets the start of the trigger port range for the control session. | N/A |
| endport | Sets the end of the trigger port range for the control session. | N/A |
| maxactinterval | Sets the maximum interval time (in milliseconds) between the use of secondary port sessions. If a secondary port opened by a trigger has not been used for the specified time, it is closed. | 3000 |

    - Example

The following example creates a Netmeeting (H323) trigger:

prompt> security add trigger t1 tcp 1720 1720 30000

    - See also

security list triggers

security add trigger netmeeting

security add trigger netmeeting

    - Syntax

security add trigger <name> netmeeting

    - Description

This command allows you to use the example trigger provided by the CLI. It allows you to add a trigger to allow Netmeeting to transport data through the security package. This application opens a secondary port session. You do not have to set the port range or maxactinterval for a Netmeeting trigger - the CLI automatically sets this for you.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the trigger. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |

    - Example

prompt> security add trigger t2 netmeeting

    - See also

security list triggers

security add trigger tcp|udp

security clear triggers

    - Syntax

security clear triggers

    - Description

This command deletes all triggers that were added to the Security module using the security add trigger commands.

    - Example

prompt> security clear triggers

    - See also

security delete trigger

security delete trigger

    - Syntax

security delete trigger <name>

    - Description

This command deletes a single trigger that was added to the Security module using the security add trigger commands.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list trigger command. | N/A |

    - Example

prompt> security delete trigger t2

    - See also

security list triggers

security clear triggers

security list triggers

    - Syntax

security list triggers

    - Description

This command lists triggers that were added to the Security module using the security add trigger command. It displays the following information about triggers:

- Trigger ID number

- Trigger name

- Trigger transport type (TCP or UDP)

- Port range

- Secondary port range

- Interval

    - Example

prompt> security list triggers

Security Triggers:

ID | Name  | Type | Port Range   |  Sec Port Range |Interval

--------------------------------------------------------------

1 |  tr1  | tcp  | 21   - 21    |  1720 – 1720    |3000

--------------------------------------------------------------

    - See also

security show trigger

security set trigger addressreplacement

    - Syntax

security set trigger <name> addressreplacement {none|tcp|udp|both}

    - Description

The settings in this command are only effective if you enable address translation using the command security set trigger binaryaddressreplacement.

This command allows you to specify what type of address replacement is set on an trigger. Incoming packets are searched in order to find their embedded IP address. The address is then replaced by the correct inside host IP address, and NAT translates the packets to the correct destination.

You can specify whether you want to carry out address replacement on TCP packets, on UDP packets or on both TCP and UDP packets.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an trigger. To display trigger names, use the security list triggers command. | N/A |
| none | Disables address replacement. | none |
| tcp | Sets address replacement on TCP packets for an existing trigger. |  |
| udp | Sets address replacement on UDP packets for an existing trigger. |  |
| both | Sets address replacement on TCP and UDP packets for an existing trigger. |  |

    - Example

prompt> security set trigger t2 addressreplacement tcp

    - See also

security set trigger binaryaddressreplacement

security set trigger multihost

    - Syntax

security set trigger <name> multihost {enable | disable}

    - Description

This command sets whether or not a secondary session can be initiated to/from different remote hosts or the same remote host on an existing trigger.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |
| enable | A secondary session can be initiated to/from different remote hosts. | disable |
| disable | A secondary session can only be initiated to/from the same remote host. |  |

    - Example

prompt> security set trigger t1 multihost enable

    - See also

security list triggers

security set trigger binaryaddressreplacement

    - Syntax

security set trigger <name> binaryaddressreplacement {enable | disable}

    - Description

This command enables/disables binary address replacement on an existing trigger. You can then set the type of address replacement (TCP, UDP, both or none) using the command security set trigger addressreplacement.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |
| enable | Enables the use of binary address replacement on an existing trigger. | disable |
| disable | Disables the use of binary address replacement on an existing trigger. |  |

    - Example

prompt> security set trigger t5 binaryaddressreplacement enable

    - See also

security set trigger addressreplacement

security list triggers

security set trigger maxactinterval

    - Syntax

security set trigger <name> maxactinterval <interval>

    - Description

This command sets the maximum activity interval limit on existing session entries for an existing trigger.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |
| interval | Sets the maximum interval time (in milliseconds) between the use of secondary port sessions. If a secondary port opened by a trigger has not been used for the specified time, it is closed. | N/A |

    - Example

prompt> security set trigger t2 maxactinterval 5000

    - See also

security list triggers

security set trigger endport

    - Syntax

security set trigger <name> endport <portnumber>

    - Description

This command sets the end of the port number range for an existing trigger.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |
| portnumber | Sets the end of the trigger port range. | N/A |

    - Example

prompt> security set trigger t3 endport 21

    - See also

security set trigger startport

security set trigger startport

    - Syntax

security policy <name> set trigger startport <portnumber>

    - Description

This command sets the start of the port number range for an existing trigger.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |
| port | Sets the start of the trigger port range. | N/A |

    - Example

prompt> security set trigger t3 startport 21

    - See also

security set trigger endport

security set trigger secondaryendport

    - Syntax

security set trigger <name> secondaryendport <portnumber>

    - Description

This command sets the end of the secondary port number range for an existing trigger. It allows you to restrict ports that a trigger will open, however, this is not usually necessary.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |
| portnumber | Sets the end of the trigger port range. | 65535 |

    - Example

prompt> security set trigger t3 secondaryendport 1933

    - See also

security set trigger secondarystartport

security set trigger secondarystartport

    - Syntax

security policy <name> set trigger secondarystartport <portnumber>

    - Description

This command sets the start of the secondary port number range for an existing trigger. It allows you to restrict the ports that a trigger will open, however, this is not usually necessary.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |
| port | Sets the start of the trigger port range. | 1024 |

    - Example

prompt> security set trigger t3 startport 1923

    - See also

security set trigger secondaryendport

security set trigger sessionchaining

    - Syntax

security set trigger <name> sessionchaining {enable | disable}

    - Description

This command determines whether or not a triggering protocol can be chained. If session chaining is enabled, TCP dynamic sessions also become triggering sessions, which allows multi-level session triggering.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |
| enable | Enables TCP sessionchaining on an existing trigger. | disable |
| disable | Disables all session chaining (TCP and UDP) on an existing trigger. |  |

    - Example

prompt> security set trigger t4 sessionnchaining enable

    - See also

security set trigger UDPsessionchaining

security set trigger UDPsessionchaining

    - Syntax

security set trigger <name> UDPsessionchaining {enable | disable}

    - Description

You must set the security set trigger sessionchaining enable command in order for this command to become effective.

If UDP session chaining is enabled, both UDP and TCP dynamic sessions also become triggering sessions, which allows multi-level session triggering.

Note - This CLI command is case-sensitive. You must type the command attributes exactly as they appear in the syntax section on this page. If you do not use the same case-sensitive syntax, the command fails and the CLI displays a syntax error message.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |
| enable | Enables UDP sessionchaining on an existing trigger. TCP and UDP session chaining is allowed if the security set trigger sessionchaining command is enabled. | disable |
| disable | Disables UDP session chaining on an existing trigger. TCP session chaining is allowed if the security set trigger sessionchaining command is enabled. |  |

    - Example

prompt> security set trigger t3 UDPsessionchaining enable

    - See also

security set trigger sessionchaining

security show trigger

    - Syntax

security show trigger <name>

    - Description

This command displays information about a single trigger that was added to the Security module using the security add trigger command. The following trigger information is displayed:

- Trigger name

- Transport type (TCP or UDP)

- Start of the port range

- End of the port range

- Multiple host permission (true/false)

- Maximum activity interval (in milliseconds)

- Session chaining permission (true/false)

- Session chaining on UDP permission (true/false)

- Binary address replacement permission (true/false)

- Address translation type (UDP, TCP, none or both)

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing trigger. To display trigger names, use the security list triggers command. | N/A |

    - Example

prompt> security show trigger t2

Security Trigger: t2

Transport Type: tcp

Starting port number: 1000

Ending port number: 1000

Allow multiple hosts: false

Max activity interval: 30000

Session chaining: false

Session chaining on UDP: false

Binary address replacement: false

Address translation type: none

    - See also

security list triggers