- Port CLI commands

This chapter describes CLI support for ports.

Summary

    - Port CLI commands

The table below lists the port commands provided by the CLI:

| Command | Reference |
| --- | --- |
| port ? | port ? |
| port list | port list |
| port set | port set |
| port show | port show |

    - Port Console commands

The table below lists the port console commands and, if available, their equivalent CLI command:

| Command | CLI Equivalent |
| --- | --- |
| bun list ports | Replaced by port list |
| bun set port | Replaced by port set |
| bun show port | Replaced by port show |

port ?

    - Syntax

port ?

    - Description

This command lists the ports that are currently available in your system. It depends on the hardware you are using.

    - Example

prompt> port ?

a1			Port name

ethernet		Port name

list			List ports by type

wireless		Port name

    - See also

port list

port set

port show

port list

    - Syntax

port list ?

port list <port class>

    - Description

The port list ? command lists all possible types of port and their port classes. The port list <port class> command allows you to display the port names that belong to a specific class (if that class is available in your system).

    - Example

prompt> port list ?

adsl			Port class

all			Port class

atm			Port class

ethernet		Port class

uart			Port class

prompt> port list

The last line of the port list ? command output displays the first part of the port list <port class> command.

    - See also

port ?

port set

port show

port set

    - Syntax

port <portname> set <attribute> <value>

    - Description

This command allows you to modify attributes on a port. Any modifications override existing attribute values specified in your device and compiled at run-time. The attributes available depend on:

- the type of port that you are using

- the system that you are using

To display a list of valid attributes for a specific port, use the ‘?’ syntax options key after port <portname> set. For example:

prompt> port ethernet set ?

MaxMulticastListSize

MaxQueue

Reset

Disable

PromiscuousEnable

RxTestEnable

RxBroadcastEnable

RxMulticastEnable

RxMulticastAllEnable

RxAddressEnable

RxShortPktEnable

FastClockEnable

FullDuplexEnable

CrcEnable

PadShortDataEnable

Loopback

RxAnyEnable

HwFilterHigh

HwFilterLow

MAC

SnmpIfAdminStatus

SnmpIfLinkUpDownTrapEnable

SnmpIfPromiscuousMode

SnmpIfAlias

Once you have identified the attribute that you want to modify, you can specify the new value that you want to set it to.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| portname | The name of a port that is configured in your system. To display available ports, use the port command. | N/A |
| attribute | A single attribute of a specified port. An attribute has a value attached to it which you can modify. | N/A |
| value | A value attached to an attribute. The value could be a numerical setting or a true/false qualifier. | N/A |

    - Example

prompt> port a1 set DefaultPCR 3000

    - See also

port list

port show

port show

    - Syntax

port <portname> show

    - Description

This command displays the current attributes and values of a port. The attributes available depend on:

- the type of port that you are using

- the system that you are using

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| portname | The name of a port that is configured in your system. To display available ports, use the port command. | N/A |

    - Example

prompt> port hfa3841 show

UpdateStats                     = 0

Version                         = 1.10

PortClassEthernet               = true

Reset                           = false

Disable                         = false

PromiscuousEnable               = false

RxBroadcastEnable               = true

RxMulticastEnable               = true

RxMulticastAllEnable            = true

RxAddressEnable                 = true

FullDuplexEnable                = true

CrcEnable                       = true

MAC                             = 00:90:96:00:00:49

ApFirmwareVersion               =

PrimaryFirmwareVersion          =

ESSID                           = act1

DefaultChannel                  = 1

WepEncryption                   = disabled

DefaultTxKey                    = 0

Mode64Key0                      = 00-00-00-00-00

Mode64Key1                      = 00-00-00-00-00

Mode64Key2                      = 00-00-00-00-00

Mode64Key3                      = 00-00-00-00-00

Mode128Key0                     = 00-00-00-00-00-00-00-00-00-00-00-00-00

Mode128Key1                     = 00-00-00-00-00-00-00-00-00-00-00-00-00

Mode128Key2                     = 00-00-00-00-00-00-00-00-00-00-00-00-00

Mode128Key3                     = 00-00-00-00-00-00-00-00-00-00-00-00-00

RtsThreshold                    = 2432

FragmentationThreshold          = 2346

TxUnicastFrames                 = 0

TxMulticastFrames               = 0

TxFragments                     = 0

TxUnicastOctets                 = 0

TxMulticastOctets               = 0

TxDeferredTransmissions         = 0

TxSingleRetryFrames             = 0

TxMultipleRetryFrames           = 0

TxRetryLimitExceeded            = 0

TxDiscards                      = 0

RxUnicastFrames                 = 0

RxMulticastFrames               = 0

RxFragments                     = 0

RxUnicastOctets                 = 0

RxMulticastOctets               = 0

RxFCSErrors                     = 0

RxDiscardsNoBuffer              = 0

TxDiscardsWrongSA               = 0

RxDiscardsWEPUndecryptable      = 0

RxMessageInMsgFragments         = 0

RxMessageInBadMsgFragments      = 0

RxOk                            = 0

TxOk                            = 0

TxAborts                        = 0

RxCRCErrors                     = 0

TxCollisions                    = 0

RxOverlongPackets               = 0

TxExcessiveCollisions           = 0

RxShortPackets                  = 0

SnmpIfIndex                     = 2

SnmpIfDescr                     = BUN Ethernet device

SnmpIfType                      = 6

SnmpIfMtu                       = 1500

SnmpIfSpeed                     = 100000000

SnmpIfPhysAddress               = 00:90:96:00:00:49

SnmpIfOperStatus                = 1

SnmpIfAdminStatus               = 1

SnmpIfLastChange                = 218

SnmpIfInOctets                  = 0

SnmpIfInUcastPkts               = 0

SnmpIfInNUcastPkts              = 0

SnmpIfInDiscards                = 0

SnmpIfInErrors                  = 0

SnmpIfInUnknownProtos           = 0

SnmpIfOutOctets                 = 0

SnmpIfOutUcastPkts              = 0

SnmpIfOutNUcastPkts             = 0

SnmpIfOutDiscards               = 0

SnmpIfOutErrors                 = 0

SnmpIfOutQLen                   = 0

SnmpIfSpecific                  = (unset)

SnmpIfLinkUpDownTrapEnable      = 2

SnmpIfPromiscuousMode           = 2

SnmpIfAlias                     =

    - See also

port list

port set