- Web Server CLI commands

This chapter describes the Web Server CLI commands.

Summary

    - Web Server CLI commands

The table below lists the Web Server commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| webserver clear stats | webserver clear stats |
| webserver enable¦disable | webserver enable¦disable |
| webserver logout | webserver logout |
| webserver set idletimeout | webserver set idletimeout |
| webserver set interface | webserver set interface |
| webserver set managementip | webserver set managementip |
| webserver set port | webserver set port |
| webserver set telnetport | webserver set telnetport |
| webserver set upnp | webserver set upnp |
| webserver set upnpport | webserver set upnpport |
| webserver show info | webserver show info |
| webserver show stats | webserver show stats |

    - Web Server Console commands

The table below lists the Web Server console commands and, if available, their CLI equivalent command:

| Command | CLI Equivalent |
| --- | --- |
| webserver archive | Usable command see webserver archive |
| webserver clearstats | Replaced by CLI command webserver clear stats |
| webserver {enable¦disable} | Replaced by CLI command webserver enable¦disable |
| webserver load | Usable command see webserver archive |
| webserver mgmtip | Replaced by CLI command webserver set managementip |
| webserver port | Replaced by CLI command webserver set port |
| webserver stats | Replaced by CLI command webserver show stats |
| webserver status | Replaced by CLI command webserver show stats |
| webserver version | Replaced by CLI command webserver show stats |

webserver clear stats

    - Syntax

webserver clear stats

    - Description

This command sets all of the Web Server process counters to 0.

    - Example

prompt> webserver clear stats

    - See also

webserver show info

webserver enable|disable

    - Syntax

webserver {enable|disable}

    - Description

This command enables or disables the Web Server process.

By default, the Web Server process is enabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enable | Enables the Web Server process. | enable |
| disable | Disables the Web Server process. |  |

    - Example

prompt> webserver disable

WebServer is disabled

webserver logout

    - Syntax

webserver logout

    - Description

The web server is allowed only one user connect to at a time. Before the user logout, the other users cannot login to web server. Sometimes, the logged user did not logout before he/she closes the browser. This command force the association released immediately. Then, the other users can access the web server.

Besides this command, there is a idle timer to release the pending association in the background. Please refer to webserver set idletimeout command for details.

    - Example

prompt> webserver logout

    - See also

webserver set idletimeout

webserver set idletimeout

    - Syntax

webserver set idletimeout <value>

    - Description

This command specifies the idle time for releasing the pending association. If the user forget to logout before closing browser, the web server will release the association automatically when the time expires.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default |
| --- | --- | --- |
| value | The idle time (in seconds) to logout the existing user automatically. | 300 |

    - Example

prompt> webserver set idletimeout 60

    - See also

webserver logout

webserver set interface

    - Syntax

webserver set interface <interface>

    - Description

This command specifies the name of an IP interface that the system will use for UPnP (Universal Plug and Play) communication with other devices on the local area network.

By default, your system creates an IP interface with an Ethernet transport attached to it. This interface is called iplan, and it is the default interface that UPnP uses for its communication.

Once you have set the UPnP interface, the IGD (Internet Gateway Device) monitors the interface. The IGD can handle changes to the interface definition (for example, if the IP address changes through a DCHP update, the IGD will use the newly assigned address).

You must save your configuration (see system config save) and restart your system (see system restart) to activate the Web Server settings.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default |
| --- | --- | --- |
| Interface | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | iplan |

    - Example

prompt> webserver set interface ip

    - See also

webserver set upnpport

webserver set managementip

    - Syntax

webserver set managementip <ip-address>

    - Description

This command allows connection requests to be restricted to only one IP address, (e.g. from an IP address that is used by a management entity) or from any IP address (by setting the IP address to 0.0.0.0).

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ip-address | The only IP address that the Web Server will allow connection requests from. The IP address is displayed in the following format: 192.168.102.3 | 0.0.0.0 |

    - Example

prompt> webserver set managementip 192.168.102.3

webserver set port

    - Syntax

webserver set port <port>

    - Description

This command sets the HTTP port number that the Web Server process will use to transfer data.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| port | A valid port number that must be between 0 and 65535. | 80 |

    - Example

prompt> webserver set port 100

    - See also

webserver set telnetport

webserver set upnpport

webserver set telnetport

    - Syntax

webserver set telnetport <port>

    - Description

This command sets the Telnet port number that the Web Server process will use for Telnet communication.

You must save your configuration (see system config save) and restart your system (see system restart) to activate the Web Server settings.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| port | A valid UPnP port number that must be between 0 and 65535. | 23 |

    - Example

prompt> webserver set telnetport 200

    - See also

webserver set port

webserver set upnpport

webserver set upnp

    - Syntax

webserver set upnp <enable|disable>

    - Description

This command enables/disable the UPnP (Universal Plug and Play) function in the Web Server process will use for UPnP communication. When UPnP disabled, the Web Server process will not perform the UPnP communicateion any more.

This command will take effect immediately.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enable¦disable | Enable or disable the UPnP function. | enable |

    - Example

prompt> webserver set upnp disable

    - See also

webserver set upnpport

webserver set upnpport

    - Syntax

webserver set upnpport <port>

    - Description

This command sets the UPnP (Universal Plug and Play) port number that the Web Server process will use for UPnP communication.

You must save your configuration (see system config save) and restart your system (see system restart) to activate the Web Server settings.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| port | A valid UPnP port number that must be between 0 and 65535. | N/A |

    - Example

prompt> webserver set upnpport 280

    - See also

webserver set upnp

webserver show info

    - Syntax

webserver show info

    - Description

This command displays the following information about the Web Server process:

- EmWeb (Embedded Web Server) release details

- Web Server enabled status (true or false)

- Interface set

- HTTP port set

- UPnP port set

- Telnet port set

- Auxillary HTTP port set

- Management IP address

    - Example

prompt> webserver show info

Web server configuration:

EmWeb release: R6_1_0

Enabled: true

Interface: iplan

HTTP port: 80

UPnP port: 2800

Telnet port: 23

Auxillary HTTP port: 8008

Management IP address: 1.2.3.4

    - See also

webserver clear stats

webserver show stats

    - Syntax

webserver show stats

    - Description

This command tells you how many bytes have been transmitted and received by the Web Server.

    - Example

prompt> webserver show stats

Web Server statistics:

Bytes transmitted:       2122

Bytes received:          0

    - See also

webserver show info