Scheduler CLI commands

This chapter describes the Scheduler CLI commands

| 1 Summary |  |  |
| --- | --- | --- |
| 1.1 Scheduler CLI commands |  |  |
|  | The table below lists the scheduler commands provided by the CLI: |  |

| Command | Description/Console command |
| --- | --- |
| scheduler add profile | scheduler add profile |
| scheduler clear profiles | scheduler clear profiles |
| scheduler delete profile | scheduler delete profile |
| scheduler list profiles | scheduler list profiles |
| scheduler set profile shaping | scheduler set profile shaping |
| scheduler profile set queue weight | scheduler profile set queue weight |
| scheduler profile set queue ald profile | scheduler profile set queue ald profile |
| scheduler profile set queue ald disabled | scheduler profile set queue ald disabled |
| scheduler show profile | scheduler show profile |
| scheduler show profile queues | scheduler show profile queues |

| 2 | scheduler add profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2.1 |  |  |  | Syntax |  |  |  |  |  |
|  | scheduler add profile <name> {wf2qplus ¦ priority} |  |  |  |  |  |  |  |  |  |
|  | 2.2 |  |  |  | Description |  |  |  |  |  |
|  |  |  |  |  | This command creates a scheduler profile with either WF2Q+ or Priority Queuing set as the type of service discipline. Classifier assigns scheduling priority to a packet (configured by the classifier profile set rule priority command). This value determines the scheduling behavior received by a packet. If a packet is not classified, scheduling priority 0 is assigned to the packet by default. |  |  |  |  |  |
|  |  |  |  |  | If you select the priority service discipline, the profile supports 8 queues, each corresponding to a scheduling priority from 0-7. Each queue can hold a maximum of 32 packets at any instant. Scheduling priority value 0 assigns the lowest priority while 7 assigns the highest priority. |  |  |  |  |  |
|  |  |  |  |  | If you select the wf2qplus service discipline, the weight value (percentage) configured for a scheduling priority determines the share of bandwidth received by packets belonging to the scheduling priority level. A single default queue with a weight of 100% is created by default. Packets with scheduling priority 0 are always enqueued in the default queue. You may configure weights for scheduling priority levels (1-7) using the scheduler profile set queue weight command. |  |  |  |  |  |
|  |  |  |  |  | If you do configure weight for a scheduling priority level, the corresponding packets are enqueued in a separate queue and the bandwidth is allocated in accordance with the configured weights. Otherwise, packets are enqueued in the default queue. |  |  |  |  |  |
|  |  |  |  |  | Each queue can hold a maximum of 32 packets at any instance. |  |  |  |  |  |
|  |  |  |  |  | In addition, this created scheduler profile may be configured with shaping and algorithmic dropper (RED/WRED): |  |  |  |  |  |
|  |  |  |  |  | • Shaping: you can configure the scheduler profile for shaping by using the scheduler set profile shaping command. Shaping is disabled by default. |  |  |  |  |  |
|  |  |  |  |  | • Algorithmic Dropper: you can configure Algorithmic Dropper (RED/WRED) for a queue by using the scheduler profile set queue ald profile command. |  |  |  |  |  |
|  |  |  |  | You can set the profile to work on a transport by using the transport set scheduler profile command. |  |  |  |  |  |  |
|  |  | 2.3 | Options |  |  |  |  |  |  |  |
|  |  |  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Option | Description | Default value |  |  |
|  |  |  |  |  |  |  | An arbitrary name that identifies the profile. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. |  |  |  |
|  |  |  |  |  |  | name |  | N/A |  |  |
|  |  |  |  |  |  |  | WF2Q+ (Worst Case Weighted Fair Queueing Plus) service discipline. This service discipline distributes the link bandwidth among participating queues in ratio of their respective configured weights. |  |  |  |
|  |  |  |  |  |  | wf2qplus |  | N/A |  |  |
|  |  |  |  |  |  | priority | Priority Queueing provides prioritized treatment to higher priority traffic. | N/A |  |  |

| 2.4 | Example prompt> scheduler add profile prioschlr priority |
| --- | --- |
| 2.5 | See also |
|  | scheduler show profile |

| 3 | scheduler clear profiles |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3.1 Syntax |  |  |  |
| scheduler clear profiles |  |  |  |  |  |
|  |  | 3.2 Description |  |  |  |
|  | This command deletes all scheduler profiles that were previously created using the scheduler add profile command. |  |  |  |  |
|  |  | 3.3 Example |  |  |  |
| prompt> scheduler clear profiles |  |  |  |  |  |
|  |  | 3.4 See also |  |  |  |
|  |  |  | scheduler add profile |  |  |

| 4 |  | scheduler delete profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 4.1 |  |  | Syntax |  |  |  |  |  |  |
| scheduler delete profile <name> |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4.2 |  |  | Description |  |  |  |  |  |  |
|  | This command deletes a single scheduler profile that was previously created using the scheduler add profile command. |  |  |  |  |  |  |  |  |  |  |
|  |  | 4.3 |  |  | Options |  |  |  |  |  |  |
| The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Option | Description | Default value |  |  |  |
|  |  |  |  |  |  | name | A name that identifies an existing profile. To display names, use the scheduler list Profiles command. | N/A |  |  |  |
|  |  |  | 4.4 | Example prompt> scheduler delete profile prioschlr |  |  |  |  |  |  |  |
|  |  |  | 4.5 | See also |  |  |  |  |  |  |  |
|  |  |  |  | scheduler add profile |  |  |  |  |  |  |  |
|  |  |  |  | scheduler list profiles |  |  |  |  |  |  |  |

| 5 | scheduler list profiles |  |  |
| --- | --- | --- | --- |
|  | 5.1 | Syntax |  |
|  | scheduler list profiles |  |  |
|  | 5.2 | Description |  |
|  |  | This command lists information about profiles previously created using the scheduler add profile command. The following information is displayed: |  |
|  |  | • profile ID number |  |
|  |  | • profile name |  |
| 5.3 |  | Example |  |
|  |  | prompt> scheduler list profiles |  |
|  |  | Scheduler Profiles: |  |
|  |  | ID ¦ Name |  |
|  |  | ----¦-------------- |  |
|  |  | 1 | ¦ sch1 |
|  |  | 2 | ¦ sch2 |
|  |  | ------------------- |  |
| 5.4 |  | See also |  |
|  |  | scheduler add profile |  |

| 6 | scheduler set profile shaping |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6.1 Syntax |  |  |  |  |  |  |
| scheduler set profile <name> shaping <MaxRate> <MaxBurst> |  |  |  |  |  |  |  |
|  | 6.2 Description |  |  |  |  |  |  |
|  |  | This command configures the specified scheduler profile to shape aggregate traffic to the specified maximum rate and burst size. |  |  |  |  |  |
|  |  | To disable shaping, set MaxRate and MaxBurst values to 0 and apply the profile to the transport using the transport set scheduler profile command. |  |  |  |  |  |
|  |  | If scheduler is applied over an Ethernet interface, the padding and FCS are not taken into account. When applied over other types of interfaces, the extra bytes added by devices lower than scheduler may not be taken into account. For example, when configured over an ATM interface, the padding and trailer added by AAL5 are not taken into account. |  |  |  |  |  |
|  | 6.3 Options |  |  |  |  |  |  |
|  |  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |
|  |  |  | Option | Description | Default value |  |  |
|  |  |  | name | A name that identifies an existing profile. To display names, use the scheduler list profiles command. | N/A |  |  |
|  |  |  | MaxRate | Maximum rate for outgoing traffic in kilobits per second (kbps). | N/A |  |  |
|  |  |  | MaxBurst | Maximum burst size value in bytes. This value should be greater than the MTU, otherwise the packets of greater size shall be dropped. | N/A |  |  |
|  | 6.4 |  | Example |  |  |  |  |
|  |  |  | prompt> scheduler set profile prioschlr shaping 2000 10000 |  |  |  |  |
|  | 6.5 |  | See also |  |  |  |  |
|  |  |  | scheduler list profiles |  |  |  |  |
|  |  |  | scheduler show profile |  |  |  |  |

| 7 | scheduler profile set queue weight |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 7.1 | Syntax |  |  |  |  |  |
| scheduler profile <name> set queue [1-7] weight <weight> |  |  |  |  |  |  |  |
|  | 7.2 | Description |  |  |  |  |  |
|  |  | This command configures a scheduler profile of wf2qplusservice discipline to configure a separate queue for a scheduling priority level with the specified weight value. The weight value is set as a percentage. The configured queue is identified by a queueid which is the same value as the scheduling priority value. |  |  |  |  |  |
|  |  | The weight value configured for a scheduling priority level determines the share of bandwidth received by packets belonging to the scheduling priority. A single queue with the weight set to 100% is configured by default when the profile is created using the scheduler add profile command. Packets with scheduling priority 0 are always enqueued in the default queue. You may configure weights for scheduling priority (1-7) using this command. If the weight for a scheduling priority level is configured, the corresponding packets are enqueued in a separate queue and the bandwidth is allocated in accordance with the configured weight. Otherwise, packets are enqueued in the default queue. |  |  |  |  |  |
|  |  | You cannot set the weight for the default queue directly. As you assign values to the other queues (1-7), the default queue weight is decreased by that amount. The sum total of weights of other queues cannot exceed 99%. |  |  |  |  |  |
|  | 7.3 | Options |  |  |  |  |  |
|  |  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |
|  |  |  | Option | Description | Default value |  |  |
|  |  |  | name | A name that identifies an existing profile. To display names, use the scheduler list Profiles command. | N/A |  |  |
|  |  |  | 1-7 | The scheduling priority value or queue ID value. This can be any number between 1 and 7 (inclusive). | N/A |  |  |
|  |  |  | weight | Weight value in percentage. You cannot set weight to more than 99 for a queue. In addition, the sum total of all weight values you configure cannot be more than 99. To display weight value for scheduler profile queues, use the scheduler profile show Queues command. | N/A |  |  |

| 7.4 | Example prompt> scheduler profile prioschlr set shaping 2000 10000 |
| --- | --- |
| 7.5 | See also |
|  | scheduler list profiles |
|  | scheduler show profile |

| 8 | scheduler profile set queue ald profile |  |  |
| --- | --- | --- | --- |
|  | 8.1 Syntax |  |  |
|  |  | scheduler profile <name> set queue {default ¦ [1-7]} ald profile <GreenProfile> [<YellowProfile> <Red Profile>] |  |
|  | 8.2 Description |  |  |
|  |  | This command associates a scheduler profile queue with an Algorithmic Dropper (ALD) profile. ALD is a congestion control mechanism. The dropper associated with a particular scheduler profile decided at the time of packet enqueuing whether the packet should be enqueued in the queue or dropped. The ALD profile is created using the ald add profile command. |  |
|  |  | Each configured queue can have one of the following ALD algorithms associated with it: |  |
|  |  | • RED (Random Early Detection); applied if only one (green) ALD profile is configured. |  |
|  |  | • WRED (Weighted RED); treats packets differently depending on the drop priorities assigned to packets by the classifier profile set rule droppri command, or assigned by meter on the basis of metering results. Note that meter always sets the drop priority in the packet. Green is the lowest drop priority (0), yellow is the medium drop priority (1) and red is the highest drop priority (2). The WRED algorithm is applied if three ALD profiles (green, yellow and red; one for each drop priority) are configured. |  |
|  |  |  | Note- If the scheduler profile uses the WF2Q+ service discipline, the weight of the queue must be set to a non-zero value before you can associate an ALD profile. See scheduler profile set queue weight . |

|  |  |  | 8.3 |  |  |  |  |  |  | Options |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Option |  | Description |  |  | Default value |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | name |  | A name that identifies an existing profile. To display names, use the scheduler list profiles command. |  |  | N/A |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | default |  | The default priority value. See the scheduler add profile command Description. |  |  | N/A |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 1-7 |  | The scheduling priority value or queue ID value (created using the scheduler profile set queue weight command) that you want to associate the ALD profile with. This can be any number between 1 and 7 (inclusive). |  |  | N/A |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | GreenProfile |  | A name that identifies an existing ALD profile. This is the only profile required if the RED algorithm is used. If you are using the WRED algorithm, this ALD profile is applied to packets with green drop priority. To display ALD profiles, use the ald list profiles command. |  |  | N/A |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | YellowProfile |  | A name that identifies an existing ALD profile. If you are using the WRED algorithm, this ALD profile is applied to packets with yellow drop priority. To display ALD profiles, use the ald list profiles command. |  |  | N/A |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | RedProfile |  | A name that identifies an existing ALD profile. If you are using the WRED algorithm, this ALD profile is applied to packets with red drop priority. To display ALD profiles, use the ald list profiles command. |  |  | N/A |  |  |  |
|  | 8.4 |  |  |  |  |  |  | Examples |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | prompt> scheduler profile prioschlr set queue 1 ald profile ald1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | prompt> scheduler profile prioschlr set queue 1 ald profile ald1 ald2 ald3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8.5 |  |  |  |  |  | See also |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | scheduler list profiles |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | scheduler profile set queue weight |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | meter set profile action drop |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | ALD CLI commands |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  | scheduler profile set queue ald disabled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 9.1 Syntax |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | scheduler profile <name> set queue {default ¦ [1-7]} ald disabled |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 9.2 Description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | This command disables the algorithmic dropper (ALD) on a specified scheduler profile queue. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 9.3 Options |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Option |  | Description |  |  | Default value |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | name |  | A name that identifies an existing profile. To display names, use the scheduler list profiles command. |  |  | N/A |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | default |  | The default priority value. See the scheduler add profile command Description. |  |  | N/A |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 1-7 |  | The scheduling priority value or queue ID value (created using the scheduler profile set queue weight command) that the ALD profile is associated with. This can be any number between 1 and 7 (inclusive). |  |  | N/A |  |  |  |  |
|  |  |  |  |  | 9.4 |  |  |  | Examples |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | prompt> scheduler profile prioschlr set queue 1 disabled |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 9.5 |  |  |  | See also |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | scheduler list profiles |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | scheduler profile set queue weight |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | scheduler profile set queue ald profile |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | meter set profile action drop |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | ALD CLI commands |  |  |  |  |  |  |  |  |  |  |

| 10 scheduler show profile |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10.1 |  | Syntax |  |  |  |  |  |  |  |  |
|  |  | scheduler show profile <name> |  |  |  |  |  |  |  |  |
| 10.2 |  | Description |  |  |  |  |  |  |  |  |
|  |  | This command displays the following information about an existing scheduler profile. |  |  |  |  |  |  |  |  |
|  |  | • Type of service discipline set |  |  |  |  |  |  |  |  |
|  |  | • Maximum Rate of traffic |  |  |  |  |  |  |  |  |
|  |  | • Maximum Burst Size (MBS) of traffic |  |  |  |  |  |  |  |  |
| 10.3 |  | Options |  |  |  |  |  |  |  |  |
|  |  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |  |  |  |
|  |  |  |  | Option | Description |  | Default value |  |  |  |
|  |  |  |  | name | A name that identifies an existing profile. To display names, use the scheduler list Profiles command. |  | N/A |  |  |  |
|  | 10.4 |  | Examples |  |  |  |  |  |  |  |
|  |  |  | prompt> scheduler show profile prioschlr |  |  |  |  |  |  |  |
|  |  |  | Type |  |  | : priority |  |  |  |  |
|  |  |  | MaxRate (kbps) |  |  | : 2000 |  |  |  |  |
|  |  |  | MaxBurst (bytes) : 10000 |  |  |  |  |  |  |  |
|  | 10.5 |  | See also |  |  |  |  |  |  |  |
|  |  |  | scheduler list profiles |  |  |  |  |  |  |  |
|  |  |  | scheduler add profile |  |  |  |  |  |  |  |
|  |  |  | scheduler set profile shaping |  |  |  |  |  |  |  |

| 11 scheduler show profile queues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11.1 |  | Syntax |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | scheduler show profile <name> queues |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.2 |  | Description |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | This command displays the following information about the queues of an existing scheduler profile. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | • Queue ID number (0-7) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | • Weight value (percentage) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | • |  |  | ALD Green Profile name |  |  |  |  |  |  |  |  |  |
|  |  | • |  |  | ALD Yellow Profile name |  |  |  |  |  |  |  |  |  |
|  |  | • |  |  | ALD Red Profile name |  |  |  |  |  |  |  |  |  |
| 11.3 |  | Options |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable). |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Option |  |  |  | Description |  |  | Default value |  |  |  |
|  |  |  |  | name |  |  |  | A name that identifies an existing profile. To display names, use the scheduler list Profiles command. |  |  | N/A |  |  |  |
|  | 11.4 |  | Examples |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | prompt> scheduler show profile prioschlr show queues |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Queues: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QueueId ¦ Weight ¦ ALD Green Profile ¦ ALD Yellow Profile ¦ ALD Red Profile |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | --------¦--------¦-------------------¦--------------------¦---------------- |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 |  |  |  |  | ¦ 0 | ¦ ald1 |  | ¦ | ¦ |  |  |  |  |
|  | 1 |  |  |  |  | ¦ 0 | ¦ ald1 |  | ¦ ald2 | ¦ ald3 |  |  |  |  |
|  | 2 |  |  |  |  | ¦ 0 | ¦ |  | ¦ | ¦ |  |  |  |  |
|  | 3 |  |  |  |  | ¦ 0 | ¦ |  | ¦ | ¦ |  |  |  |  |
|  | 4 |  |  |  |  | ¦ 0 | ¦ |  | ¦ | ¦ |  |  |  |  |
|  | 5 |  |  |  |  | ¦ 0 | ¦ |  | ¦ | ¦ |  |  |  |  |
|  | 6 |  |  |  |  | ¦ 0 | ¦ |  | ¦ | ¦ |  |  |  |  |
|  | 7 |  |  |  |  | ¦ 0 | ¦ |  | ¦ | ¦ |  |  |  |  |
|  |  |  | --------------------------------------------------------------------------- |  |  |  |  |  |  |  |  |  |  |  |
|  | 11.5 |  | See also |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | scheduler list profiles |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | scheduler add profile |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | scheduler profile set queue weight |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | scheduler profile set queue ald profile |  |  |  |  |  |  |  |  |  |  |  |