- RFC1483 CLI commands

This chapter describes the RFC1483 CLI commands.

Summary

    - RFC1483 CLI commands

The table below lists the RFC1483 commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| rfc1483 add transport | rfc1483 add transport |
| rfc1483 clear transports | rfc1483 clear transports |
| rfc1483 delete transport | rfc1483 delete transport |
| rfc1483 list transports | rfc1483 list transports |
| rfc1483 set transport bt | rfc1483 set transport bt |
| rfc1483 set transport mbs | rfc1483 set transport mbs |
| rfc1483 set transport mcr | rfc1483 set transport mcr |
| rfc1483 set transport mode | rfc1483 set transport mode |
| rfc1483 set transport pcr | rfc1483 set transport pcr |
| rfc1483 set transport port | rfc1483 set transport port |
| rfc1483 set transport prilevels | rfc1483 set transport prilevels |
| rfc1483 set transport qosclass | rfc1483 set transport qosclass |
| rfc1483 set transport rxvci | rfc1483 set transport rxvci |
| rfc1483 set transport rxvpi | rfc1483 set transport rxvpi |
| rfc1483 set transport scr | rfc1483 set transport scr |
| rfc1483 set transport txvci | rfc1483 set transport txvci |
| rfc1483 set transport txvpi | rfc1483 set transport txvpi |
| rfc1483 set transport vci | rfc1483 set transport vci |
| rfc1483 set transport vpi | rfc1483 set transport vpi |
| rfc1483 show transport | rfc1483 show transport |

    - RFC1483 Console commands

The above CLI commands replace all previous RFC1483 console commands.

rfc1483 add transport

    - Syntax

rfc1483 add transport <name> <port> <vpi> <vci> {llc|vcmux} {bridged|mer|routed}

    - Description

This command creates a named RFC1483 transport and allows you to specify the following:

- the ATM port that will transport RFC1483 data. (ATM ports are initialised in the initbun file in FlashFS, or using the bun set port console command.)

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

- LLC or VcMux encapsulation (optional)

- Bridged, MER or Routed (optional)

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the transport. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over. The VCI can be any value between 1 and 65535. | N/A |
| llc | Logical Link Control encapsulation method. | llc |
| vcmux | VC Multiplexing encapsulation method. |  |
| bridged | Traffic type that is going to be transmitted/received. | bridged |
| mer | Traffic type that is going to be transmitted/received. MER is denoted as ‘MAC Encapsulated Routing’ and only combined with llc encapsulation. |  |
| routed | Traffic type that is going to be transmitted/received. |  |

    - Example

prompt> rfc1483 add transport my1483 myport 0 700 vcmux routed

    - See also

rfc1483 list transports

port set

port ?

rfc1483 clear transports

    - Syntax

rfc1483 clear transports

    - Description

This command deletes all RFC1483 transports that were created using the rfc1483 add transport command.

    - Example

prompt> rfc1483 clear transports

    - See also

rfc1483 delete transport

rfc1483 delete transport

    - Syntax

rfc1483 delete transport {<name>|<number>}

    - Description

This command deletes a single RFC1483 transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |

    - Example

prompt> rfc1483 delete transport my1483

    - See also

rfc1483 list transports

rfc1483 list transports

    - Syntax

rfc1483 list transports

    - Description

This command lists all rfc1483 transports that have been created using the rfc1483 add transport command. It displays the following information about the transports:

- transport identification number

- transport name

- name of the ATM port used to transport rfc1483 data

- transmit and receive VCI numbers

- transmit and receive VPI numbers

    - Example

prompt> rfc1483 list transports

RFC1483 transports:

ID  |    Name    | Port      | TxVci     | RxVci     | TxVpi     | RxVpi

-----|------------|-----------|-----------|-----------|-----------|--------

1 | my1483     | a1        | 700       | 700       | 0         | 0

---------------------------------------------------------------------------

    - See also

rfc1483 show transport

rfc1483 set transport bt

    - Syntax

rfc1483 set transport {<name>|<number>} bt <burst tolerance>

    - Description

This command sets the burst tolerance (bt) that an existing RFC1483 transport uses to transport data over ATM. This command is only valid if you set VBR or VBR RT as the QoS Class using the rfc1483 set transport qosclass command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| burst tolerance | Controls the duration of traffic bursts on VBR (Variable Bit Rate) and VBR RT (VBR Real Time) channels. This value overrides an existing MBS value (if set). The BT can be any value between 0 and 100. | 0 |

    - Example

prompt> rfc1483 set transport my1483 bt 5

    - See also

rfc1483 set transport mbs

rfc1483 set transport qosclass

rfc1483 set transport mbs

    - Syntax

rfc1483 set transport {<name>|<number>} mbs <maximum burst size>

    - Description

This command sets the maximum burst size (mbs) for the RFC1483 transport. This command is only valid if you set VBR or VBR RT as the QoS Class using the rfc1483 set transport qosclass command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| maximum burst size | Controls the maximum burst size for VBR (Variable Bit Rate) and VBR RT (VBR Real Time) channels. This value overrides an existing BT value (if set). The MBS can be any value between 0 and 100. | 0 |

    - Example

prompt> rfc1483 set transport my1483 mbs 10

    - See also

rfc1483 set transport bt

rfc1483 set transport qosclass

rfc1483 set transport mcr

    - Syntax

rfc1483 set transport {<name>|<number>} mcr <minimum cell rate>

    - Description

This command sets the minimum cell rate (mcr) for an existing RFC1483 transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| minimum cell rate | Determines the minimum rate at which ATM cells are allowed to be transported into the ATM network. | 0 |

    - Example

prompt> rfc1483 set transport my1483 mcr 0

    - See also

rfc1483 set transport pcr

rfc1483 set transport mode

    - Syntax

rfc1483 set transport {<name>|<number>} mode {llc|vcmux} {bridged|mer|routed}

    - Description

This command sets the mode of encapsulation that an existing RFC1483 transport uses.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| llc | Logical Link Control encapsulation method. | llc |
| vcmux | VC Multiplexing encapsulation method. |  |
| bridged | Traffic type that is going to be transmitted/received. | bridged |
| mer | Traffic type that is going to be transmitted/received. MER is denoted as ‘MAC Encapsulated Routing’ and only combined with llc encapsulation. |  |
| routed | Traffic type that is going to be transmitted/received. |  |

    - Example

prompt> rfc1483 set transport my1483 mode vcmux bridged

    - See also

rfc1483 list transports

rfc1483 set transport pcr

    - Syntax

rfc1483 set transport {<name>|<number>} pcr <peak cell rate>

    - Description

This command sets the peak cell rate (pcr) for an existing RFC1483 transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| peak cell rate | Determines the maximum rate at which ATM cells are allowed to be transported into the ATM network. The PCR can be any value from 0 up to the maximum PortSpeed parameter set when the port was created (using the initbun file in FlashFS or the CLl command port set ). | 0 |

    - Example

prompt> rfc1483 set transport my1483 pcr 50000

    - See also

rfc1483 set transport mcr

rfc1483 list transports

rfc1483 set transport port

    - Syntax

rfc1483 set transport {<name>|<number>} port <atm-port>

    - Description

This command sets the port that an existing RFC1483 transport uses to transport RFC1483 data over ATM.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| atm-port | The system port that is used to transport ATM data. The port/VPI/VCI combination must be unique for each transport. | N/A |

    - Example

prompt> rfc1483 set transport my1483 port a1

    - See also

rfc1483 list transports

port set

rfc1483 set transport prilevels

    - Syntax

rfc1483 set transport {<name>|<number>} prilevels <prilevels>

    - Description

This command enables support for multiple packet priority levels on the same ATM VC. The current system supports two prilevel values:

- 1: there are no multiple priority levels enabled so the feature is disabled.

- 2: packets with different priorities set (such as best effort and high priority traffic) are prioritized on the same ATM VC.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| prilevels | The number of priority levels available on an ATM transport. | 1 |

    - Example

prompt> rfc1483 set transport my1483 prilevels 2

    - See also

rfc1483 list transports

port set

rfc1483 set transport qosclass

    - Syntax

rfc1483 set transport {<name>|<number>} qosclass {ubr|cbr|vbr|vbrrt|abr|qfc}

    - Description

This command sets the quality of service (QoS) class for the transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| ubr | Unspecified Bit Rate; non-constant and unpredictable data transport rate. PCR (Peak Cell Rate) is the average and maximum speed of transmission. | UBR |
| cbr | Constant Bit Rate; constant demand and predictable data transport rate. PCR is the average and maximum speed of transmission. |  |
| vbr | Variable Bit Rate; non-constant but predictable data transport rate that uses Non-Real-Time (NRT). You can specify the PCR, SCR, BT and MBS for VBR traffic. |  |
| vbrrt | Variable Bit Rate Real-Time; non- constant but predictable data transport rate that uses Real-Time (RT). You can specify the PCR, SCR, BT and MBS for VBRRT traffic. |  |
| abr | Available Bit Rate; non-constant and unpredictable data transport rate that provides ATM-layer feedback and flow control. |  |
| qfc | QFC; ATM flow control protocol that supports ABR. |  |

    - Example

prompt> rfc1483 set transport my1483 abr

    - See also

rfc1483 show transport

rfc1483 set transport bt

rfc1483 set transport mbs

rfc1483 set transport pcr

rfc1483 set transport scr

rfc1483 set transport rxvci

    - Syntax

rfc1483 set transport {<name>|<number>} rxvci <vci>

    - Description

This command sets the receive Virtual Circuit Identifier channel. If you later set the VCI using the rfc1483 set transport vci command, the RX VCI setting will be overridden.

The port/VCI/VPI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| rxvci | Part of the ATM header. The RXVCI is a tag that identifies which channel a cell will be received over. The RXVCI can be any value between 1 and 65535. | VCI value set when the transport was created using the rfc1483 add transport command |

    - Example

prompt> rfc1483 set transport my1483 rxvci 700

    - See also

rfc1483 list transports

rfc1483 set transport txvci

rfc1483 set transport rxvpi

rfc1483 set transport {<name>|<number>} rxvpi <vpi>

    - Description

This command sets the receive Virtual Path Identifier. If you later set the VPI using the rfc1483 set transport vpi command, the rxvpi setting will be overridden.

The port/VCI/VPI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| rxvpi | A field in the ATM header. The RXVPI is used to identify the virtual path that a circuit belongs to and receives information on. The RXVPI can be any value between 0 and 255. | VPI value set when the transport was created using the rfc1483 add transport command |

    - Example

prompt> rfc1483 set transport my1483 rxvpi 0

    - See also

rfc1483 list transports

rfc1483 set transport txvpi

rfc1483 set transport scr

    - Syntax

rfc1483 set transport {<name>|<number>} scr <sustainable cell rate>

    - Description

This command sets the Sustainable Cell Rate. This command is only valid if you set VBR or VBR RT as the QoS Class using the rfc1483 set transport qosclass command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| sustainable cell rate | Sustainable Cell Rate; the average cell rate for a VBR or VBR RT connection. The SCR can be any positive value that is less than the both PortSpeed set (when the port was created) and the PCR set for the channel. (The port is initialised using the initbun file in FlashFS or the CLI command port set ) | 0 |

    - Example

prompt> rfc1483 set transport my1483 scr 25000

    - See also

rfc1483 set transport qosclass

rfc1483 list transports

rfc1483 set transport txvci

    - Syntax

rfc1483 set transport {<name>|<number>} txvci <vci>

    - Description

This command sets the transmit Virtual Circuit Identifier channel. If you later set the VCI using the rfc1483 set transport vci command, the txvci setting will be overridden.

The port/VCI/VPI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| txvci | Part of the ATM header. The TX VCI is a tag that identifies which channel a cell will be transmitted over. The TX VCI can be any value between 1 and 65535. | VCI value set when the transport was created using the rfc1483 add transport command |

    - Example

prompt> rfc1483 set transport my1483 txvci 800

    - See also

rfc1483 list transports

rfc1483 set transport rxvci

rfc1483 set transport txvpi

    - Syntax

rfc1483 set transport {<name>|<number>} txvpi <vpi>

    - Description

This command sets the transmit Virtual Path Identifier. If you later set the VPI using the rfc1483 set transport vpi command, the rxvpi setting will be overridden.

The port/VCI/VPI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| txvpi | A field in the ATM header. The TX VPI is used to identify the virtual path that a circuit belongs to and transmits information on. The TX VPI can be any value between 0 and 255. | VPI value set when the transport was created using the rfc1483 add transport command |

    - Example

prompt> rfc1483 set transport my1483 txvpi 0

    - See also

rfc1483 list transports

rfc1483 set transport rxvpi

rfc1483 set transport vci

    - Syntax

rfc1483 set transport {<name>|<number>} vci <vci>

    - Description

This command sets the Virtual Circuit Identifier channel. It overrides existing VCI settings (including RX VCI and TX VCI).

The port/VCI/VPI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over. The VCI can be any value between 1 and 65535. | VCI value set when the transport was created using the add transport command |

    - Example

prompt> rfc1483 set transport my1483 vci 800

    - See also

rfc1483 list transports

rfc1483 set transport txvci

rfc1483 set transport rxvci

rfc1483 set transport vpi

    - Syntax

rfc1483 set transport {<name>|<number>} vpi <vpi>

    - Description

This command sets the Virtual Path Identifier. It overrides existing VPI settings (including RX VPI and TX VPI).

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | VPI value set when the transport was created using the add transport command |

    - Example

prompt> rfc1483 set transport my1483 vpi 0

    - See also

rfc1483 list transports

rfc1483 set transport rxvpi

rfc1483 set transport txvpi

rfc1483 show transport

    - Syntax

rfc1483 show transport {<name>|<number>}

    - Description

This command displays the following information about an existing rfc1483 transport:

- Name

- Description

- Encapsulation method

- ATM port

- TX VPI - transmit Virtual Path Identifier

- RX VPI - receive Virtual Path Identifier

- TX VCI - transmit Virtual Circuit Identifier

- TX VCI - receive Virtual Circuit Identifier

- QOS class - Quality of Service class

- PCR - Peak Cell Rate

- SCR - Sustainable Cell Rate

- MCR - Minimum Cell Rate

- BT - Burst Tolerance

- MBS - Maximum Burst Size

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing RFC1483 transport. To display transport names, use the rfc1483 list transports command. | N/A |
| number | A number that identifies an existing RFC1483 transport. To display transport numbers, use the rfc1483 list transports command. | N/A |

    - Example

prompt> rfc1483 show transport my1483

RFC1483 Transport: my1483

Description: Default LAN port

Encapsulation: LlcBridged

ATM port: a1

Tx VPI: 0

Rx VPI: 0

Tx VCI: 800

Rx VCI: 800

QOS class: UBR

Peak cell rate: 0         Burst tolerance: 0

Sustainable cell rate: 0         Max. burst size: 0

Minimum cell rate: 0

    - See also

rfc1483 list transports