- DHCP Server CLI commands

This chapter describes the DHCP server CLI commands.

Summary

    - DHCP server CLI commands

The table below lists the DHCP server commands provided by the CLI:

| Command | Reference |
| --- | --- |
| dhcpserver add interface | dhcpserver add interface |
| dhcpserver clear interfaces | dhcpserver clear interfaces |
| dhcpserver delete interface | dhcpserver delete interface |
| dhcpserver list interfaces | dhcpserver list interfaces |
| dhcpserver add fixedhost | dhcpserver add fixedhost |
| dhcpserver add subnet | dhcpserver add subnet |
| dhcpserver clear fixedhosts | dhcpserver clear fixedhosts |
| dhcpserver clear subnets | dhcpserver clear subnets |
| dhcpserver delete fixedhost | dhcpserver delete fixedhost |
| dhcpserver delete subnet | dhcpserver delete subnet |
| dhcpserver enable¦disable | dhcpserver enable¦disable |
| dhcpserver list fixedhosts | dhcpserver list fixedhosts |
| dhcpserver list options | dhcpserver list options |
| dhcpserver list subnets | dhcpserver list subnets |
| dhcpserver set allowunknownclients | dhcpserver set allowunknownclients |
| dhcpserver set bootp | dhcpserver set bootp |
| dhcpserver set defaultleasetime | dhcpserver set defaultleasetime |
| dhcpserver set fixedhost ipaddress | dhcpserver set fixedhost ipaddress |
| dhcpserver set fixedhost macaddress | dhcpserver set fixedhost macaddress |
| dhcpserver set fixedhost maxleasetime | dhcpserver set fixedhost maxleasetime |
| dhcpserver set maxleasetime | dhcpserver set maxleasetime |
| dhcpserver set subnet defaultleasetime | dhcpserver set subnet defaultleasetime |
| dhcpserver set subnet hostisdefaultgateway | dhcpserver set subnet hostisdefaultgateway |
| dhcpserver set subnet hostisdnsserver | dhcpserver set subnet hostisdnsserver |
| dhcpserver set subnet maxleasetime | dhcpserver set subnet maxleasetime |
| dhcpserver set subnet subnet | dhcpserver set subnet subnet |
| dhcpserver show | dhcpserver show |
| dhcpserver show subnet | dhcpserver show subnet |
| dhcpserver subnet add iprange | dhcpserver subnet add iprange |
| dhcpserver subnet add option | dhcpserver subnet add option |
| dhcpserver subnet clear ipranges | dhcpserver subnet clear ipranges |
| dhcpserver subnet clear options | dhcpserver subnet clear options |
| dhcpserver subnet delete iprange | dhcpserver subnet delete iprange |
| dhcpserver subnet delete option | dhcpserver subnet delete option |
| dhcpserver subnet list ipranges | dhcpserver subnet list ipranges |
| dhcpserver subnet list options | dhcpserver subnet list options |
| dhcpserver update | dhcpserver update |

    - DHCP server Console commands

The table below lists the dhcp server console commands and, if available, their equivalent CLI command:

| Command | CLI Equivalent |
| --- | --- |
| dhcpserver config | Replaced by CLI command dhcpserver show |
| dhcpserver help | Usable command, see help |
| dhcpserver pool | Usable command, see pool |
| dhcpserver reset | Blacklisted command, see reset |
| dhcpserver status | Usable command, see status |
| dhcpserver trace | Usable command, see trace |
| dhcpserver version | Usable command, see version |

dhcpserver add interface

    - Syntax

dhcpserver add interface <ipinterface>

    - Description

This command sets DHCP server to operate on a specific IP interface. The IP interface is defined as a DHCP server IP interface. By setting DHCP relay to operate on other interfaces, you can simultaneously use DHCP server and relay in your configuration.

Note – You cannot set both DHCP relay and DHCP server to operate on the same interface – you must use different interfaces for each..

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ipinterface | The name of the existing interface that you want DHCP server to operate on. To display interface names, use the ip list interfaces command. | N/A |

    - Example

prompt> dhcpserver add interface iplan

    - See also

dhcprelay add interface

ip list interfaces

dhcpserver clear interfaces

    - Syntax

dhcpserver clear interfaces

    - Description

This command deletes all DHCP server IP interfaces previously defined using the dhcpserver add interface command.

Note – This command does not delete the IP interfaces from the router.

    - Example

prompt> dhcpserver clear interfaces

    - See also

dhcpserver add interface

ip list interfaces

dhcpserver delete interface

    - Syntax

dhcpserver delete interface <ipinterface>

    - Description

This command deletes a single DHCP server IP interface previously defined using the dhcpserver add interface command.

Note – This command does not delete the IP interface from the router.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| ipinterface | The name of an existing IP interface that you want DHCP server to operate on. To display interface names, use the ip list interfaces command. | N/A |

    - Example

prompt> dhcpserver delete interface iplan

    - See also

dhcpserver add interface

dhcpserver list interfaces

dhcpserver list interfaces

    - Syntax

dhcpserver list interfaces

    - Description

This command lists the existing DHCP server IP interfaces previously defined using the dhcpserver add interface command. It displays the following information:

- DHCP server interface ID number

- IP interface name

Example

prompt> dhcpserver list interfaces

DHCP Server Interfaces:

ID  |    Name

-----|---------

1 |    lan

-----|---------

2 |    wan

---------------

    - See also

dhcpserver add interface

dhcpserver add fixedhost

    - Syntax

dhcpserver add fixedhost <name> <ipaddress> <macaddress>

    - Description

This command creates a new fixed host mapping in the DHCP server. This allows you to configure the DHCP server to assign a specific IP address to a specific DHCP client based on the client’s MAC address. If a DHCPDISCOVER or DHCPREQUEST is received from a DHCP client with a matching MAC address, it will have the specified fixed IP address assigned to it. You must also create a suitable DHCP subnet definition in order for fixed host mapping to work.

Note – You are not allowed to create a fixed host mapping with an IP address that is already present inside a configured, dynamic IP range on a subnet. The reverse is also forbidden; you cannot add addresses into a dynamic IP range that are already configured as fixed host addresses. The CLI will display a warning if you attempt to do this.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the fixed host mapping. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| ipaddress | The IP address that is assigned to a DHCP client based on the client’s MAC address, displayed in the following format: 192.168.102.3 | N/A |
| macaddress | A MAC address displayed in the following format: ##:##:##:##:##:## | N/A |

    - Example

The example below creates a fixed host mapping:

prompt> dhcpserver add fixedhost myhost 192.168.219.1 00:20:2b:01:02:03

The example below creates a suitable subnet for the above fixed host mapping. Note that the IP address used above is not present in the following IP range:

prompt> dhcpserver add subnet mysubnet 192.168.219.0 255.255.255.0 192.168.219.10 192.168.219.20

    - See also

dhcpserver delete fixedhost

dhcpserver list fixedhosts

dhcpserver add subnet

    - Syntax

dhcpserver add subnet <name> <ipaddress> <netmask> [<startaddr> <endaddr>]

    - Description

This command creates a subnet that stores a pool of IP addresses. The DHCP server can allocate IP addresses from this pool to clients on request.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the subnet. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| ipaddress | The IP address of the subnet, displayed in the following format: 192.168.102.3 | N/A |
| netmask | The netmask address of the subnet, for example: 255.255.255.0 | N/A |
| startaddr | The first IP address in the pool of addresses. The IP address is displayed in the following format: 192.168.102.3 | N/A |
| endaddr | The last IP address in the pool of addresses. The IP address is displayed in the following format: 192.168.102.3 | N/A |

    - Example

prompt>dhcpserver add subnet sub1 239.252.197.0 255.255.255.0 239.252.197.10 239.252.197.107

    - See also

dhcpserver list subnets

dhcpserver clear fixedhosts

    - Syntax

dhcpserver clear fixedhosts

    - Description

This command deletes all DHCP server fixedhosts that were created using the dhcpserver add fixedhost commands.

    - Example

prompt> dhcpserver clear fixedhosts

    - See also

dhcpserver add fixedhost

dhcpserver delete fixedhost

dhcpserver clear subnets

    - Syntax

dhcpserver clear subnets

    - Description

This command deletes all DHCP server subnets that were created using the dhcpserver add subnet commands.

    - Example

prompt> dhcpserver clear subnets

    - See also

dhcpserver delete subnet

dhcpserver delete fixedhost

    - Syntax

dhcpserver delete fixedhost <name>

    - Description

This command deletes a single fixed host mapping in the DHCP server that was created using the dhcpserver add fixedhost command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing fixed host. To display fixed host names, use the dhcpserver list fixedhosts command. | N/A |

    - Example

prompt> dhcpserver delete fixedhost myhost

    - See also

dhcpserver add fixedhost

dhcpserver clear fixedhosts

dhcpserver list fixedhosts

dhcpserver delete subnet

    - Syntax

dhcpserver delete subnet {<name>|<number>}

    - Description

This command deletes a single DHCP server subnet. The pool of IP addresses in the subnet are also deleted.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |

    - Example

prompt> dhcpserver delete subnet sub1

    - See also

dhcpserver clear subnets

dhcpserver enable|disable

    - Syntax

dhcpserver {enable|disable}

    - Description

This command enables/disables the DHCP server. You must have the DHCP server enabled in order to carry out any DHCP server configuration. If you try configuring DHCP server when dhcpserver enable is set, the CLI issues a warning message.

You cannot have DHCP server and DHCP relay enabled at the same time. If you try to configure DHCP server when DHCP relay is enabled, the CLI issues a warning message.

If you have set DHCP server to operate on an existing IP interface and you want to make configuration changes to that IP interface, must first disable DHCP server, then re-enable it once your IP configuration is complete.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enable | Enables configuration of the DHCP server. | enable |
| disable | Disables configuration of the DHCP server. |  |

    - Example

prompt> dhcpserver enable

    - See also

dhcprelay enable|disable

dhcpserver list fixedhosts

    - Syntax

dhcpserver list fixedhosts

    - Description

This command lists the following information about existing DHCP fixed host mappings:

- fixed host ID number

- fixed host name

- IP address

- MAC address

- Max lease time

    - Example

prompt> dhcpserver list fixedhosts

DHCP server fixed host mappings:

ID  |    Name |    IP address   |    MAC address     | Max Lease Time

-----|---------|-----------------|--------------------|---------------

1 | myhost  | 192.168.219.0   | 00:20:2b:01:02:03  | 86400

----------------------------------------------------------------------

    - See also

dhcpserver add fixedhost

dhcpserver set fixedhost ipaddress

dhcpserver set fixedhost macaddress

dhcpserver set fixedhost maxleasetime

dhcpserver list options

    - Syntax

dhcpserver list options

    - Description

This command lists the option data types available for DHCP server. These options are detailed in RFC2132.

You can configure the DHCP server using any of the options listed.

    - Example

prompt> dhcpserver list options

subnet-mask

time-offset

routers

time-servers

ien116-name-servers

domain-name-servers

log-servers

cookie-servers

lpr-servers

impress-servers

resource-location-servers

host-name

boot-size

merit-dump

domain-name

swap-server

root-path

extensions-path

ip-forwarding

non-local-source-routing

policy-filter

max-dgram-reassembly

default-ip-ttl

path-mtu-aging-timeout

path-mtu-plateau-table

interface-mtu

all-subnets-local

broadcast-address

perform-mask-discovery

mask-supplier

router-discovery

router-solicitation-address

static-routes

trailer-encapsulation

arp-cache-timeout

ieee802-3-encapsulation

default-tcp-ttl

tcp-keepalive-interval

tcp-keepalive-garbage

nis-domain

nis-servers

ntp-servers

vendor-encapsulated-options

netbios-name-servers

netbios-dd-server

netbios-node-type

netbios-scope

font-servers

x-display-manager

dhcp-requested-address

dhcp-lease-time

dhcp-option-overload

dhcp-message-type

dhcp-server-identifier

dhcp-parameter-request-list

dhcp-message

dhcp-max-message-size

dhcp-renewal-time

dhcp-rebinding-time

dhcp-class-identifier

dhcp-client-identifier

option-62

option-63

nisplus-domain

nisplus-servers

tftp-server-name

bootfile-name

mobile-ip-home-agent

smtp-server

pop-server

nntp-server

www-server

finger-server

irc-server

streettalk-server

streettalk-directory-assistance-server

user-class

option-78

option-79

option-80

option-81

option-82

option-83

option-84

nds-servers

nds-tree-name

nds-context

option-88

option-89

...(more options down to)

option-115

auto-configure

option-117

...(more options down to)

option-254

option-end

    - See also

dhcpserver subnet add option

For information on RFC 2132, see http://www.ietf.org/rfc/rfc2132.txt

dhcpserver list subnets

    - Syntax

dhcpserver list subnets

    - Description

This command lists the following information about existing DHCP server subnets:

- subnet number

- subnet name

- subnet IP address

- subnet netmask address

- default lease time (in seconds)

- maximum lease time (in seconds)

- whether the host is a DNS server (true or false)

    - Example

prompt> dhcpserver list subnets

DHCP Server subnets:

Default       Max       Host is

ID  |    IP Address    |     Netmask     | Lease time | Lease time | DNS svr

-----|------------------|-----------------|------------|------------|--------

1 | 192.168.102.0    | 255.255.255.0   | 43200      | 86400      | false

-----------------------------------------------------------------------------

    - See also

dhcpserver show subnet

dhcpserver set allowunknownclients

    - Syntax

dhcpserver set allowunknownclients {enabled|disabled}

    - Description

This command enables/disables the dynamic assignment of addresses to unknown clients.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | Allows IP addresses to be dynamically assigned to unknown clients. | enabled |
| disabled | Does not allow IP addresses to be dynamically assigned to unknown clients. |  |

    - Example

prompt> dhcpserver set allowunknownclients disabled

    - See also

dhcpclient set interfaceconfig clientid

dhcpserver set bootp

    - Syntax

dhcpserver set bootp {enabled|disabled}

    - Description

This command determines whether or not DHCP server can respond to BOOTP requests.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | DHCP server responds to BOOTP queries. | enabled |
| disabled | DHCP server does not respond to BOOTP queries. |  |

    - Example

prompt> dhcpserver set bootp disabled

dhcpserver set defaultleasetime

    - Syntax

dhcpserver set defaultleasetime <defaultleasetime>

    - Description

This command sets the global default lease time for DHCP server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| defaultleasetime | The default time (in seconds) that is assigned to a lease if the client requesting the lease does not ask for a specific expiry time. | 43200 |

    - Example

prompt> dhcpserver set defaultleasetime 50000

    - See also

dhcpserver set subnet maxleasetime

dhcpserver set fixedhost ipaddress

    - Syntax

dhcpserver set fixedhost <host name> ipaddress <ipaddress>

    - Description

This command sets the IP address that will be allocated to a DHCP client by the fixed host mapping.

Note – You are not allowed to create a fixed host mapping with an IP address that is already present inside a configured, dynamic IP range on a subnet. The reverse is also forbidden; you cannot add addresses into a dynamic IP range that are already configured as fixed host addresses. The CLI will display a warning if you attempt to do this.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| host name | A name that identifies an existing fixedhost. To display fixedhost names, use the dhcpserver list fixedhosts command. | N/A |
| ip address | The IP address that is assigned to a DHCP client based on the client’s MAC address, displayed in the following format: 192.168.102.3 | N/A |

    - Example

prompt> dhcpserver set fixedhost myhost ipaddress 192.168.219.2

    - See also

dhcpserver list fixedhosts

dhcpserver set fixedhost macaddress

dhcpserver set fixedhost macaddress

    - Syntax

dhcpserver set fixedhost <host name> macaddress <macaddress>

    - Description

This command sets the MAC address for an existing fixed host mapping.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| host name | A name that identifies an existing fixedhost. To display fixedhost names, use the dhcpserver list fixedhosts command. | N/A |
| mac address | A MAC address displayed in the following format: ##:##:##:##:##:## | N/A |

    - Example

prompt> dhcpserver set fixedhost myhost macaddress 00:20:2b:01:02:03

    - See also

dhcpserver list fixedhosts

dhcpserver set fixedhost ipaddress

dhcpserver set fixedhost maxleasetime

    - Syntax

dhcpserver set fixedhost <host name> maxleasetime <maxleasetime>

    - Description

This command sets the maximum lease time for an existing fixed host mapping.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| maxleasetime | The maximum time (in seconds) that is assigned to a lease if the client requesting the lease does not ask for a specific expiry time. | 86400 |

    - Example

prompt> dhcpserver set fixedhost myhost maxleasetime 90000

    - See also

dhcpserver list fixedhosts

dhcpserver set maxleasetime

    - Syntax

dhcpserver set maxleasetime <maxleasetime>

    - Description

This command sets the global maximum lease time for DHCP server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| maxleasetime | The maximum time (in seconds) that is assigned to a lease if the client requesting the lease does not ask for a specific expiry time. | 86400 |

    - Example

prompt> dhcpserver set maxleasetime 90000

    - See also

dhcpserver set defaultleasetime

dhcpserver set subnet defaultleasetime

    - Syntax

dhcpserver set subnet {<name>|<number>} defaultleasetime <defaultleasetime>

    - Description

This command sets the default lease time for an existing subnet. This command setting overrides the global default lease time setting for this particular subnet.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |
| defaultleasetime | The default time (in seconds) that a subnet assigns to a lease if the client requesting the lease does not ask for a specific expiry time. | 43200 |

    - Example

prompt> dhcpserver set subnet sub1 defaultleasetime 30000

    - See also

dhcpserver show subnet

dhcpserver set subnet hostisdefaultgateway

    - Syntax

dhcpserver set subnet <{<name>|<number>} hostisdefaultgateway {enabled | disabled}

    - Description

This command tells the DHCP server to give out its own host IP address as the default gateway address. This is useful when combined with DNS Relay.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |
| enabled | Allows DHCP server to give out its own host IP address as the default gateway address. | disabled |
| disabled | Disallows DHCP server from giving out its own host IP address as the default gateway address. |  |

    - Example

prompt> dhcpserver set subnet sub1 hostisdefaultgateway enabled

    - See also

dhcpserver set subnet hostisdnsserver

dhcpserver set subnet hostisdnsserver

    - Syntax

dhcpserver set subnet {<name>|<number>} hostisdnsserver {enabled | disabled}

    - Description

This command tells the DHCP server to give out its own host IP address as the DNS server address. This is useful when combined with DNS Relay.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |
| enabled | Allows DHCP server to give out its own host IP address as the DNS server address. | disabled |
| disabled | Disallows DHCP server from giving out its own host IP address as the DNS server address. |  |

    - Example

prompt> dhcpserver set subnet sub1 hostisdnsserver enabled

    - See also

dhcpserver list subnets

dhcpserver set subnet maxleasetime

    - Syntax

dhcpserver set subnet {<name>|<number>} maxleasetime <maxleasetime>

    - Description

This command sets the maximum lease time for an existing subnet. This command setting overrides the global maximum lease time setting for this particular subnet.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |
| maxleasetime | The maximum time (in seconds) that a subnet assigns to a lease if the client requesting the lease does not ask for a specific expiry time. | 86400 |

    - Example

prompt> dhcpserver set subnet sub1 maxleasetime 70000

    - See also

dhcpserver show subnet

dhcpserver set subnet subnet

    - Syntax

dhcpserver set subnet {<name>|<number>} subnet <ip address> <netmask>

    - Description

This command allows you to change the IP address and netmask used by an existing DHCP server subnet.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |
| ip address | The new IP address for the subnet, displayed in the following format: 192.168.102.3 | N/A |
| netmask | The new netmask address for the subnet, for example: 255.255.255.0 | N/A |

    - Example

prompt> dhcpserver set subnet sub1 subnet 239.252.197.0 255.255.255.0

    - See also

dhcpserver list subnets

dhcpserver show

    - Syntax

dhcpserver show

    - Description

This command displays the following global configuration information about the DHCP server:

- status of the server (enabled/disabled)

- global default lease time

- global maximum lease time

- bootp requests setting (enable/disable)

- allow unknown clients setting (enable/disable)

    - Example

prompt> dhcpserver show

Global DHCP Server Configuration:

Status: ENABLED

Default lease time: 43200 seconds

Max. lease time: 86400 seconds

Allow BOOTP requests: true

Allow unknown clients: true

    - See also

dhcpserver show subnet

dhcpserver show subnet

    - Syntax

dhcpserver show subnet {<name>|<number>}

    - Description

This command displays the following information about a subnet:

- subnet name

- subnet IP address

- subnet netmask

- subnet maximum lease time

- subnet default lease time

- subnet host is DNS server

- subnet host is default gateway

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |

    - Example

prompt> dhcpserver show subnet sub1

DHCP Server Subnet: sub1

Subnet: 192.168.103.0

Netmask: 255.255.255.0

Max. lease time: 70000 seconds

Default lease time: 30000 seconds

Host Is Dns Server: true

Host Is Default Gateway: true

    - See also

dhcpserver show

dhcpserver subnet add iprange

    - Syntax

dhcpserver subnet {<name>|<number>} add iprange <startaddr> <endaddr>

    - Description

This command adds a pool of IP addresses to an existing subnet. DHCP server can allocate IP addresses from this pool to clients on request.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |
| startaddr | The first IP address in the pool of addresses. The IP address is displayed in the following format: 192.168.102.3 | N/A |
| endaddr | The last IP address in the pool of addresses. The IP address is displayed in the following format: 192.168.102.3 | N/A |

    - Example

prompt> dhcpserver subnet sub1 add iprange 239.252.197.0 239.252.197.107

    - See also

dhcpserver add subnet

dhcpserver list subnets

dhcpserver subnet list ipranges

dhcpserver subnet add option

    - Syntax

dhcpserver subnet {<name>|<number>} add option <identifier> <value>

    - Description

This command allows you to configure the DHCP server using the options detailed in RFC2132. To display a list of available options, use the command dhcpserver list options .

The heading of each option in the list contains the option identifier and the required value (in italics) for that specific option. The following is an extract from the option list:

option auto-configure flag;

This option, based on RFC2563, controls whether or not the auto configuration of IP address is to be allowed for clients on this subnet.

It only applies in cases where the DHCP server is unwilling or unable to supply an IP address lease. In this case, if this option is set to 1, then the DHCP server will not intervene to prevent clients from using auto-configuration to determine an IP address. If this option is set to 0, the use of IP address auto-configuration on the network will be explicitly forbidden by the DHCP server.

If this option is not explicitly configured, then it will be assumed that auto-configuration is allowed on the network.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |
| identifier | A text string that identifies a DHCP server configuration option. | N/A |
| value | The value associated with the option identifier. | N/A |

    - Example

prompt> dhcpserver subnet sub1 add option auto-configure 1

    - See also

For a list of options that you can choose from, see dhcpserver list options .

For information on RFC 2132, see http://www.ietf.org/rfc/rfc2132.txt

dhcpclient set interfaceconfig autoip enabled|disabled

dhcpserver subnet clear ipranges

    - Syntax

dhcpserver subnet {<name>|<number>} clear ipranges

    - Description

This command deletes all of the IP ranges set for an existing subnet.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |

    - Example

prompt> dhcpserver subnet sub1 clear ipranges

    - See also

dhcpserver subnet list ipranges

dhcpserver subnet delete iprange

dhcpserver subnet clear options

    - Syntax

dhcpserver subnet {<name>|<number>} clear options

    - Description

This command deletes the options set for an existing subnet.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |

    - Example

prompt> dhcpserver subnet sub1 clear options

    - See also

dhcpserver add subnet

dhcpserver list subnets

dhcpserver subnet delete option

dhcpserver subnet delete iprange

    - Syntax

dhcpserver subnet {<name>|<number>} delete iprange <range-id>

    - Description

This command deletes a single IP range from an existing subnet.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |
| range-id | A number that identifies an IP range. To list the existing range-ids for a subnet, use the dhcpserver subnet list ipranges command. | N/A |

    - Example

prompt> dhcpserver subnet sub1 delete iprange 1

    - See also

dhcpserver list subnets

dhcpserver subnet list ipranges

dhcpserver subnet delete option

    - Syntax

dhcpserver subnet {<name>|<number>} delete option <option number>

    - Description

This command deletes a single option that was created using the dhcpserver subnet add option command.Once deleted, the option will no longer be given out by the DHCP server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |
| option number | A number that identifies an existing option. To list all existing options, use the dhcpserver subnet list options command. | N/A |

    - Example

prompt> dhcpserver subnet sub1 delete option 2

    - See also

dhcpserver add subnet

dhcpserver clear subnets

dhcpserver list subnets

dhcpserver subnet list options

dhcpserver subnet list ipranges

    - Syntax

dhcpserver subnet {<name>|<number>} list ipranges

    - Description

This command lists the IP range(s) for an existing subnet that has been added using the dhcpserver add subnet command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |

    - Example

prompt> dhcpserver subnet sub1 list ipranges

IP Ranges for subnet: sub1

ID  |  Start Address   |    End Address

-----|------------------|------------------

1 | 192.168.102.0    | 192.168.102.100

2 | 192.168.102.200  | 192.168.102.300

-------------------------------------------

    - See also

dhcpserver list subnets

dhcpserver add subnet

dhcpserver subnet list options

    - Syntax

dhcpserver subnet {<name>|<number>} list options

    - Description

This command lists the options for an existing subnet that has been added using the dhcpserver add subnet command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing subnet. To display subnet names, use the dhcpserver list subnets command. | N/A |
| number | A number that identifies an existing subnet. To display subnet numbers, use the dhcpserver list subnets command. | N/A |

    - Example

prompt> dhcpserver subnet sub1 list options

Options for subnet: sub1

ID  |    Identifier    |       Value

-----|------------------|------------------

1 | ip-forwarding    | false

2 | subnet-mask      | 255.255.255.0

-------------------------------------------

    - See also

dhcpserver add subnet

dhcpserver list subnets

dhcpserver update

    - Syntax

dhcpserver update

    - Description

This command updates the DHCP server configuration. Changes made to the server configuration will not take effect until this command has been entered.

    - Example

prompt> dhcpserver update

dhcpserver: Reset request acknowledged. Reset imminent.