- PPPoE CLI commands

This chapter describes the PPP over Ethernet CLI commands.

Summary

    - PPPoE CLI commands

The table below lists the PPPoE commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| pppoe add transport dialout pvc | pppoe add transport dialout pvc |
| pppoe add transport dialout eth | pppoe add transport dialout eth |
| pppoe clear transports | pppoe clear transports |
| pppoe delete transport | pppoe delete transport |
| pppoe list transports | pppoe list transports |
| pppoe set transport enabled¦disabled | pppoe set transport enabled¦disabled |
| pppoe set transport atmaddress | pppoe set transport atmaddress |
| pppoe set transport autoconnect enabled¦disabled | pppoe set transport autoconnect |
| pppoe set transport bt | pppoe set transport bt |
| pppoe set transport createroute | pppoe set transport createroute |
| pppoe set transport discoverdns primary | pppoe set transport discoverdns primary |
| pppoe set transport discoverdns secondary | pppoe set transport discoverdns secondary |
| pppoe set transport discovernbns primary | pppoe set transport discovernbns primary |
| pppoe set transport discovernbns secondary | pppoe set transport discovernbns secondary |
| pppoe set transport discoversubnetmask | pppoe set transport discoversubnetmask |
| pppoe set transport eth | pppoe set transport eth |
| pppoe set transport enabled¦disabled | pppoe set transport enabled¦disabled |
| pppoe set transport givedns client | pppoe set transport givedns client enabled¦disabled |
| pppoe set transport givedns relay | pppoe set transport givedns relay enabled¦disabled |
| pppoe set transport givesubnetmask dhcpserver | pppoe set transport givesubnetmask dhcpserver enabled¦disabled |
| pppoe set transport headers hdlc | pppoe set transport headers hdlc |
| pppoe set transport hostunique | pppoe set transport hostunique |
| pppoe set transport headers llc | pppoe set transport headers llc |
| pppoe set transport interface | pppoe set transport interface |
| pppoe set transport idletimeout | pppoe set transport idletimeout |
| pppoe set transport lcpechoevery | pppoe set transport lcpechoevery |
| pppoe set transport lcpmaxconf | pppoe set transport lcpmaxconf |
| pppoe set transport lcpmaxfail | pppoe set transport lcpmaxfail |
| pppoe set transport lcpmaxterm | pppoe set transport lcpmaxterm |
| pppoe set transport localip | pppoe set transport localip |
| pppoe set transport mbs | pppoe set transport mbs |
| pppoe set transport mcr | pppoe set transport mcr |
| pppoe set transport password | pppoe set transport password |
| pppoe set transport pcr | pppoe set transport pcr |
| pppoe set transport port | pppoe set transport port |
| pppoe set transport qosclass | pppoe set transport qosclass |
| pppoe set transport remotedns | pppoe set transport remotedns |
| pppoe set transport remoteip | pppoe set transport remoteip |
| pppoe set transport routemask | pppoe set transport routemask |
| pppoe set transport scr | pppoe set transport scr |
| pppoe set transport specificroute | pppoe set transport specificroute |
| pppoe set transport subnetmask | pppoe set transport subnetmask |
| pppoe set transport servicename | pppoe set transport servicename |
| pppoe set transport theylogin | pppoe set transport theylogin |
| pppoe set transport username | pppoe set transport username |
| pppoe set transport vci | pppoe set transport vci |
| pppoe set transport vpi | pppoe set transport vpi |
| pppoe set transport welogin | pppoe set transport welogin |
| pppoe show transport | pppoe show transport |

    - PPP Console commands

The table below lists the ppp console commands and, if available, their equivalent CLI command:

| Command | CLI Equivalent |
| --- | --- |
| ppp <channel> clear | Blacklisted command, see <channel> clear |
| ppp <channel> disable | Blacklisted command, see <channel> disable |
| ppp <channel> discard | Usable command, see <channel> discard |
| ppp <channel> echo | Usable command, see <channel> echo |
| ppp <channel> echo every | Usable command, see <channel> echo every |
| ppp <channel> enable | Blacklisted command, see <channel> enable |
| ppp <channel> event | Usable command, see <channel> event |
| ppp <channel> hdlc | Replaced by CLI command pppoe set transport headers hdlc, using the hdlc option. |
| ppp <channel> info | Replaced by CLI command pppoe show transport |
| ppp <channel> interface | Replaced by CLI command pppoe set transport interface |
| ppp <channel> lcpmaxconfigure | Replaced by CLI command pppoe set transport lcpmaxconf |
| ppp <channel> lcpmaxfailure | Replaced by CLI command pppoe set transport lcpmaxfail |
| ppp <channel> lcpmaxterminate | Replaced by CLI command pppoe set transport lcpmaxterm |
| ppp <channel> llc | Replaced by CLI command pppoe set transport headers llc, using the llc option. |
| ppp <channel> qos | Replaced by CLI command pppoe set transport qosclass, pppoe set transport pcr, pppoe set transport bt |
| ppp <channel> remoteip | Replaced by CLI command pppoe set transport remoteip |
| ppp <channel> theylogin | Replaced by CLI command pppoe set transport theylogin |
| ppp <channel> tunnel | Replaced by CLI command pptp attach |
| ppp <channel> welogin | Replaced by CLI command pppoe set transport welogin |
| ppp bcp | Replaced by CLI command pppoe set transport headers llc , using the hdlc option |
| ppp interface - localip | Replaced by CLI command pppoe set transport localip |
| ppp interface - stats | Replaced by CLI command pppoe show transport |
| ppp user | Blacklisted command, see user |
| ppp version | Replaced by CLI command pppoe show transport |

pppoe add transport dialout pvc

    - Syntax

pppoe add transport <name> dialout pvc <interface> <port> <vpi> <vci> [accessconcentrator <concentrator>] [servicename <servicename>] [hostunique <hostunique>]

    - Description

This command creates a PPPoE transport that performs dialout over a PVC (Permanent Virtual Circuit). It allows you to specify the following PVC information:

- the PPP interface to the channel that the PVC will use

- the ATM port that will transport data

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

- access concentrator (optional)

- service name (optional)

- host unique tag (0x0103) in RFC2516 Appendix A

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the transport. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interface | The PPP interface to a channel that transports PPPoE data. A single interface can be used by multiple channels. The interface value can be any positive integer. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over.The VCI can be any value between 1 and 65535. | N/A |
| concentrator | A PPPoE tag that identifies a remote access concentrator (or PPPoE server). PPPoE will only connect to the named access concentrator. If no concentrator tag is set, PPPoE connects to the first access concentrator that responds. The tag name/number is determined by your ISP. | N/A |
| service name | A PPPoE tag that identifies a specific service that is acceptable to the PPPoE client. If set, the PPPoE transport will connect to the first access concentrator it finds that uses this service. If an access concentrator is also set, the PPPoE transport will connect to the specified service on the named concentrator. The service name is determined by your ISP. | N/A |
| hostunique | The hostunique tag is a series of hex digits representing an 8-bit octet that is to be included in the tag. The digits must be a multiple of 2, and can be in the range 0x00 - 0xFF.The actual representation of the host unique tag appears to be a string for configuration, but will be converted internally into its binary representation upon the initialization of sending the tag in the packet. | N/A |

    - Example

prompt> pppoe add transport pppoe1 dialout pvc 1 a1 0 800 accessconcentrator server32 servicename mercury hostunique FF44556677A0

    - See also

pppoe list transports

To list ATM ports, see port ? .

For more information on host unique tags, see http://www.ietf.org/rfc/rfc2516.txt?number=2516

pppoe add transport dialout eth

    - Syntax

pppoe add transport <name> dialout eth <interface> <port> [accessconcentrator <concentrator>] [servicename <servicename>] [hostunique <hostunique>]

    - Description

This command creates a PPPoE transport that performs dialout over Ethernet. This command is commonly used in Ethernet-based Broadband routers to WAN interface. It allows you to specify the following Ethernet information:

- the PPP interface to the channel that Ethernet will use

- the Ethernet port that will transport data

- access concentrator (optional)

- service name (optional)

- host unique tag (0x0103) in RFC2516 Appendix A

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the transport. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| interface | The PPP interface to a channel that transports PPPoE data. A single interface can be used by multiple channels. The interface value can be any positive integer. | N/A |
| port | The system port that is used to transport Ethernet data. To display Ethernet ports, use the ethernet list ports command. | N/A |
| concentrator | A PPPoE tag that identifies a remote access concentrator (or PPPoE server). PPPoE will only connect to the named access concentrator. If no concentrator tag is set, PPPoE connects to the first access concentrator that responds. The tag name/number is determined by your ISP. | N/A |
| service name | A PPPoE tag that identifies a specific service that is acceptable to the PPPoE client. If set, the PPPoE transport will connect to the first access concentrator it finds that uses this service. If an access concentrator is also set, the PPPoE transport will connect to the specified service on the named concentrator. The service name is determined by your ISP. | N/A |
| hostunique | The hostunique tag is a series of hex digits representing an 8-bit octet that is to be included in the tag. The digits must be a multiple of 2, and can be in the range 0x00 - 0xFF.The actual representation of the host unique tag appears to be a string for configuration, but will be converted internally into its binary representation upon the initialization of sending the tag in the packet. | N/A |

    - Example

prompt> pppoe add transport pppoe1 dialout eth 1 ethernet accessconcentrator server40 servicename virginia hostunique FF44556677A0

    - See also

pppoe list transports

ethernet list ports

For more information on host unique tags, see http://www.ietf.org/rfc/rfc2516.txt?number=2516

pppoe clear transports

    - Syntax

pppoe clear transports

    - Description

This command deletes all PPPoE transports that were created using the pppoe add transport commands.

    - Example

prompt> pppoe clear transports

    - See also

pppoe delete transport

pppoe delete transport

    - Syntax

pppoe delete transport {<name>|<number>}

    - Description

This command deletes a single PPPoE transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |

    - Example

prompt> pppoe delete transport pppoe1

    - See also

pppoe list transports

pppoe list transports

    - Syntax

pppoe list transports

    - Description

This command lists PPPoE transports that have been created using the pppoe add transport command. It displays the following information about the transports:

- transport identification number

- transport name

- Name of port used

- Virtual Circuit Identifier (VCI) used (PVC transports only)

- Virtual Path Identifier (VPI) used (PVC transports only)

    - Example

prompt> pppoe list transports

PPPoE transports:

ID  |    Name    | Port      | Vci       | Vpi

-----|------------|-----------|-----------|-----------

1 | p3         | realtek   | N/A       | N/A

2 | p2         | a1        | 800       | 0

3 | p1         | ethernet0 | N/A       | N/A

------------------------------------------------------

    - See also

pppoe show transport

pppoe set transport accessconcentrator

    - Syntax

pppoe set transport {<name>|<number>} accessconcentrator <concentrator>

    - Description

This command specifies the access concentrator that you want PPPoE to connect to.

You can also specify a service name using the set transport servicename command so that PPPoE will only accept a specific service via a specific access concentrator.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| concentrator | A PPPoE tag that identifies a remote access concentrator (or PPPoE server). PPPoE will only connect to the named access concentrator. If no concentrator tag is set, PPPoE connects to the first access concentrator that responds. The tag name/number is determined by your ISP. | Empty string |

    - Example

prompt> pppoe set transport pppoe1 accessconcentrator server5

    - See also

pppoe list transports

pppoe set transport servicename

pppoe show transport

For more information on PPPoE and access concentrators, see RFC2516; http://www.ietf.org/rfc/rfc2516.txt.

pppoe set transport autoconnect

    - Syntax

pppoe set transport {<name>|<number>} autoconnect {enabled|disabled}

    - Description

This command allows you to enable/disable the PPPoE autoconnect function. If enabled, PPPoE automatically connects to TCP/IP whenever a user requests TCP/IP packets from a public destination.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | Enables PPPoE autoconnect. | disabled |
| disabled | Disables PPPoE autoconnect. |  |

    - Example

prompt> pppoe set transport pppoe1 autoconnect enabled

    - See also

pppoe list transports

pppoe set transport bt

    - Syntax

pppoe set transport {<name>|<number>} bt <burst tolerance>

    - Description

This command sets the burst tolerance (bt) for an existing PPPoE transport. This command is only valid if you set VBR or VBR RT as the QoS Class using the pppoe set transport qosclass command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| burst tolerance | Controls the duration of traffic bursts on VBR (Variable Bit Rate) and VBR RT (VBR Real Time) channels. This value overrides an existing MBS value (if set). The BT can be any value between 0 and 100. | 0 |

    - Example

prompt> pppoe set transport pppoe1 bt 5

    - See also

pppoe set transport mbs

pppoe set transport createroute

    - Syntax

pppoe set transport {<name>|<number>} createroute {enabled|disabled}

    - Description

This command specifies whether a route is added to the system after IPCP (Internet Protocol Control Protocol) negotiation is completed. If set to enabled, a route will be created which directs packets to the remote end of the PPP link.

This route can either be a default route or a specific route, depending on the value set using the pppoe set transport specificroute command.

To display the createroute setting, use the pppoe show transport command. The route is removed when the PPP link is disconnected.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | Adds a route to the system after IPCP negotiation. | enabled |
| disabled | Does not add a route to the system after IPCP negotiation. |  |

    - Example

prompt> pppoe set transport pppoe1 createroute disabled

    - See also

pppoe show transport

pppoe set transport specificroute

ip list routes

pppoe set transport discoverdns primary

    - Syntax

pppoe set transport {<name>|<number>} discoverdns primary {enabled|disabled}

    - Description

Note - You must enable one of the pppoe set transport givedns commands in order for this command setting to work. See pppoe set transport givedns client enabled|disabled, or pppoe set transport givedns relay enabled|disabled

This command enables/disables whether the primary DNS server address is requested from a remote PPP peer using IPCP. The default setting for this command is enabled. The default setting for the pppoe set transport givedns commands is also enabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | A primary DNS server IP address is requested. | enabled |
| disabled | A primary DNS server IP address is not requested. |  |

    - Example

prompt> pppoe set transport pppoe3 discoverdns primary enabled

    - See also

pppoe set transport discoverdns secondary

pppoe set transport givedns client enabled|disabled

pppoe set transport givedns relay enabled|disabled

pppoe set transport remotedns

pppoe set transport discoverdns secondary

    - Syntax

pppoe set transport {<name>|<number>} discoverdns secondary {enabled|disabled}

    - Description

Note - You must enable one of the pppoe set transport givedns commands in order for this command setting to work. See pppoe set transport givedns client enabled|disabled, or pppoe set transport givedns relay enabled|disabled

This command enables/disables whether the secondary DNS server address is requested from a remote PPP peer using IPCP. The default setting for this command is enabled. The default setting for the pppoe set transport givedns commands is also enabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | A secondary DNS server IP address is requested. | enabled |
| disabled | A secondary DNS server IP address is not requested. |  |

    - Example

prompt> pppoe set transport pppoe3 discoverdns secondary enabled

    - See also

pppoe set transport discoverdns primary

pppoe set transport givedns client enabled|disabled

pppoe set transport givedns relay enabled|disabled

pppoe set transport remotedns

pppoe set transport discovernbns primary

    - Syntax

pppoe set transport {<name>|<number>} discovernbns primary {enabled|disabled}

    - Description

This command enables/disables whether the primary NBNS (NetBIOS Name Server) server address is requested from a remote PPP peer using IPCP. The default setting for this command is disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | A primary NBNS server IP address is requested. | disabled |
| disabled | A primary NBNS server IP address is not requested. |  |

    - Example

prompt> pppoe set transport pppoe3 discovernbns primary enabled

    - See also

pppoe set transport discovernbns secondary

pppoe set transport discovernbns secondary

    - Syntax

pppoe set transport {<name>|<number>} discovernbns secondary {enabled|disabled}

    - Description

This command enables/disables whether the secondary NBNS server address is requested from a remote PPP peer using IPCP. The default setting for this command is disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | A secondary NBNS server IP address is requested. | disabled |
| disabled | A secondary NBNS server IP address is not requested. |  |

    - Example

prompt> pppoe set transport pppoe3 discovernbns secondary enabled

    - See also

pppoe set transport discovernbns primary

pppoe set transport discoversubnetmask

    - Syntax

pppoe set transport {<name>|<number>} discoversubnetmask {enabled|disabled}

    - Description

This command enables/disables whether the netmask option is requested from a remote PPP peer using IPCP. The default setting for this command is disabled. Some of PPP peer can assign an IP pool instead of a single IP address.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | A netmask option is requested. | disabled |
| disabled | A netmask option is not requested. |  |

    - Example

prompt> pppoe set transport pppoe3 discoversubnetmask enabled

    - See also

pppoe set transport discoverdns primary

pppoe set transport eth

    - Syntax

pppoe set transport {<name>|<number>} eth <port>

    - Description

This command sets the ethernet port that an existing PPPoE transport uses to transport PPPoE data.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| port | The system port that is used to transport Ethernet data. To display Ethernet ports, use the ethernet list ports command. | N/A |

    - Example

prompt> pppoe set transport pppoe3 eth ethernet0

    - See also

pppoe list transports

ethernet list ports

pppoe set transport enabled|disabled

    - Syntax

pppoe set transport {<name>|<number>} {enabled|disabled}

    - Description

This command explicitly enables/disables a PPPoE transport. Attaching a transport to an interface implicitly enables it, but for cases where no attach is performed (for example, multiple channels on an interface, a PPP session that is not attached but needed for testing purposes) the transport must be enabled explicitly.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value for each option (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | Enables a PPPoE transport. | disabled |
| disabled | Disables a PPPoE transport. |  |

    - Example

prompt> pppoe set transport pppoe1 enabled

    - See also

pppoe list transports

pppoe set transport givedns client enabled|disabled

    - Syntax

pppoe set transport {<name>|<number>} givedns client {enabled | disabled}

    - Description

This command controls whether the PPP Internet Protocol Control Protocol (IPCP) can request a DNS server IP address for a remote PPP peer. Once IPCP has discovered the DNS server IP address, it automatically gives the address to the local DNS client so that a connection can be established.

You must have the DNS client process included in your image build in order to use this protocol.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | IPCP can request a DNS server IP address and then give the address to DNS client. | enabled |
| disabled | IPCP cannot request a DNS server IP address and then give the address to DNS client. |  |

    - Example

prompt> pppoe set transport pppoe1 givedns client enabled

    - See also

pppoe set transport givedns relay enabled|disabled

pppoe set transport remotedns

pppoe set transport discoverdns primary

pppoe set transport discoverdns secondary

For information on DNS implementation and specification, see http://www.ietf.org/rfc/rfc1035.txt.

pppoe set transport givedns relay enabled|disabled

    - See also

    - Syntax

pppoe set transport {<name>|<number>} givedns relay {enabled | disabled}

    - Description

This command controls whether the PPP Internet Protocol Control Protocol (IPCP) can request the DNS server IP address for a remote PPP peer. Once IPCP has discovered the DNS server IP address, it automatically gives the address to the local DNS relay so that a connection can be established.

You must have the DNS relay process included in your image build in order to use this protocol.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | IPCP can request a DNS server IP address and then give the address to DNS relay. | enabled |
| disabled | IPCP cannot request a DNS server IP address and then give the address to DNS relay. |  |

    - Example

prompt> pppoe set transport pppoe1 givedns relay enabled

    - See also

pppoe set transport givedns client enabled|disabled

pppoe set transport discoverdns primary

pppoe set transport discoverdns secondary

pppoe set transport remotedns

DNS Relay CLI commands

For information on DNS implementation and specification, see http://www.ietf.org/rfc/rfc1035.txt.

pppoe set transport givesubnetmask dhcpserver enabled|disabled

    - Syntax

pppoe set transport {<name>|<number>} givesubnetmask dhcpserver {enabled | disabled}

    - Description

This command controls once IPCP has discovered the netmask option from PPP peer, it automatically gives the netmask to the local DHCP server so that the DHCP server creates a new IP pools for leasing.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | Give the netmask to DHCP server. | disabled |
| disabled | Cannot give the address to DHCP server. |  |

    - Example

prompt> pppoe set transport pppoe1 givesubnetmask dhcpserver enabled

    - See also

pppoe set transport discoversubnetmask enabled|disabled

pppoe set transport headers hdlc

    - Syntax

pppoe set transport {<name>|<number>} headers hdlc {enabled|disabled}

    - Description

This command allows you to enable/disable whether your system can transmit and receive packets containing HDLC headers. If you want LLC packets to be transmitted and received instead of/as well as HDLC packets, use the pppoe set transport headers llc enabled command.

When both HDLC and LLC headers are disabled, the default encapsulation method is VC multiplexed (VC Mux). PPP determines which format to use to transmit/receive packets by ‘learning’ the format information from incoming packet headers.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | Packets that have HDLC headers can be transmitted/received. | disabled (if LLC headers are disabled too, the default value is VC Mux) |
| disabled | Packets that have HDLC headers can not be transmitted/received. |  |

    - Example

prompt> pppoe set transport pppoe1 headers hdlc enabled

    - See also

pppoe list transports

pppoe show transport

pppoe set transport headers llc

pppoe set transport hostunique

    - Syntax

pppoe set transport {<name>|<number>} hostunique <hostunique>

    - Description

This command allows you to configure the PPPoE Host Unique Tag (0x0103) documented in RFC2516 Appendix A.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| hostunique | The hostunique tag is a series of hex digits representing an 8-bit octet that is to be included in the tag. The digits must be a multiple of 2, and can be in the range 0x00 - 0xFF.The actual representation of the host unique tag appears to be a string for configuration, but will be converted internally into its binary representation upon the initialization of sending the tag in the packet. | N/A |

    - Example

prompt> pppoe set transport pppoe1 hostunique FF44556677A0

This will send 0xFF 0x44 0x55 0x66 0x77 0xA0 bytes in the tag for the next PADI and PADR packets.

    - See also

pppoe list transports

pppoe show transport

For more information on host unique tags, see http://www.ietf.org/rfc/rfc2516.txt?number=2516

pppoe set transport headers llc

    - Syntax

pppoe set transport {<name>|<number>} headers llc {enabled|disabled}

    - Description

This command allows you to enable/disable whether your system can transmit and receive packets containing LLC headers. If you want HDLC packets to be transmitted and received instead of/as well as LLC packets, use the pppoe set transport headers hdlc enabled command.

When both LLC and HDLC headers are disabled, the default encapsulation method is VC multiplexed (VC Mux). PPP determines which format to use to transmit/receive packets by ‘learning’ the format information from incoming packet headers.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | Packets that have LLC headers can be transmitted/received. | disabled (if HDLC headers are disabled too, the default value is VC Mux) |
| disabled | Packets that have LLC headers can not be transmitted/received. |  |

    - Example

prompt> pppoe set transport pppoe1 headers llc enabled

    - See also

pppoe list transports

pppoe show transport

pppoe set transport headers llc

pppoe set transport idletimeout

    - Syntax

pppoe set transport {<name>|<number>} idletimeout <idletimeout>

    - Description

This command allows you to set an ‘idle’ time out for your LAN connection. If you are connected to an ISP via PPPoE but fail to send a request for data within a specified time limit, the PPPoE session is disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| idletimeout | The length of time ( in minutes) that a PPPoE session connected to an ISP can remain idle before the session is disabled. The time can be any value between 0 and 60. A value of 0 means that no idletimeout is set. | 0 |

    - Example

prompt> pppoe set transport pppoe1 idletimeout 20

    - See also

pppoe list transports

pppoe set transport lcpechoevery

pppoe set transport interface

    - Syntax

pppoe set transport {<name>|<number>} interface <interface>

    - Description

This command sets the PPP interface for an existing PPPoE transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| interface | The PPP interface to a channel that transports PPPoE data. A single interface can be used by multiple channels. The interface value can be any positive integer. | N/A |

    - Example

prompt> pppoe set transport pppoe2 interface 4

    - See also

pppoe show transport

pppoe list transports

pppoe set transport lcpechoevery

    - Syntax

pppoe set transport {<name>|<number>} lcpechoevery <interval>

    - Description

This command tells a specified PPP transport to send an LCP (Link Control Protocol) echo request frame at specified intervals (in seconds). If no reply to the request is received, the PPP connection is torn down. This functionality is also known as ‘keep-alive’.

If you do not want to send LCP echo frames, specify zero (0) in the <interval> attribute.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| interval | The length of time (in seconds) between LCP echo request frames being sent. If you do not want echo request frames to be sent, specify ‘0’ as the interval. | 10 seconds |

    - Example

prompt> pppoe set transport pppoe2 lcpechoevery 0

    - See also

pppoe show transport

pppoe list transports

pppoe set transport lcpmaxconf

    - Syntax

pppoe set transport {<name>|<number>} lcpmaxconf <lcp max configure>

    - Description

This command sets the Link Control Protocol (LCP) maximum configure number for an existing PPPoE transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| lcp max configure | Link Control Protocol; the maximum number of configures that can be transmitted without reply before assuming that the destination address is unable to respond. The LCPmaxconf can be any positive value. | 10 |

    - Example

prompt> pppoe set transport pppoe1 lcpmaxconf 20

    - See also

pppoe show transport

pppoe list transports

pppoe set transport lcpmaxfail

    - Syntax

pppoe set transport {<name>|<number>} lcpmaxfail <lcp max fail>

    - Description

This command sets the Link Control Protocol (LCP) maximum fail number for an existing PPPoE transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| lcp max fail | Link Control Protocol; the maximum number of consecutive negative acknowledgements (indicating that the information received contains errors) that can be transmitted before assuming that parameter negotiation is not converging. The LCPmaxfail can be any positive value. | 5 |

    - Example

prompt> pppoe set transport pppoe1 lcpmaxfail 20

    - See also

pppoe show transport

pppoe list transports

pppoe set transport lcpmaxterm

    - Syntax

pppoe set transport {<name>|<number>} lcpmaxterm <lcp max terminate>

    - Description

This command sets the Link Control Protocol (LCP) maximum terminate number for an existing PPPoE transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| lcp max term | Link Control Protocol; the maximum number of consecutive Terminate Requests that will be sent without reply before assuming that the destination address is unable to respond. The LCPfailterm can be any positive value. | 2 |

    - Example

prompt> pppoe set transport pppoe1 lcpmaxterm 20

    - See also

pppoe show transport

pppoe list transports

pppoe set transport localip

    - Syntax

pppoe set transport {<name>|<number>} localip <ip-address>

    - Description

This command tells the PPP process the local IP address to be associated with the client-end of an interface. This enables remote users to have dialin access via the channel(s) that the interface is attached to.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| ip-address | The IP address of the local ‘client-end’ of an interface displayed in the following format: 111.222.254.4 | 0.0.0.0 |

    - Example

prompt> pppoe set transport pppoe1 localip 192.168.103.2

    - See also

pppoe show transport

pppoe list transports

pppoe set transport remoteip

pppoe set transport mbs

    - Syntax

pppoe set transport {<name>|<number>} mbs <max burst size>

    - Description

This command sets the maximum burst size (mbs) for an existing PPPoE transport that performs dialout over PVC.

This command is only valid if you set VBR or VBR RT as the QoS Class using the pppoe set transport quosclass command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| maximum burst size | Controls the maximum burst size for VBR (Variable Bit Rate) and VBR RT (VBR Real Time) channels. This value overrides an existing BT value (if set). The MBS can be any value between 0 and 100. | 0 |

    - Example

prompt> pppoe set transport pppoe3 mbs 10

    - See also

pppoe set transport bt

pppoe set transport qosclass

pppoe set transport mcr

    - Syntax

pppoe set transport {<name>|<number>} mcr <min cell rate>

    - Description

This command sets the minimum cell rate for an existing PPPoE transport that performs dialout over PVC.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| minimum cell rate | Determines the minimum rate at which ATM cells are allowed to be sent along the PPPoE transport. | 0 |

    - Example

prompt> pppoe set transport pppoe2 mcr 0

    - See also

pppoe set transport pcr

pppoe set transport password

    - Syntax

pppoe set transport {<name>|<number>} password <password>

    - Description

This command sets a dialout password on a named transport. The password is required when PPP negotiation takes place and is supplied to the remote PPP server for authentication.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| password | An arbitrary word that acts as a dialout password enabling you to login to the remote end. The password will be required by the PPP server when you want to login remotely. It can be made up of one or more characters and/or digits. To display the password, use the pppoe show transport command. | N/A |

    - Example

prompt> pppoe set transport pppoe2 password mercury

    - See also

pppoe list transports

pppoe show transport

pppoe set transport username

pppoe set transport pcr

    - Syntax

pppoe set transport {<name>|<number>} pcr <peak cell rate>

    - Description

This command sets the peak cell rate (pcr) for an existing PPPoE transport that performs dialout over PVC.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| peak cell rate | Determines the maximum rate at which ATM cells are allowed to be sent along the PPPoE transport. The PCR can be any value from 0 up to the maximum PortSpeed parameter set when the port was created (using the initbun file in FlashFS or the CLI command port set). | 0 |

    - Example

prompt> pppoe set transport pppoe2 pcr 50000

    - See also

pppoe set transport mcr

pppoe set transport port

    - Syntax

pppoe set transport {<name>|<number>} port <port>

    - Description

This command sets the port that is used to transport PPPoE data.

For PVC transports, the port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| port | The system port that is used to transport data. | N/A |

    - Example

prompt> pppoe set transport pppoe4 port a1

    - See also

pppoe list transports

port ?

To list Ethernet ports, see the CLI command ethernet list ports .

pppoe set transport pvc

    - Syntax

pppoe set transport {<name>|<number>} pvc <port> <vpi> <vci>

    - Description

This command sets the PVC information fro an existing PPPoE transport PVC. The PVC uses the interface that was specified when the transport was created.

The command allows you to specify the following PVC information:

- the ATM port that will transport data

- VPI (Virtual Path Identifier)

- VCI (Virtual Circuit Identifier)

The port/VPI/VCI combination must be unique for each transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| port | The system port that is used to transport ATM data. | N/A |
| Vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |
| vci | Part of ATM header. The VCI is a tag that identifies which channel a cell will travel over. The VCI can be any value between 1 and 65535. | N/A |

    - Example

prompt> pppoe set transport pppoe4 pvc a1 0 800

    - See also

pppoe list transports

pppoe set transport dialin pvct

pppoe set transport dialout pvc

port list

pppoe set transport qosclass

    - Syntax

pppoe set transport {<name>|<number>} qosclass {ubr|cbr|vbr|vbrrt|abr|qfc}

    - Description

This command sets the quality of service class for an existing PPPoE transport that performs dialout over PVC.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| ubr | Unspecified Bit Rate; non-constant and unpredictable data transport rate. PCR (Peak Cell Rate) is the average and maximum speed of transmission. | UBR |
| cbr | Constant Bit Rate; constant demand and predictable data transport rate. PCR is the average and maximum speed of transmission. |  |
| vbr | Variable Bit Rate; non-constant but predictable data transport rate that uses Non-Real-Time (NRT). You can specify the PCR, SCR, BT and MBS for VBR traffic. |  |
| vbrrt | Variable Bit Rate Real-Time; non- constant but predictable data transport rate that uses Real-Time (RT). You can specify the PCR, SCR, BT and MBS for VBRRT traffic. |  |
| abr | Available Bit Rate; non-constant and unpredictable data transport rate that provides ATM-layer feedback and flow control. |  |
| qfc | QFC; ATM flow control protocol that supports ABR. |  |

    - Example

prompt> pppoe set transport pppoe3 abr

    - See also

pppoe show transport

pppoe set transport qosclass

pppoe set transport remotedns

    - Syntax

pppoe set transport {<name>|<number>} remotedns <ipaddress> [<ipaddress2>]

    - Description

This command is a PPP server function.

This command sets the primary and secondary local DNS server addresses that will be given to a remote PPP peer when the peer requests a primary or secondary DNS server IP address using IPCP. Setting the secondary IP address is optional.

If you want to delete an IP address, set the IP address to 0.0.0.0.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| ipaddress | The ip address of the primary local DNS server displayed in the following format: 192.168.102.3 | 0.0.0.0 (no primary address set) |
| ipaddress2 | The ip address of the secondary local DNS server displayed in the following format: 192.168.102.3 | 0.0.0.0 (no secondary address set) |

    - Examples

Example One - setting a primary address

prompt> pppoe set transport pppoe1 remotedns 192.168.102.3

Example Two - setting primary and secondary addresses

To set primary and secondary addresses, use this command syntax:

prompt> pppoe set transport pppoe1 remotedns 192.168.102.3 192.168.105.1

Example Three - deleting an address

To delete an address, set it to 0.0.0.0. The example below deletes the secondary address that was set in Example Two:

prompt> pppoe set transport pppoe1 remotedns 192.168.102.3 0.0.0.0

    - See also

pppoe set transport givedns client enabled|disabled

pppoe set transport givedns relay enabled|disabled

pppoe set transport discoverdns primary

pppoe set transport discoverdns secondary

pppoe set transport remoteip

    - Syntax

pppoe set transport {<name>|<number>} remoteip <ip-address>

    - Description

This command sets the IP address supplied to the remote end of the PPP connection during negotiation. This is particularly important for PPP dialin transports.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| ip-address | The IP address of the local ‘server-end’ of an interface displayed in the following format: 111.222.254.4 | 0.0.0.0 |

    - Example

prompt> pppoe set transport pppoe1 remoteip 192.168.103.2

    - See also

pppoe show transport

pppoe list transports

pppoe set transport localip

pppoe set transport remotenbns

    - Syntax

pppoe set transport {<name>|<number>} remotenbns <ipaddress> [<ipaddress2>]

    - Description

This command is a PPP server function.

This command sets the primary and secondary local NBNS server addresses that will be given to a remote PPP peer when the peer requests a primary or secondary NBNS server IP address using IPCP. Setting the secondary IP address is optional.

If you want to delete an IP address, set the IP address to 0.0.0.0.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| ipaddress | The ip address of the primary local NBNS server displayed in the following format: 192.168.102.3 | 0.0.0.0 (no primary address set) |
| ipaddress2 | The ip address of the secondary local NBNS server displayed in the following format: 192.168.102.3 | 0.0.0.0 (no secondary address set) |

    - Examples

Example One - setting a primary address

prompt> pppoe set transport pppoe1 remotenbns 192.168.102.3

Example Two - setting primary and secondary addresses

To set primary and secondary addresses, use this command syntax:

prompt> pppoe set transport pppoe1 remotenbns 192.168.102.3 192.168.105.1

Example Three - deleting an address

To delete an address, set it to 0.0.0.0. The example below deletes the secondary address that was set in Example Two:

prompt> pppoe set transport pppoe1 remotenbns 192.168.102.3 0.0.0.0

pppoe set transport routemask

    - Syntax

pppoe set transport {<name>|<number>} routemask <mask>

    - Description

This command sets the subnet mask used by the route that is created when a PPP link comes up. If it is set to 0.0.0.0, the subnet mask is determined by the IP address of the remote end of the link. The class of the IP address is obtained during IPCP (Internet Protocol Control Protocol) negotiation.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| mask | The subnet mask that is used for the route that is created when a PPP link comes up. 0.0.0.0 | 0.0.0.0 |

    - Example

prompt> pppoe set transport pppoe1 routemask 0.0.0.0

    - See also

pppoe show transport

pppoe list transports

pppoe set transport scr

    - Syntax

pppoe set transport {<name>|<number>} scr <sustainable cell rate>

    - Description

This command sets the sustainable cell rate for an existing PPPoE transport that performs dialout over PVC. This command is only valid if you set VBR or VBR RT as the QoS Class using the pppoe set transport qosclass command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| sustainable cell rate | Sustainable Cell Rate; the average cell rate for a VBR or VBR RT connection. The SCR can be any positive value that is less than the PortSpeed and the PCR for the channel. | 0 |

    - Example

prompt> pppoe set transport pppoe2 scr 25000

    - See also

pppoe set transport qosclass

pppoe set transport servicename

pppoe set transport {<name>|<number>} servicename <servicename>

    - Description

This command specifies the service name that is acceptable to the PPPoE client.

You can also set the access concentrator using the set transport accessconcentrator command so that PPPoE will only accept a specific service via a specific access concentrator.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| service name | A PPPoE tag that identifies a specific service that is acceptable to the PPPoE client. If set, the PPPoE transport will connect to the first access concentrator it finds that uses this service. If an access concentrator is also set, the PPPoE transport will connect to the specified service on the named concentrator. The service name is determined by your ISP. | Empty string |

    - Example

prompt> pppoe set transport pppoe1 servicename jupiter

    - See also

pppoe list transports

pppoe set transport accessconcentrator

pppoe show transport

For more information on PPPoE and service names, see RFC2516; http://www.ietf.org/rfc/rfc2516.txt.

pppoe set transport specificroute

    - Syntax

pppoe set transport {<name>|<number>} specificroute {enabled | disabled}

    - Description

Note - This command is only valid if the pppoe set transport createroute command is set to enabled. If the createroute command is set to disabled, no route is created and therefore the pppoe set transport specificroute command is ignored.

This command specifies whether the route created when a PPP link comes up is a specific or default route. If set to enabled, the route created will only apply to packets for the subnet at the remote end of the PPP link. The address of this subnet is obtained during IPCP negotiation.

The mask for the route is calculated from the class of the remote subnet unless an alternative has been specified using the pppoe set transport routemask command. If specificroute is set to disabled, a default route to the subnet at the remote end of the PPP link is created. Note that the current setting of this command is ignored if pppoe set transport createroute command is set to disabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| enabled | Allows the created route to apply to packets for the subnet at the remote end of the PPP link. | disabled |
| disabled | A default route to the subnet at the remote end of the PPP link is created. |  |

    - Example

prompt> pppoe set transport pppoe1 specificroute disabled

    - See also

pppoe set transport routemask

pppoe set transport createroute

pppoe list transports

pppoe set transport subnetmask

    - Syntax

pppoe set transport {<name>|<number>} subnetmask <mask>

    - Description

This command sets the subnet mask used for the local IP interface connected to the PPP transport. If the value 0.0.0.0 is supplied, the netmask will be calculated from the class of the IP address obtained during IPCP negotiation.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| mask | The subnet mask used by the local IP interface connected to the PPP transport. | 0.0.0.0 |

    - Example

prompt> pppoe set transport pppoe1 subnetmask 255.255.255.0

    - See also

pppoe list transports

pppoe set transport theylogin

    - Syntax

pppoe set transport {<name>|<number>} theylogin {none|pap|chap}

    - Description

This command sets the authentication method that remote PPP clients must use to dialin to the server. If authentication is used, clients must use the specified authentication method and provide the username set using the system add user command.

This command is only valid if the user has maydialin set using the system set login maydialin command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| none | No authentication method is set. | None |
| pap | Password Authentication Protocol; the server sends an authentication request to the remote user dialing in. PAP passes the unencrypted username and password and identifies the remote end. |  |
| chap | Challenge Handshake Authentication Protocol; the server sends an authentication request to the remote user dialing in. PAP passes the encrypted username and password and identifies the remote end. |  |

    - Example

prompt> pppoe set transport pppoe2 theylogin pap

    - See also

pppoe list transports

pppoe show transport

system add user

system set user maydialin

pppoe set transport username

    - Syntax

pppoe set transport {<name>|<number>} username <username>

    - Description

This command sets a (dialout) username on a named transport. The username is required when PPP negotiation takes place and is supplied to the remote PPP server for authentication.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| username | A name that identifies a user and, together with the dialout password, enables a user to login to the remote end. The username will be required by the PPP server when the user wants to login remotely. It can be made up of one or more characters and/or digits. To display the username, use the pppoe show transport command. | N/A |

    - Example

prompt> pppoe set transport pppoe2 username jsmith

    - See also

pppoe set transport password

pppoe set transport vci

    - Syntax

pppoe set transport {<name>|<number>} vci <vci>

    - Description

This command sets the Virtual Circuit Identifier for an existing PPPoE transport that performs dialout over PVC.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| vci | Part of the ATM header. The VCI is a tag that identifies which channel a cell will travel over. The VCI can be any value between 1 and 65535. | N/A |

    - Example

prompt> pppoe set transport pppoe4 vci 800

    - See also

pppoe list transports

pppoe show transport

pppoe set transport vpi

pppoe set transport vpi

    - Syntax

pppoe set transport {<name>|<number>} vpi <vpi>

    - Description

This command sets the Virtual Path Identifier for an existing PPPoE transport that performs dialout over PVC.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| vpi | A field in the ATM header. The VPI is used to identify the virtual path that a circuit belongs to. The VPI can be any value between 0 and 255. | N/A |

    - Example

prompt> pppoe set transport pppoe3 vpi 0

    - See also

pppoe list transports

pppoe show transport

pppoe set transport vci

pppoe set transport welogin

    - Syntax

pppoe set transport {<name>|<number>} welogin {none|auto|pap|chap}

    - Description

This command sets the authentication protocol used to connect to external PPP servers (dialout).

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |
| none | No authentication method is used. | None |
| auto | The authentication protocol used by the remote PPP server is discovered and used. |  |
| pap | Password Authentication Protocol; the server sends an authentication request to the remote user dialing in. PAP passes the unencrypted username and password and identifies the remote end. |  |
| chap | Challenge Handshake Authentication Protocol; the server sends an authentication request to the remote user dialing in. PAP passes the encrypted username and password and identifies the remote end. |  |

    - Example

prompt> pppoe set transport pppoe2 theylogin pap

    - See also

pppoe set transport theylogin

pppoe show transport

pppoe list transports

pppoe show transport

    - Syntax

pppoe show transport {<name>|<number>}

    - Description

This command displays the following information about an existing PPPoE transport:

- Description

- Interface number

- Server - dialin status

- Headers - the data format that the transport can accept or receive

- SVC status (false)

- Local IP address

- Subnet mask

- Remote IP address

- Remote DNS

- Propagate DNS to client (true or false)

- Propagate DNS to relay (true or false)

- Create route (true or false)

- Specific route (true or false)

- Route netmask

- Dialout Username

- Dialout Password

- Dialout Authentication method

- Dialin Authentication method

- LCP Max Configure

- LCP Max Failure

- LCP Max Terminate

- LCP Echo Every

- Autoconnect status (true or false)

- User Idle Timeout setting (in minutes)

- Access concentrator

- Service name

- Port name

- VPI (PVC transport only)

- VCI (PVC transport only)

- Quality of Service (QoS) class (PVC transport only)

- Peak cell rate (PVC transport only)

- Burst tolerance (PVC transport only)

- Sustainable Cell Rate (SCR) (PVC transport only)

- Maximum burst size (MBS) (PVC transport only)

- Maximum Cell Rate (MCR) (PVC transport only)

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing PPPoE transport. To display transport names, use the pppoe list transports command. | N/A |
| number | A number that identifies an existing PPPoE transport. To display transport numbers, use the pppoe list transports command. | N/A |

    - Example

prompt> pppoe show transport pppoe2

PPP Transport: pppoe2

Description: pppoe2

Interface ID: 1                 Server: false

Headers: learn             SVC: false

Local IP: 0.0.0.0

Subnet mask: 0.0.0.0

Remote IP: 0.0.0.0

Remote DNS: 0.0.0.0

Propogate DNS to client: true  To relay: true

Create route: true

Specific route: false

Route netmask: 0.0.0.0

Dialout username:

Dialout password:

Dialout auth.: none

Dialin auth.: none

LCP Max. Conf.: 10

LCP Max. Failure: 5

LCP Max Terminate: 2

LCP Echo Every: 10

Autoconnect: true

User Idle Timeout: 30

Access Conc.:

Service name: y

Port: a1          VPI: 0           VCI: 300

QOS class: UBR

Peak cell rate: 59111     Burst tolerance: 0

Sustainable cell rate: 0         Max. burst size: 0

Sustainable cell rate: 0         Max. burst size: 0

    - See also

pppoe list transports