TFTPC CLI commands

This chapter describes CLI support for TFTP Client.

34.1 Summary

34.1.1 TFTP Client CLI commands

The table below lists the TFTP Client commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| tftpc connect | tftpc connect |
| tftpc disconnect | tftpc disconnect |
| tftpc get | tftpc get |
| tftpc put | tftpc put |

34.1.2 TFTP Console commands

The table below lists the tftp console commands and, if they are usable or blacklisted:

| Command | CLI status |
| --- | --- |
| connect | Replaced by CLI command tftpc connect |
| get | Replaced by CLI command tftpc get |
| help | Blacklisted command, see help |
| init | Blacklisted command, see init |
| list | Blacklisted command, see list |
| put | Replaced by CLI command tftpc put |
| trace | Blacklisted command, see trace |
| version | Blacklisted command, see version |

tftpc connect

## Syntax

tftpc connect <host>

## Description

This command allows you to specify the remote node name or IP address for the host node that will be used in subsequent client mode transfers.

Either a node name may be entered, searched for in the ipaddresses configuration file, or an IP address in the form abc.def.ghi.jkl.

This command is required before a client mode user first attempts to put or get a file, but need not be issued again unless you want to change the remote host node name or address.

## Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| host | The host name or IP address of TFTP server. | N/A |

## Example

prompt> tftpc connect 192.168.200.10

## See also

tftpc disconnect

tftpc disconnect

## Syntax

tftpc disconnect

## Description

This command disconnects the existing host that was specified in tftpc connect command.

## Example

prompt> tftpc disconnect

## See also

tftpc connect

tftpc get

## Syntax

tftpc get <src> <dst>

## Description

This command configures TFTP to retrieve a file from the remote host previously specified using tftpc connect command

## Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| src | The name of the file to be retrieved from the connected host. | N/A |
| dst | The name given to the file once it has been retrieved from source. | N/A |

## Example

prompt> tftpc get ipaddresses addresslistA

## See also

tftpc put

tftpc put

## Syntax

tftpc put <src> <dst>

## Description

This command configures TFTP to transmit a file to the remote host previously specified using tftpc connect command.

## Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| src | The name of the file to be transmitted from source. | N/A |
| dst | The name given to the file once it has been transmitted to the connected host. | N/A |

## Example

prompt> tftpc put addresslistA ipaddresses

## See also

tftpc get