- IPSec CLI commands

This chapter describes the IPSec CLI commands.

Summary

    - IPSec CLI commands

The table below lists the IPSec commands provided by the CLI:

| Command | Description/Console command |
| --- | --- |
| ipsec add policy | ipsec add |
| ipsec delete policy | ipsec delete |
| ipsec clear policies | ipsec clear policies |
| ipsec list policies | ipsec list policies |
| ipsec show policy | ipsec show policy |
| ipsec set policy enable | ipsec set policy enable |
| ipsec set policy peergw | ipsec set policy peergw |
| ipsec set policy localnetwork | ipsec set policy localnetwork |
| ipsec set policy peernetwork | ipsec set policy peernetwork |
| ipsec set policy proposal AH auth | ipsec set policy proposal AH auth |
| ipsec set policy proposal ESP auth | ipsec set policy proposal ESP auth |
| ipsec set policy proposal ESP encryption | ipsec set policy proposal ESP encryption |
| ipsec set policy proposal pfs | ipsec set policy proposal pfs |
| ipsec set policy ike psk | ipsec set policy ike psk |
| ipsec set policy sa lifetime isakmp | ipsec set policy sa lifetime isakmp |
| ipsec set policy sa lifetime ipsec | ipsec set policy sa lifetime ipsec |

ipsec add policy

    - Syntax

ipsec add policy <name> ipinterface <if-name>

    - Description

This command creates a policy on an interface that connects to the internal network.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| if-name | IP interface name |  |

    - Example

prompt> ipsec add policy p1 ininterface ipoa-0

    - See also

ipsec delete policy

ipsec list policies

ipsec show policy

ipsec delete policy

    - Syntax

ipsec delete policy <name>

    - Description

This command deletes a single existing policy from the IPSec configuration.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |

    - Example

prompt> ipsec delete policy p1

    - See also

ipsec add policy

ipsec list policies

ipsec show policy

ipsec clear policies

    - Syntax

ipsec clear policies

    - Description

This command deletes all existing policies from the IPSec configuration.

    - Example

prompt> ipsec clear policies

    - See also

ipsec add policy

ipsec delete policy

ipsec list policies

    - Syntax

ipsec list policies

    - Description

This command lists the following information about policies that were added to the ipsec using the ipsec add policy command:

Policy ID number

Policy name

Activate - True means if the policy isenabled. False means the policy is disabled.

Local Net – Local selector .

Remote Net – Remote selector .

    - Example

prompt> ipsec list policies

IPSec Policies:

ID |    Name     | Active    |     Local Net    |      Peer Net

-----------------------------------------------------------------------

1 |  p1         | true      | 192.168.21.0/32  | 192.168.100.2

-----------------------------------------------------------------------

    - See also

ipsec show policy

ipsec add policy

ipsec show policy

    - Syntax

ipsec show policy <name>

    - Description

This command displays information about a single policy that was added to the IPSec configuration using the ipsec add policy command. The following policy information is displayed:

Policy name

Activate - True means if the policy isenabled. False means the policy is disabled.

Remote gateway and selector.

Local selector .

Proposal.

PSK (Pre-shared Key)

SA lifetime

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |

    - Example

prompt> ipsec show policy p1

IPSec Policy: p1

Active: true

WAN i/f: ipoa-0

Encapsulation: tunnel

Auto Key: true

Peer GW: 12.7.56.20

Peer Net: IPV4_ADDR

addr: 192.168.100.2

mask/end: 0.0.0.0

Local i/f:

Local Net: IPV4_ADDR_SUBNET

addr: 192.168.21.0

mask/end: 255.255.255.0

Proposal: AH  Auth    none

ESP Auth    md5

ESP Encrypt des

PFS Group   none

IKE Parameters:

PSK: 1234567

Lifetime 1: 240 minutes

Lifetime 2: 60 minutes

    - See also

ipsec list policies

ipsec show policy state

    - Syntax

ipsec show policy <name> state

    - Description

This command displays state about a single policy that was added to the IPSec configuration using the ipsec add policy command. The following information is displayed:

Policy name

Activate - True means if the policy isenabled. False means the policy is disabled.

Remote gateway and selector.

Local selector.

Proposal.

Connected status

Connect Time

Number of outbound and inbound packets

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| Name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |

    - Example>

prompt> ipsec show policy p1 state

IPSec Policy: p1

Active: true

WAN i/f: ipoa-0

Peer GW: 12.7.56.20

Peer Net: IPV4_ADDR

addr: 192.168.100.2

mask/end: 0.0.0.0

Local i/f:

Local Net: IPV4_ADDR_SUBNET

addr: 192.168.21.0

mask/end: 255.255.255.0

Proposal: AH  Auth    none

ESP Auth    md5

ESP Encrypt des

PFS Group   none

State:

Connected: true

Connect Time: 1350 seconds

Rx: 220

Tx: 197

    - See also

ipsec show policy

ipsec set policy enable

    - Syntax

set policy <name> enabled|disabled

    - Description

This command enable or disable a single policy that was added to the IPSec configuration using the ipsec add policy command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| enabled | Enables the existing IPSec policy. | enabled |
| disabled | Disables the existing IPSec policy. |  |

    - Example

Enable an IPSec policy:

prompt> ipsec set policy enabled

Disable an IPSec policy:

prompt> ipsec set policy enabled

ipsec set policy <name> peergw

    - Syntax

ipsec set policy <name> peergw {ipaddress <addr> | hostname <domain-name>}

    - Description

This command sets the IP address or hostname of the peer gateway.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| addr | IP address of the peer gateway. |  |
| domain-name | IP host of the peer gateway |  |

    - Example

- prompt> ipsec set policy p1 peergw ipaddress 12.7.56.20

- prompt> ipsec set policy p1 peergw hostname testnet.com.tw

    - See also

ipsec set policy peernetwork

ipsec set policy localnetwork

ipsec set policy localnetwork

    - Syntax

ipsec set policy <name> localnetwork ipaddress <addr> [netmask <mask> | endaddress <addr2>]

    - Description

This command sets the selector of local network.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| addr | IP address of the local network. |  |
| mask | Netmask of the local network. |  |
| addr2 | End IP address of the local network. |  |

    - Example

- If the local selector covers only one host with IP address 192.168.1.123

- prompt> ipsec set policy p1 localnetwork ipaddress 192.168.1.123

- If the local selector covers subnet 192.168.1.0/24

- prompt> ipsec set policy p1 localnetwork ipaddress 192.168.1.0 netmask 255.255.255.0

- If the local selector covers hosts with IP from 192.168.1.100 to 192.168.1.150

- prompt> ipsec set policy p1 localnetwork ipaddress 192.168.1.100 endaddress 192.168.1.150

    - See also

ipsec set policy peergw

ipsec set policy peernetwork

ipsec set policy peernetwork

    - Syntax

ipsec set policy <name> peernetwork ipaddress <addr> [netmask <mask> | endaddress <addr2>]

    - Description

This command sets the selector of peer network.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| addr | IP address of the peer network. |  |
| mask | Netmask of the peer network. |  |
| addr2 | End IP address of the peer network. |  |

    - Example

- If the peer selector covers only one host with IP address 192.168.100.2

- prompt> ipsec set policy p1 peernetwork ipaddress 192.168.100.2

- If the peer selector covers subnet 192.168.100.0/24

- prompt> ipsec set policy p1 peernetwork ipaddress 192.168.100.0 netmask 255.255.255.0

- If the peer selector covers hosts with IP from 192.168.100.100 to 192.168.100.150

- prompt> ipsec set policy p1 peernetwork ipaddress 192.168.100.100 endaddress 192.168.100.150

    - See also

ipsec set policy peergw

ipsec set policy localnetwork

ipsec set policy proposal AH auth

    - Syntax

ipsec set policy <name> proposal AH auth {none | md5 | sha1}

    - Description

This command sets hash function used for AH protocol.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| none | AH protocol is not used. | none |
| md5 | MD5 hash function is used. |  |
| sha1 | SHA1 hash function is used. |  |

    - Example

- prompt> ipsec policy p1 proposal AH auth sha1

    - See also

ipsec set policy proposal ESP auth

ipsec set policy proposal ESP encryption

ipsec set policy proposal pfs

ipsec set policy proposal ESP auth

    - Syntax

ipsec set policy <name> proposal ESP auth {none | md5 | sha1}

    - Description

This command sets hash function used for ESP protocol.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| none | Hash function is not used for ESP protocol | sha1 |
| md5 | MD5 hash function is used. |  |
| sha1 | SHA1 hash function is used. |  |

    - Example

- prompt> ipsec policy p1 proposal ESP auth md5

    - See also

- ipsec set policy proposal AH auth

- ipsec set policy proposal ESP encryption

- ipsec set policy proposal pfs

ipsec set policy proposal ESP encryption

    - Syntax

- ipsec set policy <name> proposal ESP encryption {null | des | 3des | aes}

    - Description

- This command sets encryption algorithm used for ESP protocol.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| null | Encryption algorithm not used for ESP protocol | 3des |
| des | DES encryption is used. |  |
| 3des | Triple DES encryption is used. |  |
| aes | AES encryption with 128-bit key is used. |  |

    - Example

- prompt> ipsec policy p1 proposal ESP encryption aes

    - See also

- ipsec set policy proposal ESP auth

- ipsec set policy proposal ESP auth

- ipsec set policy proposal pfs

- ipsec set policy proposal pfs

- Syntax

- ipsec set policy <name> proposal pfs {none | dh1 | dh2 | dh5}

    - Description

- This command sets PFS (perfect forward secrecy) group.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| none | PFS is not used. | none |
| dh1 | Diffie-Hallman group 1 (768 bits). |  |
| dh2 | Diffie-Hallman group 2 (1024 bits). |  |
| dh5 | Diffie-Hallman group 5 (1536 bits). |  |

    - Example

- prompt> ipsec policy p1 proposal pfs dh2

    - See also

- ipsec set policy proposal AH auth

- ipsec set policy proposal ESP auth

- ipsec set policy proposal ESP encryption

- ipsec set policy ike psk

    - Syntax

- ipsec set policy <name> ike psk <psk-key>

    - Description

- This command sets pre-share key.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| psk-key | Preshared key |  |

    - Example

- prompt> ipsec policy p1 ike psk abcdefghijklm

    - See also

- ipsec add policy

- ipsec set policy sa lifetime isakmp

    - Syntax

- ipsec set policy <name> sa lifetime isakmp <minutes>

    - Description

- This command sets ISAKMP SA lifetime .

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| minutes | ISAKMP SA lifetime in minutes | 60 |

    - Example

- prompt> ipsec policy p1 sa lifetime isakmp 120

    - See also

- ipsec set policysa lifetime ipsec

- ipsec set policy sa lifetime ipsec

    - Syntax

- ipsec set policy <name> sa lifetime ipsec <minutes>

    - Description

- This command sets IPSec SA lifetime.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IPSec policy. To display policy names, use the ipsec list policies command. | N/A |
| minutes | IPSec SA lifetime in minutes | 240 |

    - Example

- prompt> ipsec policy p1 sa lifetime ipsec 360

    - See also

ipsec set policysa isakmp ipsec