- TCP/IP CLI commands

This chapter describes the TCP/IP CLI commands.

Summary

    - IP Tracing commands

You can carry out tracing in the IP stack using the following system commands:

- system trace enable|disable; enables/disables the tracing support output for a specific module and category.

- system trace list; displays the tracing options for the modules available in your current image.

    - IP CLI commands

The table below lists the IP commands provided by the CLI:

| Command | Reference |
| --- | --- |
| ip add interface | ip add interface |
| ip add route | ip add route |
| ip add defaultroute gateway | ip add defaultroute gateway |
| ip add defaultroute interface | ip add defaultroute interface |
| ip attachbridge | ip attachbridge |
| ip attach | ip attach |
| ip attachvirtual | ip attachvirtual |
| ip clear arpentries | ip clear arpentries |
| ip clear interfaces | ip clear interfaces |
| ip clear riproutes | ip clear riproutes |
| ip clear routes | ip clear routes |
| ip delete interface | ip delete interface |
| ip delete route | ip delete route |
| ip detach interface | ip detach interface |
| ip interface add proxyarpentry | ip interface add proxyarpentry |
| ip interface clear proxyarpentries | ip interface clear proxyarpentries |
| ip interface delete proxyarpentries | ip interface delete proxyarpentries |
| ip interface list proxyarpentries | ip interface list proxyarpentries |
| ip interface add proxyarpexclusion | ip interface add proxyarpexclusion |
| ip interface delete proxyarpexclusion | ip interface delete proxyarpexclusion |
| ip interface add secondaryipaddress | ip interface add secondaryipaddress |
| ip interface clear secondaryipaddress | ip interface clear secondaryipaddresses |
| ip interface delete secondaryipaddress | ip interface delete secondaryipaddress |
| ip interface list secondaryipaddresses | ip interface list secondaryipaddresses |
| ip list arpentries | ip list arpentries |
| ip list connections | ip list connections |
| ip list interfaces | ip list interfaces |
| ip list riproutes | ip list riproutes |
| ip list routes | ip list routes |
| ip ping | ip ping |
| ip set interface ipaddress | ip set interface ipaddress |
| ip set netmask | ip set interface netmask |
| ip set interface mtu | ip set interface mtu |
| ip set interface dhcp | ip set interface dhcp |
| ip set interface rip accept | ip set interface rip accept |
| ip set interface rip multicast | ip set interface rip multicast |
| ip set interface tcpmssclamp | ip set interface tcpmssclamp |
| ip set interface rip send | ip set interface rip send |
| ip set rip advertisedefault | ip set rip advertisedefault |
| ip set rip authentication | ip set rip authentication |
| ip set rip defaultroutecost | ip set rip defaultroutecost |
| ip set rip hostroutes | ip set rip hostroutes |
| ip set rip password | ip set rip password |
| ip set rip poison | ip set rip poison |
| ip set route destination | ip set route destination |
| ip set route gateway | ip set route gateway |
| ip set route cost | ip set route cost |
| ip set route interface | ip set route interface |
| ip show | ip show |
| ip show interface | ip show interface |
| ip show route | ip show route |

    - IP Console commands

The table below lists the ip console commands and, if available, their CLI equivalent command:

| Command | CLI status |
| --- | --- |
| ip config | Replaced by CLI command ip show and ip list interfaces |
| ip device add | Replaced by CLI command ip add interface |
| ip device delete | Replaced by CLI command ip delete interface |
| ip device flush | Blacklisted command, see device |
| ip device list | Replaced by CLI command ip list interfaces |
| ip enable | Replaced by CLI command ip attach |
| ip help | Usable command, see help |
| ip portname | Blacklisted command, see portname |
| ip snmp access list | Usable command, see snmp |
| ip snmp access delete | Blacklisted command, see snmp |
| ip snmp access flush | Blacklisted command, see snmp |
| ip snmp access read¦write | Blacklisted command, see snmp |
| ip snmp config | Usable command, see snmp |
| ip snmp config save | Blacklisted command, see snmp |
| ip snmp trap add | Blacklisted command, see snmp |
| ip snmp trap delete | Blacklisted command, see snmp |
| ip snmp trap list | Usable command, see snmp |
| ip version | Replaced by CLI command ip show |

ip add interface

    - Syntax

ip add interface <name> [<ipaddress> [<netmask>]]

    - Description

This command adds a named interface and optionally sets its IP address. The IP address is not mandatory at this stage, but if it is not specified in this command, the interface will be unconfigured. There are three ways that the IP address can be set later:

- using the ip set interface ipaddress command

- you can set the interface to obtain its configuration via Dynamic Host Configuration Protocol (DHCP) using the ip set interface dhcp enabled command. By default, DHCP is disabled.

- this interface can obtain its IP configuration via PPP IPCP (Internet Protocol Control Protocol) negotiation. See PPPoA CLI commands, PPPoE CLI commands or PPPoH CLI commands.

The IP stack automatically creates a loopback interface for address 127.0.0.1 subnet mask 255.0.0.0. This interface is not displayed by the ip list interfaces command.

You can use this command to add unnumbered interfaces. For more details, see Options.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the ip interface. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| ipaddress | The IP address of the interface displayed in the following format: 192.168.102.3 If the IP address is set to the special value 0.0.0.0, the interface is marked as unconfigured. This value is used when the interface address is obtained automatically. For unnumbered interface, the IP address parameter is used to specify the router-id of the interface. The router-id should be the same as the IP address of one of the router’s numbered interfaces. | 0.0.0.0 |
| netmask | The netmask address of the interface displayed in the following format: 255.255.255.0 The special value 255.255.255.255 is used to indicate an unnumbered interface. An unnumbered interface is configured by setting the IP address to the interface’s router-id value, and setting netmask to 255.255.255.255. | If no IP address is supplied, the natural mask of the IP address is used. |

    - Example

prompt> ip add interface ip1 192.168.103.3 255.255.255.0

    - See also

ip attach

ip show interface

ip set interface ipaddress

ip set interface dhcp

For information on setting DHCP client configuration options, see DHCP Client CLI commands.

ip add route

    - Syntax

ip add route <name> <dest_ip> <netmask> {[gateway <gateway_ip>]|[interface <interface>]}

    - Description

This command creates a static route to a destination network address via a gateway device or an existing interface. It also allows you to create a default route.

Note - You can only create one default route. A default route will not be created if you have already created a default route using the ip add defaultroute gateway command or the ip add defaultroute interface command.

A route specifies a destination network (or single host), together with a mask to indicate what range of addresses the network covers, and a next-hop gateway address or interface. If there is a choice of routes for a destination, the route with the most specific mask is chosen.

Routes are used when sending datagrams as well as forwarding them, so they are not relevant only to routers. However, a system with a single interface is likely to have a single route as a default route to the router on the network that it most often needs to use. If the interface can communicate more efficiently with a particular destination by using a different router, then it will learn this fact from an Internet Control Message Protocol (ICMP) redirect message.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the route. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. To create a default static route to a destination address, type default as the route name. You can only create one route called default. | N/A |
| dest_ip | The IP address of the destination network displayed in the following format: 192.168.102.3 | N/A |
| netmask | The destination netmask address displayed in the following format: 255.255.255.0 | N/A |
| gateway_ip | The IP address of the gateway that this route will use, displayed in the following format: 192.168.102.3 | N/A |
| interface | The name of the existing interface that this route will use. To display interface names, use the ip list interfaces command. | N/A |

    - Examples

There are two examples in this section. Example 1 routes through a gateway. Example 2 is a default route.

Example 1

prompt> ip add route route1 192.168.103.3 255.255.255.0 gateway 192.168.102.3

Example 2

prompt> ip add route default 0.0.0.0 0.0.0.0 interface ip1

    - See also

ip list interfaces

ip add defaultroute gateway

ip add defaultroute interface

ip add defaultroute gateway

    - Syntax

ip add defaultroute gateway <gateway_ip>

    - Description

This command creates a default route. It acts as a shortcut command that you can use instead of typing the following:

ip add route default 0.0.0.0 0.0.0.0 gateway 192.168.103.3

Note - You can only create one default route. A default route will not be created if you have already created a default route using the ip add route command or the ip add defaultroute interface command.

If you want RIP to advertise a default route with a default cost metric, see the ip set rip advertisedefault and ip set rip defaultroutecost commands.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| gateway_ip | The IP address of the gateway that this route will use by default, displayed in the following format: 192.168.102.3 | N/A |

    - Example

prompt> ip add defaultroute gateway 192.168.103.3

    - See also

ip add route

ip add defaultroute interface

ip set rip advertisedefault

ip set rip defaultroutecost

ip add defaultroute interface

    - Syntax

ip add defaultroute interface <interface>

    - Description

This command creates a default route. It acts as a shortcut command that you can use instead of typing the following:

ip add route default 0.0.0.0 0.0.0.0 interface ip3

Note - You can only create one default route. A default route will not be created if you have already created a default route using the ip add route command or the ip add defaultroute gateway command.

If you want RIP to advertise a default route with a default cost metric, see the ip set rip advertisedefault and ip set rip defaultroutecost commands.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| interface | The name of the existing interface that this route will use. To display interface names, use the ip list interfaces command. | N/A |

    - Example

prompt> ip add defaultroute interface ip3

    - See also

ip add route

ip add defaultroute gateway

ip set rip advertisedefault

ip set rip defaultroutecost

ip attach

    - Syntax

ip attach {<name>|<number>} <transport>

    - Description

This command attaches an existing transport to an existing IP interface (e.g., a bridge or router) so that data can be transported via the selected transport method.

This command implicitly enables the transport being attached.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| transport | A name that identifies an existing transport. | N/A |

    - Example

In the example below, eth1 is the name of an ethernet transport created using the ethernet add transport command:

prompt> ip attach ip1 eth1

    - See also

ip add interface

ip list interfaces

For information on a particular transport type, see the relevant transport chapter:

PPPoA CLI commands

PPPoE CLI commands

IPoA CLI commands

Ethernet CLI commands

RFC1483 CLI commands

ip attachbridge

    - Syntax

ip attachbridge {<name>|<number>}

    - Description

This command attaches the bridge to the router via an existing IP interface.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> ip attachbridge 2

    - See also

ip add interface

ip list interfaces

ip attachvirtual

    - Syntax

ip attachvirtual <name> <real_interface>

    - Description

This command creates a virtual interface. The virtual interface is associated with a ‘real’ IP interface that has already been attached to a transport using the ip attach command. You can attach multiple virtual interfaces to one ‘real’ IP interface.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface that will be used as a virtual interface. The IP interface should not have a transport attached to it. To display interface names, use the ip list interfaces command. | N/A |
| real_interface | A name that identifies an existing IP interface. A virtual interface must be associated with an existing ‘real’ IP interface that is attached to a transport. To display interface names, use the ip list interfaces command. | N/A |

    - Example

prompt> ip attachvirtual ip_virtual ip_real

    - See also

ip list interfaces

ip clear arpentries

    - Syntax

ip clear arpentries

    - Description

This command clears all ARP entries listed in the IP ARP table.

    - Example

prompt> ip clear arpentries

    - See also

ip list arpentries

ip clear interfaces

    - Syntax

ip clear interfaces

    - Description

This command clears all IP interfaces that were created using the ip add interface command.

    - Example

prompt> ip clear interfaces

    - See also

ip delete interface

ip clear riproutes

    - Syntax

ip clear riproutes

    - Description

This command deletes all the existing dynamic routes that have been obtained from RIP. It does not delete the static routes; see the ip clear routes command.

    - Example

prompt> ip clear riproutes

    - See also

ip clear routes

ip set rip hostroutes

ip set interface rip accept

ip set interface rip send

ip clear routes

    - Syntax

ip clear routes

    - Description

This command clears all static routes that were created using the ip add route command.

    - Example

prompt> ip clear routes

    - See also

ip delete route

ip 	delete interface

    - Syntax

ip delete interface {<name>|<number>}

    - Description

This command deletes a single IP interface that was created using the ip add interface command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command.The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> ip delete interface ip1

    - See also

ip clear interfaces

ip list interfaces

ip delete route

    - Syntax

ip delete route {<name>|<number>}

    - Description

This command deletes a single route that was created using the ip add route command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing route. To display route names, use the ip list routes command. | N/A |
| number | A number that identifies an existing route. To display route numbers, use the ip list routes command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> ip delete route route1

    - See also

ip list routes

ip 	detach interface

    - Syntax

ip detach {<name>|<number>}

    - Description

This command detaches a transport from an IP interface which was previously attached using the ip attach interface command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> ip detach ip1

    - See also

ip list interfaces

ip interface add proxyarpentry

    - Syntax

ip interface {<name>|<number>} add proxyarpentry <ipaddress> [<netmask>]

    - Description

This command configures proxy ARP functionality on an existing IP interface. This means that an interface responds to ARP requests for both its own address and for any address that has been configured as a proxy ARP address.

You can configure proxy ARP functionality on a single address or a range of addresses. Once you have configured a range of proxy ARP interfaces, you can set one or more addresses in the range to NOT respond to proxy ARP using the ip interface add proxyarpexclusion command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| ipaddress | The IP address (or range of addresses) of the interface that you want to set as a proxy ARP entry, displayed in the following format: 192.168.102.3 | N/A |
| netmask | The netmask address (or range of addresses) of the interface, displayed in the following format: 255.255.255.0 | N/A |

    - Example

The following command adds proxy ARP support to the entire subnet 192.168.100.0:

prompt> ip interface ip1 add proxyarpentry 192.168.100.0 255.255.255.0

    - See also

ip interface add proxyarpexclusion

ip interface list proxyarpentries

ip interface clear proxyarpentries

    - Syntax

ip interface {<name>|<number>} clear proxyarpentries

    - Description

This command clears all proxy arp entries and exclusions that were created using the ip interface add proxyarpentry and ip interface add proxyarpexclusion commands.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> ip interface ip1 clear proxyarpentries

    - See also

ip interface add proxyarpentry

ip interface add proxyarpexclusion

ip interface delete proxyarpentries

    - Syntax

ip interface {<name>} delete proxyarpentries <number>

    - Description

This command deletes a single proxy arp entries that was created using the ip interface add proxyarpentry command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing proxy ARP entry. To display proxy ARP entry numbers, use the ip interface list proxyarpentries command. | N/A |

    - Example

prompt> ip interface ip1 delete proxyarpentry 1

    - See also

ip interface add proxyarpentry

ip interface list proxyarpentries

ip interface list proxyarpentries

    - Syntax

ip interface {<name>|<number>} list proxyarpentries

    - Description

This command displays information about proxy arp entries and exclusions that were created using the ip interface add proxyarpentry and ip interface add proxyarpexclusion commands.

The following information is displayed:

- interface ID numbers

- IP address and netmask of proxy ARP entries and exclusions

- Exclusion status; true for exclusions, false for inclusions

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> ip interface ip1 list proxyarpentries

ID  |    IP Address  |   Netmask      |  Exclude

-----|----------------|----------------|----------

1 | 192.168.100.0  | 255.255.255.0  | false

2 | 192.168.100.8  | 255.255.255.254| true

--------------------------------------------------

    - See also

ip interface add proxyarpentry

ip interface add proxyarpexclusion

ip list interfaces

ip interface add proxyarpexclusion

    - Syntax

ip interface {<name>|<number>} add proxyarpexclusion <ipaddress> [<netmask>]

    - Description

This command configures proxy ARP exclusion functionality on an existing IP interface. This means that once you have configured an interface with a range of proxy ARP interfaces, you can set one or more addresses in the range to NOT respond to proxy ARP.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| ipaddress | The IP address (or range of addresses) of the interface that you want to set as a proxy ARP exclusion entry, displayed in the following format: 192.168.102.3 | N/A |
| netmask | The netmask address (or range of addresses) of the interface, displayed in the following format: 255.255.255.0 | N/A |

    - Example

The first command below adds proxy ARP support to the subnet 192.168.100.0 . The second command adds proxy ARP exclusion support to 192.168.100.10 255.255.255.254:

prompt> ip interface ip1 add proxyarpentry 192.168.100.0 255.255.255.0

prompt> ip interface ip1 add proxyarpexclusion 192.168.100.10 255.255.255.254

This means that the entire 192.168.100.0 subnet supports proxy ARP, EXCEPT for addresses 192.168.100.10 and 192.168.100.11.

    - See also

ip interface add proxyarpentry

ip interface list proxyarpentries

ip interface delete proxyarpexclusion

    - Syntax

ip interface {<name>} delete proxyarpexclusion <number>

    - Description

This command deletes a single proxy arp exclusion entry that was created using the ip interface add proxyarpexclusion command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing proxy ARP exclusion entry. To display proxy ARP exclusion numbers, use the ip interface list proxyarpentries command. | N/A |

    - Example

prompt> ip interface ip1 delete proxyarpexclusion 2

    - See also

ip interface add proxyarpexclusion

ip interface list proxyarpentries

ip interface add secondaryipaddress

    - Syntax

ip interface {<name>|<number>} add secondaryipaddress <ipaddress> [<netmask>]

    - Description

This command adds a secondary IP address to an existing IP interface. A secondary address may be used to create an extra IP address on an interface for management purposes, or to allow the IP stack to route between two subnets on the same interface.

The functionality of secondary IP addresses depends on several parameters including the type of IP interface and the netmask:

- if a secondary address is on the same subnet as the primary interface address, you do not need to specify a subnet mask for that secondary address. This applies to all interface types.

- if a secondary address is on a different subnet to the primary address, and the interface is Ethernet or a transport using a bridged encapsulation, you must specify the subnet mask. The IP stack will listen on the new address for connections to local services (e.g., for management purposes), and will also route packets to the new subnet.

- if a secondary address is on a different subnet to the primary address, and the interface is a point-to-point interface, specifying a netmask is optional.

- for the same behavior as described for Ethernet interfaces above, the subnet mask should be specified.

- If the subnet mask is not specified, the IP address will not be associated with any subnet, but will still be recognized as one of the IP stack’s own addresses for local traffic.

Note - The ability to specify a subnet mask with a secondary address is still supported, but in latest system software, this is superseded by the functionality of virtual interfaces. You should use virtual interfaces instead, see ip attachvirtual.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| ipaddress | A secondary IP address that you want to add to the main IP interface. You can add any number of secondary IP addresses. The IP address is displayed in the following format: 192.168.102.3 To display the secondary IP addresses, use the ip interface list secondaryipaddresses command. | N/A |
| netmask | The netmask of the secondary IP address displayed in the following format: 255.255.255.0 To display the secondary IP addresses, use the ip interface list secondaryipaddresses command. | none specified |

    - Example

prompt> ip interface ip1 add secondaryipaddress 192.168.102.3 255.255.255.0

    - See also

ip list interfaces

ip interface list secondaryipaddresses

ip interface clear secondaryipaddresses

    - Syntax

ip interface {<name>|<number>} clear secondaryipaddresses

    - Description

This command deletes all additional IP addresses that have been added to an existing IP interface using the ip interface add secondaryipaddress command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> ip interface ip1 clear secondaryipaddresses

    - See also

ip list interfaces

ip interface add secondaryipaddress

ip interface delete secondaryipaddress

ip interface list secondaryipaddresses

ip interface delete secondaryipaddress

    - Syntax

ip interface {<name>|<number>} delete secondaryipaddress <secondaryipaddress number>

    - Description

This command deletes a single secondary IP address that has previously been added to an existing IP interface using the ip interface add secondaryipaddress command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| secondary ipaddress number | The number that identifies a secondary IP address that you want to delete from the main IP interface. To display secondary IP address numbers, use the ip interface list secondaryipaddresses command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> ip interface ip1 delete secondaryipaddress 1

    - See also

ip list interfaces

ip interface list secondaryipaddresses

ip interface list secondaryipaddresses

    - Syntax

ip interface {<name>|<number>} list secondaryipaddresses

    - Description

This command lists the secondary IP addresses (and netmasks if applicable) that have been added to an existing IP interface using the ip interface add secondaryipaddress command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Example

In the example output below, secondary IP addresses without netmasks associated with them appear as 0.0.0.0 by default.

prompt> ip interface ip1 list secondaryipaddresses

ID  |    IP Address   |    Netmask

-----|-----------------------------------

1 | 192.168.104.6   |  255.255.255.0

2 | 192.168.103.4   |  0.0.0.0

3 | 192.168.103.2   |  0.0.0.0

-----------------------------------------

    - See also

ip list interfaces

ip interface list secondaryipaddresses

ip list arpentries

    - Syntax

ip list arpentries

    - Description

This command displays the ARP table which lists the following information:

- IP addresses and corresponding MAC addresses obtained by ARP.

- IP interface on which the host is connected

- Static status - ‘no’ for dynamically generated ARP entries; ‘yes’ for static entries added by the user.

    - Example

prompt> ip list arpentries

IP ARP table entries:

IP address      | MAC address       | Interface    | Static

-----------------|-------------------|--------------|--------

10.10.10.10     | 00:20:2b:e0:03:87 | 3            | no

-----------------|-------------------|--------------|--------

20.20.20.20     | 00:20:2b:03:0a:72 | 2            | no

-----------------|-------------------|--------------|--------

30.30.30.30     | 00:20:2b:03:09:c4 | 1            | no

-------------------------------------------------------------

    - See also

ip clear arpentries

ip list connections

    - Syntax

ip list connections

    - Description

This command lists the active TCP/UDP connections in use by applications running on the device. It displays the following information:

- Protocol type (TCP or UDP)

- Local connection address

- Remote connection address

- Connection state for TCP connections

This command does not show raw socket connections or UDP connections opened internally within the IP stack.

    - Example

The example below shows an active telnet connection, WebServer, TFTP server and SNMP:

prompt> ip list connections

Local TCP/UDP connections:

Proto  | Local address          | Remote address         | State

-------|------------------------|------------------------|--------------

tcp   | 192.168.91.19:23       | 192.168.91.18:1080     | ESTABLISHED

tcp   | *:80                   | *:*                    | LISTEN

udp   | *:69                   | *:*                    |

udp   | *:161                  | *:*                    |

------------------------------------------------------------------------

ip 	list interfaces

    - Syntax

ip list interfaces

    - Description

This command lists information about IP interfaces that were added using the ip add interface command. The following information is displayed:

- interface ID numbers

- interface names

- IP addresses (if previously specified)

- DHCP status

- Whether a transport is attached to the interface, and if so, the name of the transport

- Whether a virtual interface is attached to a real interface. The name of the attached virtual interface is displayed in the Transport column in square brackets, for example [ip2]

    - Example

prompt> ip list interfaces

IP Interfaces:

ID  |    Name      |    IP Address    |   DHCP   |  Transport

-----|--------------|------------------|----------|---------------

1 | ppp_device   | 192.168.102.2    | disabled | pppoe1

2 | ip2          | 192.168.102.3    | disabled | Not attached

3 | ip_real      | 192.168.101.2    | disabled | ethernet1

4 | ip_virtual   | 192.168.150.1    | disabled | [ip_real]

------------------------------------------------------------------

    - See also

ip show interface

ip set interface dhcp

ip list riproutes

    - Syntax

ip list riproutes

    - Description

This command lists information about the routes that have been obtained from RIP. It displays the following:

- destination IP addresses

- destination netmask address

- gateway address

- cost - The number of hops counted as the cost of the route.

- timeout - the number of seconds that this RIP route will remain in the routing table unless updated by RIP.

- source interface - the name of the existing interface that this route uses

    - Example

prompt> ip list riproutes

IP RIP routes:

Destination     | Mask            | Gateway         | Cost | Time | Source

-----------------|-----------------|-----------------|------|------|-------

192.168.101.1   | 255.255.255.0   | 10.10.10.10     | 1    | 3000 | ip2

---------------------------------------------------------------------------

    - See also

ip set rip hostroutes

ip set interface rip accept

ip set interface rip send

ip list routes

    - Syntax

ip list routes

    - Description

This command lists information about existing routes. It displays the ID, name, destination IP address (if applicable), netmask address (if applicable) and gateway address or interface name (whichever is applicable).

- route ID numbers

- route names

- destination IP addresses (if previously specified)

- destination netmask address (if previously specified)

- Either the gateway address or the name of the destination interface (whichever is set)

    - Example

prompt> ip list routes

IP routes:

ID  |    Name    | Destination      | Netmask          | Gateway/Interface

-----|------------|------------------|------------------|------------------

2 | route2     | 192.168.102.3    | 255.255.255.0    | ip1

1 | route1     | 192.168.50.50    | 255.255.255.0    | 192.168.68.68

---------------------------------------------------------------------------

    - See also

ip show route

ip ping

    - Syntax

ip ping <dest-ip>

    - Description

This command pings a specified destination IP address.

Note - You can only ping IP addresses. You can not ping host names using DNS client.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| dest-ip | The IP address of the destination machine that you want to ping, displayed in the following format: 192.168.102.3 | N/A |

    - Example

prompt> ip ping 192.168.102.3

ip: ping - reply received from 192.168.102.3

If ping was unsuccessful, the following output is displayed:

ip: ping - no reply received.

ip set interface ipaddress

    - Syntax

ip set interface {<name>|<number>} ipaddress <ipaddress> [<netmask>]

    - Description

This command sets the IP address for an existing IP interface.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| ip address | The IP address of the interface displayed in the following format: 192.168.102.3 If the IP address is set to the special value 0.0.0.0, the interface is marked as unconfigured. This value is used when the interface address is obtained automatically. For unnumbered interfaces, the IP address parameter is used to specify the router-id of the interface. The router-id should be the same as the IP address of one of the router’s numbered interfaces. | 0.0.0.0 |
| netmask | The netmask address of the interface displayed in the following format: 255.255.255.0 The special value 255.255.255.255 is used to indicate an unnumbered interface. An unnumbered interface is configured by setting the IP address to the interface’s router-id value, and setting netmask to 255.255.255.255. | If no IP address is supplied, the natural mask of the IP address is used. |

    - Example

prompt> ip set interface ip4 ipaddress 192.168.102.3 255.255.255.0

    - See also

ip set interface mtu

ip set interface dhcp

ip list interfaces

ip set interface netmask

ip set interface netmask

    - Syntax

ip set interface {<name>|<number>} netmask <netmask>

    - Description

This command sets the netmask for an existing IP interface.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| netmask | The netmask address of the interface displayed in the following format: 255.255.255.0 The special value 255.255.255.255 is used to indicate an unnumbered interface. An unnumbered interface is configured by setting the IP address to the interface’s router-id value, and setting netmask to 255.255.255.255. | N/A |

    - Example

prompt> ip set interface ip6 netmask 255.255.255.0

    - See also

ip set interface ipaddress

ip list interfaces

ip set interface mtu

    - Syntax

ip set interface {<name>|<number>} mtu <mtu>

    - Description

This command sets the MTU (Maximum Transmission Unit) for an existing IP interface.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | 1500 |
| mtu | Maximum Transmission Unit: maximum packet size (in bytes) that an interface can handle. The MTU should be set to a value appropriate for the transport attached to the interface (typically from 576 to 1500 bytes). For example, Ethernet and most other transports support an MTU of 1500 bytes, whereas PPPoE supports an MTU of 1492 bytes. | 1500 |

    - Example

prompt> ip set interface ip2 mtu 800

    - See also

ip set interface ipaddress

ip set interface dhcp

ip list interfaces

ip set interface dhcp

    - Syntax

ip set interface {<name>|<number>} dhcp {enabled|disabled}

    - Description

This command specifies whether a named interface should obtain its configuration via DHCP.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| enabled | The interface obtains its configuration information from DHCP client. | disabled |
| disabled | The interface does not use DHCP client configuration information. |  |

    - Example

prompt> ip set interface ip2 dhcp enabled

    - See also

ip set interface ipaddress

ip set interface mtu

For information on setting DHCP client configuration options, see DHCP Client CLI commands.

ip list interfaces

ip set interface rip accept

    - Syntax

ip set interface {<name>|<number>} rip accept {none|v1|v2|all}

    - Description

This command specifies whether or not an existing interface accepts RIP messages. You can specify what version of RIP messages are accepted by the interface.

When receiving RIP v1 messages, the IP stack tries to use the information it has available to determine the appropriate subnet mask for the addresses received.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interface command. The number appears in the first column under the heading ID. | N/A |
| none | The interface does not accept RIP messages. | none |
| v1 | The interface only accepts RIP version 1 messages (RFC1058). |  |
| v2 | The interface only accepts RIP version 2 messages (RFC1723). |  |
| all | The interface accepts RIP version 1 (RFC1058) and RIP version 2 (RFC1723) messages. |  |

    - Example

prompt> ip set interface ip3 rip accept none

    - See also

ip set interface rip send

ip set interface rip multicast

ip set rip hostroutes

ip set rip poison

ip show

ip list interfaces

ip set interface rip multicast

    - Syntax

ip set interface {<name>|<number>} rip multicast {enabled | disabled}

    - Description

This command allows you to enable/disable whether RIP version 2 messages are sent via multicast.

RIP version 2 messages sent via multicast are only received by the hosts on the network that have a multicast network address. If this command is disabled, RIP version 2 messages are sent via broadcast and are received by all the hosts on the network.

You need to set RIP to send v2 messages using the ip set interface rip send command in order for the ip set interface rip multicast enabled command to send version 2 messages via multicast.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| enabled | Allows RIP version 2 messages to be sent via multicast. | disabled |
| disabled | Disables RIP version 2 messages being sent via multicast. Messages are sent via broadcast instead. |  |

    - Example

prompt> ip set interface ip1 rip multicast enabled

    - See also

ip list interfaces

ip set interface rip send

ip set interface rip send

    - Syntax

ip set interface {<name>|<number>} rip send {none|v1|v2|all}

    - Description

This command specifies whether or not an existing interface can send RIP messages. You can specify which version of RIP messages will broadcast routing information on the interface. Routing information is broadcast every 30 seconds or when the RIP routing table is changed.

Note - RIP version 1 does not allow specification of subnet masks; a RIP version 1 route that appears to be to an individual host might in fact be to a subnet, and treating it as a route to the whole network may be the best way to make use of the information.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |
| rip send none | The interface does not accept RIP messages. | rip send none (this command affects all interfaces except loopback interfaces) |
| rip send v1 | The interface only sends RIP version 1 messages (RFC1058) |  |
| rip send v2 | The interface only sends RIP version 2 messages (RFC1723). If set, RIP version 2 is used on all non-loopback interfaces. |  |
| rip send all | The interface sends RIP version 1 (RFC1058) and RIP version 2 (RFC1723) messages. |  |

    - Example

prompt> ip set interface ip1 rip send v1

    - See also

ip set interface rip accept

ip set rip hostroutes

ip set rip poison

ip show

ip list interfaces

For information on RFC1058 and RFC1723, see http://www.ietf.org/rfc/rfc1723.txt

ip set interface tcpmssclamp

    - Syntax

ip set interface <name> tcpmssclamp {enabled|disabled}

Description

This command enables/disables TCP MSS (Maximum Segment Size) Clamp functionality on an existing IP interface. When TCP MSS Clamp is enabled on an interface, all TCP traffic routed through that interface will be examined. If a TCP SYN (synchronize/start) segment is sent with a maximum segment size larger than the interface MTU (Maximum Transmission Unit), the MSS option will be rewritten in order to allow TCP traffic to pass through the interface without requiring fragmentation.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| enabled | TCP SYN segments routed through this interface will be examined and, if necessary, modified. | disabled |
| disabled | The IP stack will not examine or modify TCP traffic routed through this interface. |  |

    - Example

prompt> ip set interface ip2 tcpmssclamp enabled

    - See also

ip set interface mtu

ip show

ip set rip advertisedefault

    - Syntax

ip set rip advertisedefault {enabled | disabled}

    - Description

This command enables/disables the advertising of a default route via RIP. If you set this to enabled, then create a default route using the ip add defaultroute commands, the route will also be added to those advertised by the RIP protocol. The cost associated with the route is the value set using the ip set rip defaultroutecost command.

You must enable default advertising before you create the default route.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | Enables RIP to advertise a default route with the cost metric set using the ip set rip defaultroutecost command. | disabled |
| disabled | Disables advertisement of a default route. |  |

    - Example

prompt> ip set rip advertisedefault enabled

    - See also

ip add defaultroute gateway

ip add defaultroute interface

ip set rip defaultroutecost

ip set rip authentication

    - Syntax

ip set rip authentication {enabled | disabled}

    - Description

This command enables/disables RIP v2 plain text authentication.

If enabled, a plain text authentication string is placed in RIP v2 packets. RIP v2 packets will only be accepted if they contain an authentication entry with the correct password string. Packets with no authentication or the wrong password will be rejected.

To set an authentication password, use the ip set rip password command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enabled | Accepts RIP v2 packets that contain an authentication entry with the correct password string. Packets with no authentication or the wrong password are rejected. | disabled |
| disabled | Rejects RIP v2 packets containing an authentication entry. |  |

    - Example

prompt> ip set rip authentication enabled

    - See also

ip set rip password

ip show

ip set rip defaultroutecost

    - Syntax

ip set rip defaultroutecost <cost>

    - Description

This command sets the number of hops counted as the cost of a default route advertised via RIP.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| cost | The number of hops counted as the cost of the default route. The cost value can be any positive integer between 1 and 15. | 1 |

    - Example

prompt> ip set rip defaultroutecost 10

    - See also

ip add defaultroute gateway

ip add defaultroute interface

ip set rip advertisedefault

ip set rip hostroutes

    - Syntax

ip set rip hostroutes {enabled | disabled}

    - Description

Specifies whether IP interfaces will accept RIP routes to specific routes.

Note - RIP version 1 does not allow specification of subnet masks; a RIP version 1 route that appears to be to an individual host might in fact be to a subnet, and treating it as a route to the whole network may be the best way to make use of the information.

To display the current state of rip hostroutes, use the ip show command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| rip hostroutes enabled | Sets the hostroutes flag to on. The interface accepts RIP routes to specific routes. | rip hostroutes disabled |
| rip hostroutes disabled | Sets the hostroutes flag to off: RIP version 1 routes to individual hosts are treated as routes to the network containing the host. RIP version 2 routes to individual hosts are ignored. |  |

    - Example

prompt> ip set rip hostroutes enabled

    - See also

ip set interface rip accept

ip set interface rip send

ip show

ip set rip password

    - Syntax

ip set rip password <password>

    - Description

This command sets an authentication string that is placed in RIP v2 packets if ip set rip authentication is enabled.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| password | An authentication password used by RIP v2 packets if ip set rip authentication is enabled. The password is a string of 0 to 16 characters. | N/A |

    - Example

prompt> ip set rip password vancouver

    - See also

ip set rip authentication

ip show

ip set rip poison

    - Syntax

ip set rip poison {enabled | disabled}

    - Description

Enables or disables the poisoned reverse flag. If this flag is on, ATMOS TCP/IP performs poisoned reverse as defined in RFC 1058; see that RFC for discussion.

To display the current state of the poisoned reverse flag, use the ip show command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| rip poison enabled | Sets the poisoned reverse flag to on. ATMOS TCP/IP performs poisoned reverse as defined in RFC 1058. | rip poison disabled |
| rip poison disabled | Sets the poisoned reverse flag to off: |  |

    - Example

prompt> ip set rip poison enabled

    - See also

ip set interface rip accept

ip set interface rip send

ip set rip hostroutes

ip show

ip set route destination

    - Syntax

ip set route {<name>|<number>} destination <dest-network> <netmask>

    - Description

This command sets the destination network address of a route previously created using the ip add route command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing route. To display route names, use the ip list routes command. | N/A |
| number | A number that identifies an existing route. To display route numbers, use the ip list routes command. The number appears in the first column under the heading ID. | N/A |
| dest-network | The IP address of the destination network displayed in the following format: 192.168.102.3 | N/A |
| netmask | The destination netmask address displayed in the following format: 255.255.255.0 | N/A |

    - Example

prompt> ip set route route1 destination 192.168.103.3 255.255.255.0

    - See also

ip set route gateway

ip set route cost

ip list routes

ip set route gateway

    - Syntax

ip set route {<name>|<number>} gateway <gateway>

    - Description

This command sets the gateway address of a route previously created using the ip add route command. If you want the route to go directly to its destination and not via a gateway, specify 0.0.0.0 as the gateway.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing route. To display route names, use the ip list routes command. | N/A |
| number | A number that identifies an existing route. To display route numbers, use the ip list routes command. The numbers appear in the first column under the heading ID. | N/A |
| gateway | The IP address of the gateway that the IP routes through, displayed in the following format: 192.168.102.3 If you added a route directly to an interface, the gateway address is set by default to 0.0.0.0 so that no gateway is specified. | N/A |

    - Example

prompt> ip set route route1 gateway 192.168.102.3

    - See also

ip add route

ip set route destination

ip set route cost

ip list routes

ip set route cost

    - Syntax

ip set route {<name>|<number>} cost <cost>

    - Description

This command sets the number of hops counted as the cost of the route for a route previously created using the ip add route command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing route. To display route names, use the ip list routes command. | N/A |
| number | A number that identifies an existing route. To display route numbers, use the ip list routes command. The number appears in the first column under the heading ID. | N/A |
| cost | The number of hops counted as the cost of the route. This may affect the choice of route when the route is competing with routes acquired from RIP. (Using a mixture of RIP and static routing is not advised). The cost value can be any positive integer. | 1 |

    - Example

prompt> ip set route route1 cost 3

    - See also

ip add route

ip set route destination

ip set route gateway

ip list routes

ip set route interface

    - Syntax

ip set route {<name>|<number>} interface {<interface>|none}

    - Description

This command sets the interface used by a route previously created by the ip add route command. If you want the existing route to route to an address via a gateway device, use none so that no interface is set.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing route. To display route names, use the ip list routes command. | N/A |
| number | A number that identifies an existing route. To display route numbers, use the ip list routes command. The number appears in the first column under the heading ID. | N/A |
| interface | The name of the existing interface that the ip routes through, displayed in the following format: 192.168.102.3 To display interface names, use the ip list interfaces command. | N/A |
| none | No interface is set. This is used for routes that route via a gateway device instead of an interface. | N/A |

    - Example

prompt> ip set route r1 interface eth1

    - See also

ip list interfaces

ip list routes

ip show

    - Syntax

ip show

    - Description

Shows current RIP configuration and any other information global to the router.

    - Example

prompt> ip show

Global IP configuration:

Host routes: true

Poison reverse: false

Authentication: true

Auth password: vancouver

    - See also

ip set rip hostroutes

ip set rip poison

ip show interface

    - Syntax

ip show interface {<name>|<number>}

    - Description

This command displays the following information about a named interface:

- IP address and netmask address (if set). For virtual interfaces, the name of the real interface that the virtual interface is attached to is also displayed.

- MTU (Maximum Transmission Unit)

- Status of DHCP

- Status of TCP MSS Clamp

- Status of RIP send and RIP accept

- Status of RIP multicast

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing IP interface. To display interface names, use the ip list interfaces command. | N/A |
| number | A number that identifies an existing IP interface. To display interface numbers, use the ip list interfaces command. The number appears in the first column under the heading ID. | N/A |

    - Examples

Real IP interface

prompt> ip show interface ip2

IP Interface: ip2

IP address: 192.168.102.3

Netmask: 255.255.255.0

MTU: 1500

DHCP: disabled

TCP MSS Clamp: disabled

Accept RIP V1: true

Send RIP V1: false

Accept RIP V2: true

Send RIP V2: false

Multicast RIP V2: disabled

Virtual IP interface

prompt> ip show interface ip3

IP Interface: ip3 - virtual [ip2]

IP address: 192.168.50.10

Netmask: 255.255.255.0

MTU: 1500

DHCP: disabled

TCP MSS Clamp: disabled

Accept RIP V1: true

Send RIP V1: false

Accept RIP V2: true

Send RIP V2: false

Multicast RIP V2: disabled

    - See also

ip show

ip show route

ip list interfaces

ip show route

    - Syntax

ip show route {<name>|<number>}

    - Description

This command displays the following information about a named route:

- Destination IP address

- Netmask address

- Gateway IP address

- Cost: the number of hops counted as the cost of the route

- Interface name

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existingroute. To display route names, use the ip list routes command. | N/A |
| number | A number that identifies an existing route. To display route numbers, use the ip list routes command. The number appears in the first column under the heading ID. | N/A |

    - Example

prompt> ip show route route3

IP route: route3

Destination: 192.168.102.3

Netmask: 255.255.255.0

Gateway: 192.168.108.3

Cost: 1

Interface:

    - See also

ip show

ip list routes