26.Port CLI commands

This chapter describes CLI support for ports.

| 1 Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.1 Port CLI commands |  |  |  |
| The table below lists the port commands provided by the CLI: |  |  |  |  |
|  |  | Command | Reference |  |
|  |  | port ? | port ? |  |
|  |  | port list | port list |  |
|  |  | port set | port set |  |
|  |  | port show | port show |  |

| 2 port ? |  |  |  |
| --- | --- | --- | --- |
|  | 2.1 Syntax |  |  |
|  |  | port ? |  |
|  | 2.2 Description |  |  |
|  |  |  | This command lists the ports that are currently available in your GlobespanVirata ISOS system. |
|  |  |  | Ports are configured in the ISFS initbun configuration file for the product that you are using. For example, the initbun file for a PC-attached Gateway (USB) is located in: |
|  |  |  | atmos/products/usb-gateway/flashfs/initbun |
|  | 2.3 Examples |  |  |
|  |  |  | The following examples lists the port names available on a DM8020: |
|  |  |  | prompt> port ? ethernet0 Port name list List ports by type usb-ethernet Port name wan Port name wireless Port name wlan_filtered Port name |
|  | 2.4 See also |  |  |
|  |  |  | port list port set port show |

| 3 port list |  |  |
| --- | --- | --- |
|  | 3.1 Syntax |  |
|  |  | port list ? |
|  |  | port list <port class> |
|  | 3.2 Description |  |
|  |  | The port list ? command lists all possible types of port and their port classes. The port list <port class> command allows you to display the port names that belong to a specific class (if that class is available in your GlobespanVirata system). |
|  |  | Ports are configured in the ISFS initbun configuration file for the product that you are using. For example, the initbun file for a PC-attached Gateway (USB) is located in: |
|  |  | atmos/products/usb-gateway/flashfs/initbun |
|  | 3.3 Examples |  |
|  |  | Example 1: port list ? prompt> port list ? 802.1x Port class all Port class ethernet Port class loopback Port class usb Port class prompt> port list |
|  |  | The last line of the port list ? command output displays the first part of the port list <port class>command. |
|  |  | Example 2: port list <port class> prompt> port list ethernet0 Valid port names in class 'ethernet': raw_ethernet ethernet0 wireless wan usb-ethernet pc-ethernet q1 pppoe wlan_filtered |
|  |  | If you type a port class that is not available in your system, an error message is displayed. |
|  | 3.4 See also |  |
|  |  | port ? port set port show |

| 4 port set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4.1 Syntax |  |  |  |  |  |
|  |  | port <portname> set <attribute> <value> |  |  |  |  |
|  | 4.2 Description |  |  |  |  |  |
|  |  | This command allows you to modify attributes on a port. Any modifications override existing attribute values specified in your ISOS device and compiled at run-time. The attributes available depend on: |  |  |  |  |
|  |  | • the type of port that you are using • the ISOS system that you are using |  |  |  |  |
|  |  | To display a list of valid attributes for a specific port, use the ‘?’syntax options key after port <portname> set. For example: |  |  |  |  |
|  |  | prompt> port ethernet0 set ? MAC resetDefaultss |  |  |  |  |
|  |  | Once you have identified the attribute that you want to modify, you can specify the new value that you want to set it to. |  |  |  |  |
|  | 4.3 Options |  |  |  |  |  |
|  |  | The following table gives the range of values for each option which can be specified with this command and a default value (if applicable). |  |  |  |  |
|  |  |  | Option | Description | Default value |  |
|  |  |  | portname | The name of a port that is configured in your ISOS system. To display available ports, use the port command. | N/A |  |
|  |  |  | attribute | A single attribute of a specified port. An attribute has a value attached to it which you can modify. | N/A |  |
|  |  |  | value | A value attached to an attribute. The value could be a numerical setting or a true/false qualifier. | N/A |  |
|  | 4.4 Example |  |  |  |  |  |
|  |  | prompt> port ethernet0 set resetDefaults true |  |  |  |  |
|  | 4.5 See also |  |  |  |  |  |
|  |  | port list port show |  |  |  |  |

| 5 port show |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5.1 Syntax |  |  |  |  |  |
|  |  | port <portname> show |  |  |  |  |
|  | 5.2 Description |  |  |  |  |  |
|  |  | This command displays the current attributes and values of a port. The attributes available depend on: • the type of port that you are using • the ISOS system that you are using |  |  |  |  |
|  | 5.3 Options |  |  |  |  |  |
|  |  | The following table gives the range of values for each option which can be specified with this command and a default value (if applicable). |  |  |  |  |
|  |  |  | Option | Description | Default value |  |
|  |  |  | portname | The name of a port that is configured in your ISOS system. To display available ports, use the port command. | N/A |  |
|  | 5.4 Example |  |  |  |  |  |

prompt> port hfa3841 show

UpdateStats                                   = 0

Version                                        = 1.10

PortClassEthernet                            = true

Reset                                          = false

Disable                                        = false

PromiscuousEnable                            = false

RxBroadcastEnable                            = true

RxMulticastEnable                            = true

RxMulticastAllEnable                        = true

RxAddressEnable                              = true

FullDuplexEnable                             = true

CrcEnable                                      = true

MAC                                             = 00:90:96:00:00:49

ApFirmwareVersion                            =

PrimaryFirmwareVersion                      =

ESSID                                           = act1

DefaultChannel                                    = 1

WepEncryption                                    = disabled

DefaultTxKey                                     = 0

Mode64Key0                                     = 00-00-00-00-00

Mode64Key1                                     = 00-00-00-00-00

Mode64Key2                                     = 00-00-00-00-00

Mode64Key3                                     = 00-00-00-00-00

Mode128Key0                                    = 00-00-00-00-00-00-00-00-00-00-00-00-00

Mode128Key1                                    = 00-00-00-00-00-00-00-00-00-00-00-00-00

Mode128Key2                                    = 00-00-00-00-00-00-00-00-00-00-00-00-00

Mode128Key3                                    = 00-00-00-00-00-00-00-00-00-00-00-00-00

RtsThreshold                                     = 2432

FragmentationThreshold                            = 2346

TxUnicastFrames                                 = 0

TxMulticastFrames                               = 0

TxFragments                                    = 0

TxUnicastOctets                                 = 0

TxMulticastOctets                                = 0

TxDeferredTransmissions                           = 0

TxSingleRetryFrames                              = 0

TxMultipleRetryFrames                            = 0

TxRetryLimitExceeded                            = 0

TxDiscards                                      = 0

RxUnicastFrames                                 = 0

RxMulticastFrames                               = 0

RxFragments                                    = 0

RxUnicastOctets                                 = 0

RxMulticastOctets                               = 0

RxFCSErrors                                   = 0

RxDiscardsNoBuffer                             = 0

TxDiscardsWrongSA                             = 0

RxDiscardsWEPUndecryptable                     = 0

RxMessageInMsgFragments                       = 0

RxMessageInBadMsgFragments                    = 0

RxOk                                          = 0

TxOk                                          = 0

TxAborts                                       = 0

RxCRCErrors                                    = 0

TxCollisions                                     = 0

RxOverlongPackets                               = 0

TxExcessiveCollisions                             = 0

RxShortPackets                                   = 0

SnmpIfIndex                                     = 2

SnmpIfDescr                                     = BUN Ethernet device

SnmpIfType                                     = 6

SnmpIfMtu                                      = 1500

SnmpIfSpeed                                     = 100000000

SnmpIfPhysAddress                               = 00:90:96:00:00:49

SnmpIfOperStatus                                = 1

SnmpIfAdminStatus                               = 1

SnmpIfLastChange                                = 218

SnmpIfInOctets                                   = 0

SnmpIfInUcastPkts                                = 0

SnmpIfInNUcastPkts                               = 0

SnmpIfInDiscards                                 = 0

SnmpIfInErrors                                   = 0

SnmpIfInUnknownProtos                           = 0

SnmpIfOutOctets                                 = 0

SnmpIfOutUcastPkts                              = 0

SnmpIfOutNUcastPkts                             = 0

SnmpIfOutDiscards                               = 0

SnmpIfOutErrors                                 = 0

SnmpIfOutQLen                                 = 0

SnmpIfSpecific                                  = (unset)

SnmpIfLinkUpDownTrapEnable                    = 2

SnmpIfPromiscuousMode                          = 2

SnmpIfAlias                                     =

|  | 5.5 See also |  |  |
| --- | --- | --- | --- |
| port list port set |  |  |  |