- Ethernet CLI commands

This chapter describes the Ethernet transport CLI commands.

Summary

    - Ethernet CLI commands

The table below lists the CLI commands for manipulating Ethernet channels:

| Command | Reference |
| --- | --- |
| ethernet add transport | ethernet add transport |
| ethernet clear transports | ethernet clear transports |
| ethernet delete transport | ethernet delete transport |
| ethernet list transports | ethernet list transports |
| ethernet list ports | ethernet list ports |
| ethernet set transport | ethernet set transport port |
| ethernet show transport | ethernet show transport |

    - Ethernet Console commands

The CLI commands above replace all previous ether console commands.

ethernet add transport

    - Syntax

ethernet add transport <name> [<port>]

    - Description

This command adds a named ethernet transport and allows you to specify which port it will use to transport ethernet data. The ports are defined in the initbun file for each type of product.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | An arbitrary name that identifies the transport. It can be made up of one or more letters or a combination of letters and digits, but it cannot start with a digit. | N/A |
| port | The system port that is used to transport ethernet data. You cannot use the same port for more than one ethernet transport at a time. | Ethernet |

    - Example

prompt> ethernet add transport eth1 ethernet

    - See also

ethernet list transports

ethernet list ports

port list

ethernet clear transports

    - Syntax

ethernet clear transports

    - Description

This command deletes all ethernet transports that were created using the ethernet add transport command.

    - Example

prompt> ethernet clear transports

    - See also

ethernet delete transport

ethernet delete transport

    - Syntax

ethernet delete transport {<name>|<number>}

    - Description

This command deletes a single ethernet transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing Ethernet transport. To display transport names, use the ethernet list transports command. | N/A |
| number | A number that identifies an existing Ethernet transport. To display transport numbers, use the ethernet list transports command. | N/A |

    - Example

prompt> ethernet delete transport eth1

    - See also

ethernet list transports

ethernet list ports

    - Syntax

ethernet list ports

    - Description

This command lists the valid ports that can be used to transport ethernet data.

The ports are defined in the initbun file for each type of product.

    - Example

prompt> ethernet list ports

Valid ethernet port names:

ethernet

hdlc

ethernet list transports

    - Syntax

ethernet list transports

    - Description

This command lists all ethernet transports that have been created using the ethernet add transport command. It displays the transport identification number and name, and the name of the port that it uses to transport ethernet data.

    - Example

prompt> ethernet list transports

Ethernet transports:

ID  |    Name    |    Port

-----|------------|------------

1 | eth2       | hdlc

2 | eth1       | ethernet

-------------------------------

    - See also

ethernet list ports

ethernet set transport port

    - Syntax

ethernet set transport {<name>|<number>} port <port>

    - Description

This command sets the port that an existing Ethernet transport uses to transport ethernet data.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing Ethernet transport. To display transport names, use the ethernet list transports command. | N/A |
| number | A number that identifies an existing Ethernet transport. To display transport numbers, use the ethernet list transports command. | N/A |
| port | The system port that is used to transport ethernet data. You cannot use the same port for more than one ethernet transport at a time. | Ethernet |

    - Example

prompt> ethernet set transport eth1 port hdlc

    - See also

ethernet add transport

ethernet list transports

port list

ethernet show transport

    - Syntax

ethernet show transport {<name>|<number>}

    - Description

This command displays the name and port used by an existing Ethernet transport.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| name | A name that identifies an existing Ethernet transport. To display transport names, use the ethernet list transports command. | N/A |
| number | A number that identifies an existing Ethernet transport. To display transport numbers, use the ethernet list transports command. | N/A |

    - Example

prompt> ethernet show transport eth1

Ethernet transport: eth1

Description: Default LAN port

Port: ethernet

    - See also

ethernet list transports