1. 802.1x CLI commands

This chapter describes the 802.1x CLI commands.

| 1 Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Note- This chapter is only relevant to you if you are running an image that contains the 802.1x module. |  |  |  |
|  | 1.1 802.1x CLI commands |  |  |  |
|  | The table below lists the 802.1x commands provided by the CLI: |  |  |  |
|  |  | Command | Description/Console command |  |
|  |  | 802.1x authenticator set authentication enabled¦disabled | 802.1x authenticator set authentication enabled¦disabled |  |
|  |  | 802.1x authenticator set authentication local¦radius | 802.1x authenticator set authentication local¦radius |  |
|  |  | 802.1x authenticator set identity | 802.1x authenticator set identity |  |
|  |  | 802.1x authenticator set rekey-timeout | 802.1x authenticator set rekey-timeout |  |
|  |  | 802.1x authenticator show | 802.1x authenticator show |  |

| 2 802.1x authenticator set authentication enabled¦disabled |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Note- This CLI command is only available if your image contains the 802.1x module. For more information, see Summary. |  |  |  |  |
|  |  | 2.1 Syntax |  |  |  |  |  |
|  |  |  | 802.1x authenticator set authentication {enabled¦disabled} |  |  |  |  |
|  |  | 2.2 Description |  |  |  |  |  |
|  |  | This command enables/disables authentication on your wireless device. Enabling this command sets the Access Point (AP) on your device as the authenticator in your network configuration. |  |  |  |  |  |
|  |  | 2.3 Options |  |  |  |  |  |
|  |  | The following table gives the range of values for each option which can be specified with this command and a default value (if applicable). |  |  |  |  |  |
|  |  |  | Option |  | Description | Default value |  |
|  |  |  | enabled |  | Enables authentication and sets the AP on your wireless device as the authenticator in your network configuration. | disabled |  |
|  |  |  | disabled |  | Disables authentication on your device. |  |  |
|  | 2.4 |  |  | Example prompt> 802.1x authenticator set authentication enabled |  |  |  |
|  | 2.5 |  |  | See also |  |  |  |
|  |  |  |  | 802.1x authenticator show. |  |  |  |

| 3 802.1x authenticator set authentication local¦radius |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Note- This CLI command is only available if your image contains the 802.1x module. For more information, see Summary. |  |  |  |
|  |  | 3.1 Syntax |  |  |  |  |  |  |
|  |  | 802.1x authenticator set authentication {local¦radius} |  |  |  |  |  |  |
|  |  | 3.2 Description |  |  |  |  |  |  |
|  |  | This command sets the method of authentication used by your authenticator (AP). |  |  |  |  |  |  |
|  |  | 3.3 Options |  |  |  |  |  |  |
| The following table gives the range of values for each option which can be specified with this command and a default value (if applicable). |  |  |  |  |  |  |  |  |
|  |  |  | Option |  |  | Description | Default value |  |
|  |  |  | local |  |  | Sets a local server as your authentication method (using MD5). | N/A |  |
|  |  |  | radius |  |  | Sets RADIUS as your authentication method (using EAP). | N/A |  |
|  | 3.4 |  |  | Example |  |  |  |  |
|  |  |  |  | prompt> 802.1x authenticator set authentication radius |  |  |  |  |
|  | 3.5 |  |  | See also |  |  |  |  |
|  |  |  |  | 802.1x authenticator show. |  |  |  |  |
|  |  |  |  | For details of RADIUS client commands, see RADIUS Client CLI commands. |  |  |  |  |

| 4 802.1x authenticator set identity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Note- This CLI command is only available if your image contains the 802.1x module. For more information, see Summary. |  |  |  |  |
|  | 4.1 Syntax |  |  |  |  |  |  |
|  | 802.1x authenticator set identity <string> |  |  |  |  |  |  |
|  | 4.2 Description |  |  |  |  |  |  |
|  | This command allows you to set a unique identity string for your device. The authenticator will use this string to identify your device. |  |  |  |  |  |  |
|  | 4.3 Options |  |  |  |  |  |  |
|  |  | The following table gives the range of values for each option which can be specified with this command and a default value (if applicable). |  |  |  |  |  |
|  |  |  | Option |  | Description | Default value |  |
|  |  |  | string |  | A unique string that identifies your device. If you want to use spaces in your string, you need to enclose the entire argument in straight double quotes. | N/A |  |
|  | 4.4 |  |  | Example |  |  |  |
|  |  |  |  | prompt> 802.1x authenticator set identity "GlobespanVirata Device 00:20:2b:80:43:20" |  |  |  |
|  | 4.5 |  |  | See also |  |  |  |
|  |  |  |  | 802.1x authenticator show. |  |  |  |

| 5 802.1x authenticator set rekey-timeout |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Note- This CLI command is only available if your image contains the 802.1x module. For more information, see Summary. |  |  |  |  |
|  |  | 5.1 Syntax |  |  |  |  |  |  |
|  |  |  |  | 802.1x authenticator set rekey-timeout <seconds> |  |  |  |  |
|  |  | 5.2 Description |  |  |  |  |  |  |
|  |  |  |  | This command sets the WEP rekey timeout that causes the issue of a new WEP key for the session. Once set to a suitable timeout, the keys will rotate at a rate fast enough to prevent an attacker from sniffing enough packets in order to discover the WEP key. |  |  |  |  |
|  |  | 5.3 Options |  |  |  |  |  |  |
|  |  |  | The following table gives the range of values for each option which can be specified with this command and a default value (if applicable). |  |  |  |  |  |
|  |  |  |  | Option |  | Description | Default value |  |
|  |  |  |  | seconds |  | The time (in seconds) before a WEP key is rotated. | 600 |  |
|  | 5.4 |  |  |  | Example prompt> 802.1x authenticator set rekey-timeout 800 |  |  |  |
|  | 5.5 |  |  |  | See also |  |  |  |
|  |  |  |  |  | 802.1x authenticator show. |  |  |  |

| 6 802.1x authenticator show |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Note- This CLI command is only available if your image contains the 802.1x module. For more information, see Summary. |
|  |  | 6.1 Syntax |  |  |  |
|  |  |  |  |  | 802.1x authenticator show |
|  |  | 6.2 Description |  |  |  |
|  |  |  |  | This command displays the following information about your 802.1x configuration: |  |
|  |  |  |  |  | • Authentication status (true/false) • Authentication Server method set • Identifier for your device created using the 802.1x authenticator set identityCLI command. • Rekey timeout only applies if WEP is enabled. It is the time (in seconds) when WEP keys are rotated. This is set using the 802.1x authenticator set rekey-timeoutCLI command. • Key transmission enabled status (true/false). This displays whether or not the authenticator is configured to send keys to supplicants. • Entropy pool. This is a series of characters automatically generated at random. Once a configuration is saved, so is the entropy pool. The saved configuration is restored back into the entropy pool saved at the end of the previous session. • MAC address of the supplicant • whether the supplicant is authenticated (granted access) • the time (in seconds) between each forced supplicant re-authentication • the current Port Access Entity (PAE) state. The PAE is the protocol entity associated by the port. The state is either AUTHENTICATED, DISCONNECTED or CONNECTING. • the number of times the supplicant has attempted to authenticate since the system was booted • the current state of the authentication state machine for that supplicant • the number of seconds since the supplicant last sent any traffic. |
|  | 6.3 Example |  |  |  |  |
|  |  |  | prompt> 802.1x authenticator show |  |  |
|  |  |  | Ser802.1x Authenticator Auth Control Enabled : true Auth Server : RADIUS Identity String : test Rekey Timeout : 600 Key Transmission Enabled : true Entropy Pool : 5fdfc111245dff00c8bff00c079fe00 Supplicants: |  |  |
|  |  |  | MacAddress ¦Authenticated¦ Re-auth-when ¦ Pae-stated ¦ Re_auth_count ¦Auth_state ¦Idletime ---------------------------------------------------------------------------------------------------- |  |  |
|  |  |  | 00:20:2b:01:02:03¦true ¦ 8347 ¦ AUTHENTICATED ¦ 0 ¦IDLE ¦ 0 ---------------------------------------------------------------------------------------------------- |  |  |