- RADIUS Client CLI commands

This chapter describes the RADIUS Client CLI commands.

Summary

Note - This chapter is only relevant to you if you are running an image that contains the 802.1x module. It assumes that your device is connected to a RADIUS Server.

    - RADIUS Client CLI commands

The table below lists the RADIUS client commands provided by the CLI:

| Command | Reference |
| --- | --- |
| radclient add | radclient add |
| radclient delete | radclient delete |
| radclient enable¦disable | radclient enable¦disable |
| radclient get | radclient get acctinterval |
| radclient list | radclient list |
| radclient set | radclient set acctinterval |
| radclient show | radclient show |
| radclient status | radclient status |

radclient add

    - Syntax

radclient add {acctserver|authserver} <servname> <ipaddr> <shared_key> [<udp_port>]

    - Description

This command configures your device to act as a client of the RADIUS server in your network. The server can either be an Accounting server (used to log events for billing purposes) or an Authentication server (used to authenticate supplicants on your network).

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| acctserver | Configures your device to act as a client of the RADIUS Accounting server. | N/A |
| authserver | Configures your device to act as a client of the RADIUS Authentication server. | N/A |
| servname | A name that identifies an existing RADIUS server attached to your network. | N/A |
| ipaddr | The IP address of the RADIUS server. | N/A |
| shared_key | A shared ‘secret’ string that must be configured on both the RADIUS client and server in order for the client to connect. | N/A |
| udp_port | The UDP port number used by EAP to transport messages. | N/A |

    - Example

prompt> radclient add authserver server_pc 192.168.50.50 shared_secret 1813

    - See also

radclient enable|disable

radclient show

radclient delete

    - Syntax

radclient delete {acctserver|authserver} <servname>

    - Description

This command deletes a single RADIUS server from the list that your RADIUS client was previously configured to work on using the radclient add command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| acctserver | Configures your device to act as a client of the RADIUS Accounting server. | N/A |
| authserver | Configures your device to act as a client of the RADIUS Authentication server. | N/A |
| servname | A name that identifies an existing RADIUS server attached to your network. | N/A |

    - Example

prompt> radclient delete authserver server_pc

    - See also

radclient enable|disable

radclient show

radclient enable|disable

    - Syntax

radclient {enable|disable}

    - Description

This command enables/disables the RADIUS client module. If disabled, your existing RADIUS client configuration is not effective.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| enable | Enables the RADIUS client module. To display the current status, enter the radclient status command. | enable |
| disable | Disables the RADIUS client module. To display the current status, enter the radclient status command. |  |

    - Example

prompt> radclient disable

    - See also

radclient status

radclient get acctinterval

    - Syntax

radclient get acctinterval

    - Description

This command displays the account update interval. Accounting records are sent to the server while the user is logged on at the intervals displayed by this command.

    - Example

prompt> radclient get acctinterval

    - See also

radclient set acctinterval

radclient list

    - Syntax

radclient list {acctserver|authserver}

    - Description

This command displays information about either the RADIUS Accounting or Authentication servers previously added to the RADIUS client configuration using the radclient add command. The following details are displayed:

• RADIUS server ID number

• RADIUS server name

• RADIUS server IP address

• UDP port used to transport data

• Retransmission retries

• Timeout

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| acctserver | Lists all RADIUS Accounting servers previously added to the RADIUS client configuration using the radclient add acctserver command. | N/A |
| authserver | Lists all RADIUS Authentication servers previously added to the RADIUS client configuration using the radclient add authserver command. | N/A |

    - Example

prompt> radclient list authserver

Radius Authentication Servers:

ID | Auth Server Name | IP Address      | Port  | retries | timeout

-----------------------------------------------------------------------

1 | server_pc        | 192.168.50.50   | 1813  | 0       | 5

-----------------------------------------------------------------------

    - See also

radclient show

radclient set acctinterval

    - Syntax

radclient set acctinterval <setacctinterval>

    - Description

This command sets the account update interval. Accounting records will be sent to the server while the user is logged on at intervals set by this command.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| setacctinterval | The account update interval (in seconds). | 600 |

    - Example

prompt> radclient set acctinterval 800

    - See also

radclient get acctinterval

radclient set acctserver|authserver

    - Syntax

radclient set {acctserver|authserver} <servname> {key <shared_key>|retries <retries>|timeout <timeout>}

    - Description

This command allows you to set the shared key, retry setting and timeout setting on your RADIUS server.

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| acctserver | Lists all RADIUS Accounting servers previously added to the RADIUS client configuration using the radclient add acctserver command. | N/A |
| authserver | Lists all RADIUS Authentication servers previously added to the RADIUS client configuration using the radclient add authserver command. | N/A |
| servname | A name that identifies an existing RADIUS server attached to your network. | N/A |
| shared_key | A shared ‘secret’ string that must be configured on both the RADIUS client and server in order for the client to connect. | N/A |
| retries | The number of times a RADIUS authentication request will be transmitted after a timeout. | 0 |
| timeout | The amount of time (in seconds) to wait for a reply from the RADIUS server before retrying | 5 |

    - Example

prompt> radclient set authserver server_pc key new_secret

    - See also

radclient show

radclient list

radclient show

    - Syntax

radclient show {acctserver|authserver} <servname>

    - Description

This command displays the following information about a single RADIUS server:

• RADIUS server name

• RADIUS server IP address

• UDP port used to transport data

• Timeout - the number of times a RADIUS authentication request will be transmitted after a timeout.

• Retries - the amount of time (in seconds) to wait for a reply from the RADIUS server before retrying

    - Options

The following table gives the range of values for each option which can be specified with this command and a default value (if applicable).

| Option | Description | Default value |
| --- | --- | --- |
| acctserver | Lists all RADIUS Accounting servers previously added to the RADIUS client configuration using the radclient add acctserver command. | N/A |
| authserver | Lists all RADIUS Authentication servers previously added to the RADIUS client configuration using the radclient add authserver command. | N/A |
| servname | A name that identifies an existing RADIUS server attached to your network. | N/A |

    - Example

prompt> radclient show authserver server_pc

Server Name  : server_pc

IP Address   : 192.168.50.50

UDP Port No  : 1813

Timeout      : 5 seconds

Retries      : 0 Times

    - See also

radclient list

radclient status

    - Syntax

radclient status

    - Description

This command displays the status of the RADIUS client module:

• true = enabled

• false = disabled

    - Example

prompt> radclient status

Status: true

    - See also

radclient enable|disable