VPN PPTP Application

Installation Guide

Configuring a Remote Access PPTP VPN Dial-in Connection

A remote worker establishes a PPTP VPN connection with the head office using Microsoft's VPN Adapter (included with Windows XP/2000/ME, etc.). The router is installed in the head office, connected to a couple of PCs and Servers.

Dial-in

Configuring PPTP VPN in the Office

The input IP address 192.168.1.200 will be assigned to the remote worker. Please make sure this IP is not used in the Office LAN.

1

4

3

2

5

| Item | Function |  | Description |
| --- | --- | --- | --- |
| 1 | Connection Name | VPN_PPTP | Given a name of PPTP connection |
| 2 | Dial in |  | Check Dial in |
|  | Private IP Address Assigned to Dialing User | 192.168.1.200 | An assigned IP address for the remote worker |
| 3 | Username | username | Input username & password to authenticate remote worker |
|  | Password | 123456 |  |
| 4 | Auth.Type | Chap(Auto) | Keep as default value in most of the cases, PPTP server & client will determine the value automatically. Refer to manual for details if you want to change the setting. |
|  | Data Encryption | Auto |  |
|  | Key Length | Auto |  |
|  | Mode | stateful |  |
| 5 | Idle Time | 0 | The connection will be disconnected when there is no traffic in a predefined period of time. Idle time 0 means the connection is always-on. |

Configuring Remote Worker for Dial-out to the Router

| Step 1 Select “New Connection Wizard” to create a VPN-L2TP dial out service. |  |
| --- | --- |
| Step 2 Click Next > to the next step. |  |
| Step 3 Check “Connect to the network at my workplace” radio button. Click Next > to the next step. |  |
| Step 4 Check “Virtual Private Network connection” radio button. Click Next > to the next step. |  |
| Step 5 Give a name to the PPTP connection. Click Next > to the next step. |  |
| Step 6 Input VPN-PPTP Server’s IP address. You can use Host name, only if the DDNS function is enabled in the Router. Refer to the user manual for more detail. Click Next > to the next step. |  |
| Step 7 Click Finish completing the VPN-PPTP setting. A window will pop up asking for User name and Password. |  |
| Step 8 User name and Password should be identical to the set up in the Router. Click Connect to establish a connection to your VPN-PPTP Server. Completed! |  |

Configuring a Remote Access PPTP VPN Dial-out Connection

A company’s office establishes a PPTP VPN connection with a file server located at a separate location. The router is installed in the office, connected to a couple of PCs and Servers.

Dial-out

Configuring the PPTP VPN in the Office

1

4

3

2

5

| Item | Function |  | Description |
| --- | --- | --- | --- |
| 1 | Connection Name | VPN_PPTP | Given name of PPTP connection |
| 2 | Dial out |  | Check Dial out |
|  | Server IP Address (or Hostname) | 69.121.1.33 | An Dialed server IP |
| 3 | Username | username | A given username & password |
|  | Password | 123456 |  |
| 4 | Auth.Type | Chap(Auto) | Keep as default value in most of the cases, PPTP server & client will determine the value automatically. Refer to manual for details if you want to change the setting. |
|  | Data Encryption | Auto |  |
|  | Key Length | Auto |  |
|  | Mode | stateful |  |
| 5 | Idle Time | 0 | The connection will be disconnected when there Is no traffic in a predefined period of time. Idle time 0 means the connection is always-on. |

Configuring the Server for Incoming Connections from the Router

| Step 1 Select “New Connection Wizard” to create a VPN-PPTP dial out service. |  |
| --- | --- |
| Step 2 Click Next > to the next step. |  |
| Step 3 Check “Set up an advanced connection” radio button. Click Next > to the next step. |  |
| Step 4 Check “Accept incoming connections” radio button. Click Next > to the next step. |  |
| Step 5 Click Next > to the next step. |  |
| Step 6 Check “Allow virtual private connections” radio button. Click Next > to the next step. |  |
| Step 7 Select user(s) that will be allowed to login to the PPTP Server. Click Next > to the next step. |  |
| Step 8 Select “Internet Protocol (TCP/IP)”. Click Properties for advanced IP setting. |  |
| Step 9 This is advanced setting for IP assignment. Please select either one of the assignments. If you choose “Assignment TCP/IP addresses automatically using DHCP”, you must have your DHCP Server enabled. Click OK to the next step. |  |
| Step 10 Click Finish to complete the connection set up. Completed! |  |

Configuring a LAN-to-LAN PPTP VPN Connection

The branch office establishes a PPTP VPN tunnel with head office to connect two private networks over the Internet. The routers are installed in the head office and branch office accordingly.

Both office LAN networks MUST in different subnet with LAN to LAN application.

Attention

Configuring PPTP VPN in the Head Office

The IP address 192.168.1.200 will be assigned to the router located in the branch office. Please make sure this IP is not used in the head office LAN.

1

4

3

2

5

6

| Item | Function |  | Description |
| --- | --- | --- | --- |
| 1 | Connection Name | HeadOffice | Given a name of PPTP connection |
| 2 | Dial in |  | Check Dial in |
|  | Private IP Address Assigned to Dialing User | 192.168.1.200 | IP address assigned to branch office network |
| 3 | Peer Network IP | 192.168.0.0 | Branch office network |
|  | Netmask | 255.255.255.0 |  |
| 4 | Username | username | Input username & password to authenticate branch office network |
|  | Password | 123456 |  |
| 5 | Auth.Type | Chap(Auto) | Keep as default value in most of the cases, PPTP server & client will determine the value automatically. Refer to manual for details if you want to change the setting. |
|  | Data Encryption | Auto |  |
|  | Key Length | Auto |  |
|  | Mode | stateful |  |
| 6 | Idle Time | 0 | The connection will be disconnected when there Is no traffic in a predefined period of time. Idle time 0 means the connection is always-on. |

Configuring PPTP VPN in the Branch Office

The IP address 69.1.121.30 is the Public IP address of the router located in head office. If you registered the DDNS (please refer to the DDNS section of this manual), you can also use the domain name instead of the IP address to reach the router.

1

4

3

2

5

6

| Item | Function |  | Description |
| --- | --- | --- | --- |
| 1 | Connection Name | BranchOffice | Given a name of PPTP connection |
| 2 | Dial out |  | Check Dial out |
|  | Server IP Address (or Hostname) | 69.121.1.33 | IP address of the head office router (in WAN side) |
| 3 | Peer Network IP | 192.168.1.0 | Head office network |
|  | Netmask | 255.255.255.0 |  |
| 4 | Username | username | Input username & password to authenticate branch office network |
|  | Password | 123456 |  |
| 5 | Auth.Type | Chap(Auto) | Keep as default value in most of the cases, PPTP server & client will determine the value automatically. Refer to manual for details if you want to change the setting. |
|  | Data Encryption | Auto |  |
|  | Key Length | Auto |  |
|  | Mode | stateful |  |
| 6 | Idle Time | 0 | The connection will be disconnected when there Is no traffic in a predefined period of time. Idle time 0 means the connection is always-on. |